Draft

An environmental and social impact risk analysis of a plantation project in Uruguay

# Foreword (to come from Stora Enso or another)

# Contents

Acknowledgements

We are grateful for the chance to carry out a study with this degree of freedom and acknowledge the lead taken by Stora Enso in investing in a detailed survey at a time when they could have got away with doing far less. If the study and accompanying tools work, we have an opportunity to help set best practice for fast wood plantations far beyond the current project and country. The tools being developed will hopefully be useful throughout the region – in some cases throughout the world.

Many more acknowledgements to come…

Summary of key conclusions

Introduction

Development of forest plantations has become one of the more controversial issues in forest or natural resource management. Its proponents believe that plantations can reduce pressure on natural forests, produce a sustainable timber resource and perhaps in the long term also supply an important contribution to global energy needs. Opponents believe that plantations if anything increases the rate of forest loss, creating a new range of social and environmental problems in the process, reducing local peoples’ control on land in favour of major corporations and encouraging further wasteful consumption.

First draft – needs additional material

There are things to be said for both points of view, and examples of both good and very bad plantations from around the world. Plantations are also changing – generally becoming even more intensive but also being managed more scientifically to minimise wastage and off-site effects. There is strong pressure to move towards genetically modified trees and plantations already rely on clonal varieties and encourage uniformity. A plantation bears the same relationship to a natural forest as a wheat field to a meadow: both may have their place in the landscape but the differences need to be understood.

The following document draws from the work of many different researchers, mainly in Uruguay but also based internationally. It is inevitably selective and often draws on multiple sources. But what follows also hopefully reflects the diversity of thinking that has gone into the report. The experts commissioned in Uruguay all had the same terms of reference but tackled them in a variety of ways: some included much more theoretical discussion, some focused on recommendations for the company, others drew more heavily on opinions of local stakeholders etc. This diversity has helped us in understanding the very complex set of conditions that relate to the plantation and to make what we hope are a reasonable set of recommendations. With this in mind we have attempted to retain at least some of the diversity within the report: the sections of analysis within Uruguay, although in each case rewritten at least in part by the coordinating team, do not slavishly follow a single pattern but reflect the ways in which different groups tackled the issues. A full set of papers is available on CD and a volume of papers specifically on the landscape characterisation in Uruguay is planned.

Aims of the study

Background

The current study is not a classic Environmental and Social Impact Assessment but rather a risk analysis and set of advice for the company.

The original terms of reference for the project were to carry out an environmental and social impact assessment (ESIA). A classic ESIA generally takes place before a project is started and analyses the stated intentions of any development, usually by drawing on a management plan or at least on operational plans. The theory is rooted in an assumption that developers state what they intend to do and independent assessors look at these plans and make a judgement as to the likely impacts, based on prior experience, field surveys, interviews with a range of stakeholders and other research results. In cases where an ESIA is a legal obligation, the ESIA team may be asked to decide if the project should go ahead or not, or perhaps more commonly to suggest changes and improvements that are needed before implementation. However, for several reasons, a classic ESIA is not suitable in this case:

- The project has already started and so part of the analysis is of ongoing impacts as well as making predictions of future impact;

- Stora Enso has thus already decided to go ahead with the work and the assessors are not in a position to halt the project if they feel it does not meet acceptable requirements

- There is no management plan to analyse, and no plan to complete one in the near future, because the aims of the project are still being refined and some key decisions have yet to be made – for instance about the location (and to a certain extent even the existence) of a pulp mill, the mix of tree species to be planted and so on;

- The Environmental Management System is not yet decided so we cannot comment on many aspects of management, but rather can give advice about what types of management might be set in the future;

- The area under consideration for planting is evolving and is now twice as large as described when the project started, which also means that quite different planting processes may be needed on the new area;

- The Uruguay government does not require a detailed ESIA and the current study is a voluntary exercise by Stora Enso – this being the case there is no particular reason to keep to a model that does not fit the circumstances;

- Some information – particularly relating to the pulp mill – remains confidential – while this does not impact directly on the current study (which is limited to the plantation) it clearly has implications for the work;

- A large part of the study is devoted to developing tools and methodologies for undertaking the plantation and to giving advice on approaches, both well beyond the scope of a standard ESIA.

We believe that some or all of these limitations are often in place in situations where a standard ESIA is carried out and that many professional assessment consultancies sketch over gaps in information, uncertainties and changing conditions. Impact assessments have a relatively low reputation and part of the reason is that they are often trying to operate in sub-optimal conditions.

Eucalyptus plantation

Nigel Dudley

Yet there are good reasons for companies to delay finalising plans: they often need to learn about conditions in new parts of the world, the technical, political and economic conditions will continue to change during the development of the project, and individual projects are also affected by wider changes within the operation so that decisions made today may be irrelevant if a new CEO or new board makes changes at a higher level. At the same time, the people working for or impacted by major development projects managed offshore have a right to some level of security with respect to plans and operations. One of the more justifiable fears of globalisation is that the additional manoeuvrability permitted to a global company increases the risk that it will change, downgrade or discard projects much more readily than was the case in the past. Impact assessments therefore not only need to look at what is planned but also try some crystal ball gazing to work out what might be planned in the future, how a company or a government might react if conditions change and what the long-term implications of this will be. It also needs to consider issues of profitability. Wonderful social and environmental policies are of little use if the company does not stay in profit, because controls are likely to be abandoned as staff members rush to make up shortfalls or a project can simply be sold on to other concerns that may take little notice of any previous commitments. A new style of analysis is needed to take account of these realities.

Wonderful social and environmental policies are of little use if the company does not stay in profit….

The approach taken in this study

The study has developed from being a standard ESIA into something more akin to a risk analysis with accompanying mitigation strategies, aimed at helping Stora Enso to develop a project that could be a model for such developments both within the company and more generally. It falls short of a whole new conceptual approach to impact analysis (which may well be needed) but moves beyond traditional practice. The study has a number of distinct but related elements:

- Analysis of experience from other parts of the world to identify areas of potential risk

- Assessment of the landscape being considered for plantations (i.e. more than just planted areas) to identify ecological and social factors likely to be affected by the plantation;

- Interviews with representative stakeholders in communities within the broader landscape and within Uruguay as a whole to find their hopes and fears about the plantation

- Detailed assessment of the current operations in light of the three parts listed above to make judgements about likely impacts of the plantation – good and bad

- Development of a series of tools to help improve plantation policy covering:

- Additions to the criteria used for site selection

- Tools for site planning

- A social and environmental monitoring and evaluation system

- Advice about approaches to rights based development

- Analysis of possible future trends and their likely impacts on the project

Section 1

Background

Forestry in Uruguay

Stora Enso

Plantations in a global context

Forestry in Uruguay

Uruguay is one of the few countries in the world that is basing its entire forestr industry on intensive plantations. Over the last twenty years, plantation forestry has undergone a rapid expansion, peaking in 1997 and falling of thereafter but still a major economic force in the country. With the current round of investments rate of establishment might increase once again.

Uruguay has a very low level of natural forest cover, although this has undoubtedly declined because of agriculture and other human activities, it would never have been high. But the type of grassland seen over much of the vountry today is not the natural vegetation either in most cases; much of the land would under normal circumstances be covered by bushes and scrub (see section on ecological history on page ##).

The government has actively encouraged plantation development through the Forestry Act of Law 15.939 of December 15 1987, which provides tax benefits and financial subsidies for a proportion of tree establishment costs. The programme has been successful in attracting domestic investors and also companies from amongst others Spain, Finland, Canada and the United States. The tax incentive programme is still in place but the financial subsidies were phased out gradually between 2004 and 2007.

MORE HERE – SHOULD BE TWO PAGES TOTAL

Stora Enso company profile and activities in Latin America

Stora Enso is currently the leading producer of paper and paperboard in the world, with 2006 sales of €14.6 billion. It employs 44,000 people, at present concentrated in Finland (28 per cent), Sweden (18 per cent), Germany (13 per cent) and North America (11 per cent). There are major offices in Helsinki and Stockholm although the CEO sits in London. It has annual production capacity of paper and board of 16.5 million tonnes plus 7.4 million m3 of sawn wood products, including 3.2 million m3 of value-added products.

Comparison of Stora Enso with other major pulp and paper producers for 2006

The company’s mission is to be “the leading forest products company in the world” and has a core set of mission objectives and principles outlined in the box below.

Vision

We will be the leading forest products company in the world

  We take the lead in developing the industry

  Customers choose us for the value we create for them

  We attract investors for the value we create

  Our employees are proud to work with us

  We are an attractive partner for our suppliers.

Values

  Customer focus - We are the customers' first choice

  Performance - We deliver results

  Responsibility - We comply with principles of sustainable development

  Emphasis on people - Motivated people create success

  Focus on the future - We take the first step

Changing geographies

Stora Enso has been involved in plantation establishment outside Europe for two decades. Initial efforts focused on Indonesia, particularly Kalimantan, and more recently Portugal. The company has recently divested in several operations and has adopted an Assets Performance Review to refine its investment policy. Stora Enso’s new initiatives are in Russia, China and Latin America, with some interest in India. The drivers for this expansion are low-cost fibre, fast-growing market segments and balancing the Group’s portfolio geographically.

Stora Enso’s new initiatives are in Russia, China and Latin America, with some interest in India.

Operations in Latin America currently focus on Brazil (the Veracel plantations and pulp mill and new plantations in Rio Grande do Sul) and Uruguay. The overall management for the Latin American division is based in Sao Paulo Brazil, with offices amongst other places in Porto Alegre, Montevideo and field locations in Uruguay. The Veracel pulp mill in Brazil, a joint venture with Aracruz, started production of eucalyptus pulp in May 2005. The annual mill capacity is 900 000 tonnes. Stora Enso’s 50 per cent share of the output will be utilised in its mills in Europe, China and North America. Veracel is one of the largest private-sector investments in Brazil in recent years; total investment in plantations, mill and infrastructure is about US$ 1.2 billion. The mill draws on 76,000 hectares of eucalyptus plantations in Brazil.

The group became the sole producer of coated mechanical paper in Latin America in August 2006 by buying Vinson Industria de Papel Arapoti Ltda. and Vinson Empreendimentos Agricolas Ltda. from International Paper. The business includes a paper mill producing coated mechanical paper (205,000 tonnes annual capacity), sawmill (150,000 m3 sawn timber annual capacity) and approximately 50,000 hectares of land including 30,000 hectares of productive plantations.

Stora Enso is developing new plantations in Uruguay and Rio Grande do Sul. Both are adopting a similar approach, buying medium sized farms to establish plantations, aimed eventually at supplying company-owned mills. The situation is politically more complex in Brazil because of pressure on land and a powerful landless peoples’ movement that does not exist in Uruguay. A demonstration by five hundred farmers against the Stora Enso plantation in Rio Grande do Sul was reported in March 2007 (Helsingen Sanomat 2007). Figure ## below identifies the main areas of operation in Rio Grande do Sul, Brazil.

Stora Enso

ARA

VCP

VCP

ARA

Figure ##: Approximate location of the Stora Enso plantation landscape in Brazil

The group aims to become a major producer of paper in the region, centred on Brazil but with significant operations in Uruguay. The Latin American strategy is still emerging.

Planned project activities

The project aims to create a sustainable supply of high quality pulp wood in a part of the world where production costs are still comparatively low.

The operation is managed from Stora-Enso offices in Porto Alegre, Brazil, along with a similar project being developed in Rio Grande do Sul, although the Uruguay operation is managed on a day-to-day basis from offices in Montevideo, San Gregorio and Durazno. It is intended to separate the two operations at some stage in the future, whereupon management will be solely within Uruguay.

The final plantation estate is intended to cover around 100,000 hectares and will be mainly of eucalyptus and pine at a ratio of 4:1.

The final plantation estate is intended to cover around 100,000 hectares and under current proposals will be mainly of eucalyptus (Eucalyptus dunnii) and pine at a ratio of 4:1, although this proportion has not been finally decided. There is currently some discussion about whether Eucalyptus grandis could be a better option than E. dunnii. If pine were to replace eucalyptus as the dominant species used, this would have implications on the area planted.

The company plans to plant 13,000 hectares per year in Uruguay in the establishment period although it only planted 5,000 hectares during 2006, the first year of planting. Most planting will be on company-owned land (currently plans are that around 80 per cent of trees should come from land owned by Stora Enso) and there is currently an active land purchasing programme. Around a fifth will come from out-grower schemes, buying logs from local land-owners under contract, and it is possible that this proportion could increase in the future.

The final plantation estate will therefore be made up of separate holdings of various sizes scattered across the whole plantation landscape and with a network of public roads used to transport logs to their eventual destination. This model – both scattered holdings and an accompanying outgrower scheme – has been used successfully in other plantation projects in Uruguay.

The plantation system will be intensive, aiming at a rapid (5-8 year) growth cycle and using fertilizers, herbicides, formicides and other pesticides, along with mechanised harvesting systems to achieve a high output. Many of the details of the operation, including the Environmental Management System, have yet to be determined and some recommendations are made at the end of this report.

It is currently assumed that the operation will also include a pulp mill somewhere within the plantation landscape and various sites are being investigated. But we understand that no final decision has been made on either location or indeed on the final need for a mill. If no mill were built, fibre or possibly logs in the case of pine would be exported.

Stora-Enso sustainability policy

Stora Enso has made a company-wide commitment to a sustainability agenda. This part of its activities is being expanded, with a six person team in Stockholm and Helsinki developing a new policy and goals and harmonising approaches across all operations. The current report is a contribution to this process.

The Sustainability Committee is one of Stora Enso’s operative committees; its chairman has overall responsibility for sustainability issues. Operational management is responsible for sustainability performance at each organisational level, to guarantee compliance with the Group’s commitments. The Sustainability Committee has four support teams:

- Environment Co-ordination Team (including a Climate Change Steering Group)

- Customer Support Team

- Corporate Social Responsibility Team

- Forest Environment Team (including a Forest Environment Plantations Team)

“Stora Enso is committed to sustainability – economic, environmental and social responsibility underpins our thinking and our approach to every aspect of doing business.

The group’s attitude to sustainability is outlined in its Mission, Vision and Values paper (Anon undated a): “Stora Enso is committed to sustainability – economic, environmental and social responsibility underpins our thinking and our approach to every aspect of doing business. The Group builds accountability into its operations by being transparent and engaging in open dialogue with stakeholders. Group-wide targets and clear governance are used to monitor and measure how well Stora Enso performs in terms of sustainability”. This is part of a wider commitment outlined in a Code of Ethics statement (Härmälä 2006) quoted below:

Box: Extract from the Stora Enso Code of Ethics statement

Stora Enso is committed to sustainable business practices. Sustainability is one of the Group’s key success factors, and is seen as inseparable from good corporate governance. Stora Enso expects that its management and employees follow ethical principles in their work. This statement sets out a code of fair and ethical conduct to be followed by the management and employees of the Group. Stora Enso’s Code of Ethics is based on policies and principles established by Stora Enso’s Sustainability management. The principles and practices referred to in this Code of Ethics statement are found in the following documents:

- Stora Enso Corporate Governance

- Stora Enso Communications Policy and Principles

- Stora Enso Electronic Communications Media Policy

- Stora Enso Financial Code of Ethics

- Stora Enso Anti-Fraud Policy

- Stora Enso’s Sustainability Policy

- Stora Enso Corporate Social Responsibility Principles

- Stora Enso Occupational Health and Safety Policy

- Stora Enso Competition Law Compliance Programme

The management of Stora Enso will be responsible for monitoring and enforcing these policies. Any violation of these polices, principles and guidelines will be closely examined, and the necessary action will be taken.  [Our emphasis]

Further quotations from the Mission, Values and Vision statement explain how the commitment to sustainability is applied in practice [our emphasis].

“Management systems in the areas of environment, forestry, occupational health and safety and social responsibility help units to recognise the most important sustainability aspects of their operations, develop action plans and follow-up on performance on a regular basis.

“Stora Enso has established sustainability due diligence procedures, which are used in mergers, acquisitions and divestments, to identify and mitigate possible risks. Greenfield projects must additionally undergo environmental and social impact assessments. Sustainability is an essential part of Stora Enso’s risk management. Potential risks related to sustainability could result in material or reputational damage if not pro-actively managed.”

Forest certification

Forest certification is a process whereby an independent auditor assesses performance of forest management against an agreed set of criteria and, if the operation passes, awards a certificate of assurance. A number of forest certification schemes exist with different standards and approaches. Stora Enso has traceability systems in place for all its wood fibre and pulp, 90 per cent of which are third party certified. In addition, 55 per cent of the wood fibre used by the group comes from certified forests (with or without chain of custody certification). Stora Enso “promotes forest certification and aims globally to increase the volumes of wood originating from third-party certified forests.” It works with more than one forest certification system including PEFC, FSC, SFI, CSA, ATFS and CERFLOR (Grönroos 2007). This statement suggests that whle Stora Enso is committed to the principle of certification it has drawn back from committing to certification in every single operation. However, Stora Enso staff members have said that they are committed to certification of the operations in Uruguay, but that this will not take place immediately and the actual certification system has yet to be selected. Some further principles related to forest certification (Anon, undated b) are outlined below.

Stora Enso's principles for the development of forest certification

I. Credible forest certification systems must:

- Take into account national and regional characteristics such as natural conditions, forest ownership structures and legislation, to ensure they are widely applicable

- Formulate certification criteria through open stakeholder dialogue.

- Include social, economic and environmental aspects in criteria.

- Promote continuous improvements, through procedures similar to those used in Environmental Management Systems.

- Require independent third party audits.

II. Stora Enso supports the mutual recognition of credible forest certification systems

III. Forest certification systems should complement the environmental and quality management systems used in wood procurement.

Plantations in a global context

The following section puts the proposals from Uruguay into a broader perspective, looking first at the history of and debates about plantations in general and then summarising the main questions relating to environmental, social and economic impacts.

History of tree plantations

Trees have been planted for millennia. The longest continual written records of a forest management unit dates back over two thousand years for forests managed in Japan to provide particular timber species for rebuilding Shinto temples. Fruit trees have been deliberately cultivated for even longer; for example olives have been cultivated for at least 5000 years (Perlin 1989). Over the centuries, management approaches have evolved gradually, until the 20th century when the type of evenly-spaced, single species forest stands that most people recognise as “plantations” started to become more common. Many of these were set up initially to provide either environmental services such as soil stabilisation or slope control, or to build up strategic timber supplies. They were located mainly on state-owned land and supported by government funds or tax breaks.

In the United Kingdom, intensive plantations emerged as a state policy following the First World War to build up strategic supplies of timber, using exotic species and established on land compulsorily-purchased by the government.

Sitka spruce plantation in Snowdonia National Park, Wales

Over the past two decades, a proportion of plantations have been managed far more intensively, leading to increases in productivity. At the same time, there has been a shift from public to private sector in terms of both finance and control

Despite some recent changes most forests around the world are still controlled by governments (White and Martin 2002). However, within plantations two factors have altered quite dramatically over the last couple of decades:

- A rapid intensification of management and output: including changes to aspects of: (1) composition including species selection and breeding with growing use of clonal varieties and considerable pressure to introduce genetically modified trees (Strauss et al 2001); (2) approaches to management, particularly increased use of agrochemical inputs and the degree of mechanisation; (3) scale and density of planting; and (4) a resulting increase in productivity. This has led to the recognition of a new form of fast-wood plantation (Cossalter and Pye-Smith 2003), with rotation times as short as 5-6 years and almost double the productivity of even those softwood plantations that have until recently been regarded as intensively-managed.

- A shift from public to private sector: until the 1980s, global tree planting was still dominated by states and even most of the emerging fast wood plantations were financed by grants (both from within the country of planting and from donor agencies) and various concessions and tax breaks. More recently, private investment has controlled an increasingly important proportion of the global market, focusing particularly on those plantations fast rotation and returns on investment (Garforth and Mayers, 2005).

These two trends are linked. In the past, the rate of return from tree production was too slow to attract much private capital unless trees were used as a way of “storing” money in the medium term. Most plantations were developed using a variety of state-based incentives varying from direct grants through various forms of tax breaks. Although this tendency remains, the much more rapid return from fast wood plantations (down to 5-6 years in some cases) has made it far easier to attract private investment into forest development.

While these super-fast forests are still an almost insignificant proportion of the global forest estate in terms of the land that they cover, they provide a disproportionate amount of the industrial timber and interest in them is growing fast. The largest plantation estates are found in China, the United States, the Russian Federation, India and Japan, which each have 10 million hectares or more planted, but very little of this is highly intensive. Fast wood plantations tend to be in developing countries and in the tropics or semi-tropics.

“We still know far less about the impacts and long-term sustainability of fast wood plantations than we do about other plantations”

It is therefore important to separate out the much shorter history of privately-owned, intensively-managed fast wood forest plantations from the long experience of traditional tree planting (resulting sometimes in forests that it would take a specialist to distinguish from a natural forest) and even from other forms of plantations. We still know far less about the impacts and long-term sustainability of fast wood plantations than we do about other plantations. The most intense forest plantation systems run like factories, with the process of felling and cutting logs taking place 24 hours a day, new areas continually being planted or replanted and vast factories” preparing cloned seedlings under rigidly controlled conditions.

Definitions

Opinions about when a forest is more properly defined as a plantation differ and this causes statistical problems – not everyone is measuring the same thing. The UN Forest Resource Assessment, run by FAO and the UN Economic Commission for Europe, has carried out a lengthy process to agree definitions of the term “plantation”. The latest version, used as a standard in the Global Forest Assessment in 2000, is: “Forest stands established by planting or/and seeding in the process of afforestation or reforestation. They are either of introduced species (all planted stands), or intensively managed stands of indigenous species, which meet all the following criteria: one or two species at planting, even age class, regular spacing” (FAO 1998). This is quite precise and constraining, it means that many planted and quite intensively-managed forests do not “qualify” as plantations – for example many Scandinavian forests are replenished through natural regeneration by leaving seed trees and scarifying the soil, which would not qualify as a plantation.

At the other extreme, the more intensive form of plantations, known variously as Intensively Managed Planted Forests (Kanowski 2005) and Fast Wood Plantations (Cossalter and Pye-Smith 2003) are now increasingly being recognised as forming a distinct subset of the plantation estate, although this has yet to be distinguished in official data sets or even properly defined. The precise cut-of point is unclear: one analysis proposes a minimum productivity of 14 m3 per hectare per year for a planted forest to be categorised as a fast wood plantation (James and del Lugo 2005) although Cossalter and Pye-Smith (2003) include species with slower growth rates amongst their fast wood survey.

History and rapid growth

A modern phase of establishing plantations, as they are defined in the FAO definition above, started in the first half of the twentieth century, particularly in Western Europe, the United States, Australia, New Zealand and South Africa. During the 1950s major planting efforts spread to Japan, South Korea and China. During the 1960s increased planting also took place in the tropics, with plantation area tripling in size between 1965 and 1980. This process has continued into the new century.

Three phases can be identified in the development of intensive plantations (adapted from Kanowski 2005):

- First generation temperate softwoods: from the 1920s onwards and concentrating on temperate softwoods, mainly pines, with a 25-45 year rotation – roughly half the established plantations are in the United States and the rest in the southern hemisphere in South America, Australia, New Zealand and South Africa

- Second generation tropical hardwoods: emerging particularly in the last twenty years and based in the tropics, focusing almost exclusively on tropical eucalyptus and acacia with  rotation times down as low as five years

- Third generation former waste wood: inclusion of wood from plantations that are mainly focused on other materials, including particularly rubber, oil palm and coconut. Changes in pulp technology and in the use of fibre in various manufactured panels have allowed these previously discarded timber sources to be utilised.

First generation: temperate softwoods: UK

Second generation: tropical hardwoods: Indonesia

Third generation: former waste wood: oil palm in Malaysia

According to the latest figures from FAO, forest plantations covered 140 million hectares in 2005, making up 3.8 per cent of the total forest area, of which 78 per cent are established primarily for timbre and fibre production and 22 per cent primarily for conservation of soil and water. Plantations already supply a third of industrial timber and FAO estimates that this will increase to half by 2040 (FAO 2001). Plantations also supply 10 per cent of fuelwood (Commonwealth of Australia 1999) and there are continuing efforts to establish small-scale woodlots for energy supply; these fall outside our analysis here.

Between 2000 and 2005 the area of forest plantations has increased by about 2.8 million hectares per year (FAO 2005). Global estimations of the total differ considerably depending in part on the precise definition of plantation (for instance in the 2000 version of the Forest Resource Assessment FAO itself estimated that there were already 187 million ha of plantations (FAO 2001), although it seems as if this has since been substantially modified downwards). Whatever the total, increase has been extremely rapid, from a negligible amount of plantations at the beginning of the twentieth century and still only 17.8 million hectares as recently as 1980 (ibid).

Fast wood plantations make up a much smaller fraction of the total. Kanowski (2005) estimates perhaps 15 per cent of the current plantations qualify as intensively-managed while Cossalter and Pye-Smith estimated a lower total of 10 million hectares in 2003 with over 70 per cent consisting of tropical or temperate eucalyptus, plus various pines, Gmelina arborea, Paraserianthes falcataria and poplars. However, it is clear that this is an area of the most rapid expansion and the proportion of fast wood plantations looks set ot continue to increase.

“Fast wood plantations make up … perhaps 15 per cent of the current plantations”

The growing debate

Fast wood plantations avoid one of the major constraints against bringing private money into forestry; namely the long lead time to repay investment. Rotation times of a few years coupled with high productivity have changed investors’ attitudes to plantations in less than a decade. But at the same time these forests have intensified the debate about the worth of plantations. Many prior agreements about sustainable forest management are no longer possible if expectations are for production at such a high level – for instance this is only possible with relatively high levels of agrochemicals and relatively high water use. It is significant that at the “International Experts’ Meeting on the Role of Planted Forests in Sustainable Forest Management”, held in Chile in 1999 under the auspices of the Intergovernmental Forum on Forests, whilst it was agreed that all sustainable forest management (SFM) principles should be met at a national level there was disagreement about the extent to which all SFM objectives should be met at landscape or stand scale (Anon 1999). The question about how far such intensive plantations can relax the standards relating to other forestry operations remains unresolved and lies at the heart of much of our analysis here.

Plantations debate and reforms

Initial welcoming of plantations was based around concerns to develop a rapid response to deforestation and energy shortages

Drakensberg, South Africa: Nigel Dudley

Plantations have become a flash point in debate between environmental groups, human rights organisations, governments and industry, sometimes catching forest management companies unprepared for the scale of reactions against intensive tree planting.

This has not always been the case, and attitudes to fast wood plantations have developed through three distinct phases amongst environmental and social NGOs, and to some extent also academics:

- Initial welcoming of plantations as a response to the social and environmental impacts of forest loss and degradation

- Growing criticism of particularly large-scale plantations on social, environmental, aesthetic and economic grounds

- Efforts at improvement: acceptance of the concept of fast-growing tree crops and attempts to influence policy on location and management to minimise detrimental side effects

It should be noted that the second and third of these strands are still running simultaneously; with entrenched opposition from some critics and active participation by others, a situation that is itself leading to tensions within the NGO community.

Initial welcome for plantations

Early fears about forest loss included a strong element of social concern and in particular fears that a perceived “fuelwood crisis” was causing social problems and exacerbating deforestation (Eckholm, 1975). Although some of these fears proved exaggerated (Leach and Mearns, 1988) they encouraged the new environmental movement to argue for rapid replanting particularly in areas where poor people were reliant almost entirely on wood supply for their energy needs. These concerns were further heightened by alarming reports about the rate of tropical forest loss (Myers, 1979). In the 1970s, proposals for replanting often included both large and small plantations and were supported by groups such as Friends of the Earth and Greenpeace. Think tanks and research institutes pointed to options for using fast-growing timber for energy (e.g. Eckholm et al, 1984; Miller et al, 1986) although in most cases these focused on small-scale and community-owned plantations. Interest in biomass as a future energy source focused mainly on faster-growing plants such as sugar beet although this situation is now changing.

Growing criticism of plantations

“By the end of the 1980s the early enthusiasm for… plantations was being replaced by growing criticism from a variety of social and environmental NGOs”

By the end of the 1980s, the early enthusiasm for (or at least neutrality towards) plantations was being replaced by growing criticism from a variety of social and environmental NGOs. Initial concerns surfaced in Europe and had as much to do with aesthetics and access as with environmental impacts, with amenity groups criticising large state-owned or privately-owned forest plantations for their visual appearance and for blocking access to open country (Tompkins, 1986). Much of the early debate centred on the UK, where proposals for major planting of the Flows Country of north-east corner of Scotland, which had probably never been naturally tree-covered, led to protests and the partial abandonment of the scheme (Stroud et al, undated). Later criticisms of Eucalyptus plantations grew in Mediterranean Europe, particularly in Spain and Portugal along with heavy criticism of the grant schemes that encouraged farmers to choose these rather than native species.

This unease soon broadened to a more general environmental critique, with studies claiming that plantations were a net cause of forest loss in countries like Indonesia, because natural forest was being cleared to provide space for planting fast crowing Acacia or Eucalyptus (Down to Earth, 1991). Problems were also identified with respect to a decline in forest quality in temperate and boreal regions as natural forests were replaced by intensive plantations, which had fewer values for wildlife, recreation or in some cases environmental services (Dudley, 1992; Dudley at al, 1995). The effects of a switch to plantations from natural woodland were analysed with respect to a range of wildlife species, initially in Europe (Watkins, 1993). Eucalyptus plantations were also subject to a sustained critique originating in India and relating to their impacts on water flow (Shiva and Bandyopadhyay, 1985), prompting a detailed response from the Food and Agricultural Organization of the UN (Poore and Fries, 1985). The long-term viability of repeated harvests of fast-growing plantations was also questioned, leading to research carried out for the UK government (Evans, 1999).

An associated debate also sprang up about the impacts of pulp mills, which have become associated with fast wood plantations in many countries. Criticism focused particularly on the impact that effluents had on aquatic life (Södergren, 1988) and human health and eventually led to changes in pulp mill technology, that continue to develop. This debate is still very active in Latin America and was intensified in 2005 by a serious poisoning incident of a Ramsar site (a conserved wetland area) in Chile. This has led, for example to blockades and political tensions between Argentina and Uruguay about a pulp mill being built in Fray Bentos, southern Uruguay, from 2005 to the present.

The pulp mill under construction at Fray Bentos, Uruguay, has become the focus for unprecedented protests from Argentina, which have gone as far as the International Court in The Hague

.

Nigel Dudley

Simultaneously, social-welfare organisations were questioning the impacts of plantation establishment on local communities, particularly in respect to the impacts through environmental degradation and loss of human rights (Carrere and Lohmann, 1996) and disruption to employment (Carrere, 1999). The human health impacts from pulp effluents were stressed by organisations such as Greenpeace in a long-running campaign to change technology in mills focused initially in Canada and Europe (Kroesa, 1990). The social and economic impacts of intensified forestry efforts were also assessed by some donor organisations, including FINNIDA (Hisham et al, 1991). A particular issue of concern related to where the land for planting came from, with governments in some countries being prepared to sell concessions in areas that included the traditional homes of indigenous peoples, or where settler families had already established communities. This situation occurred particularly in parts of south east Asia, including in a plantation at one time owned by Enso in Kalimantan on the island of Borneo, but has also been a long-running source of tension in some Latin American countries where indigenous peoples still have claims on land that has been managed by timber companies for decades.

To some extent, the debate about plantations has always focused on large, monoculture plantations and more recently especially on those controlled by foreign companies. Few people have criticised the use of small village woodlots, whatever species are chosen, or of small-scale farmers focusing on trees as one amongst a variety of crops.

Attempts at improvement

A series of high profile governmental and non-governmental initiatives aimed to examine the issues and propose a way forward. Within the United Nations, the Intergovernmental Forum on Forests (now known as the UN Forum on Forests) developed a study on the Global Outlook for Plantations, hosted by the government of Australia (Commonwealth of Australia, 1999) and also held an experts’ meeting in Chile in 1999 (Anon, 1999). These efforts were boycotted by some NGOs because they were seen as being biased towards plantations and the industry. The International Institute for Environment and Development in London prepared a major study on the pulp and paper industry for the World Business Council on Sustainable Development (Grieg-Gran, 1996), which was again criticised for being funded by the timber industry. The Center for International Forestry Research (CIFOR) published a review of fast wood forestry that sought to present a balance between the two points of view (Cossalter and Pye-Smith, 2003).

Some environmental groups also started to engage with the plantations industry (e.g. Elliott 1999). In what amounted to a major change in policy, the Forest Stewardship Council (FSC), which was originally set up to certify natural forest management, developed certification systems for plantations and many environmental and social NGOs engaged in this process. This is now currently under review, although the principle of certifying plantations remain and in fact almost all tropical FSC certifications focus on plantations. Most other certification bodies, such as the PEFC, work with plantations, as does the ISO-14000 series of environmental standards.

The idea that the FSC should certify in plantations has drawn sharp criticism from some NGOs including the World Rainforest Movement (Carrere, 2006) and the Rainforest Foundation (Counsell and Loraas, 2002). Other NGOs, including WWF, have worked closely with several companies involved in plantations and developed partnerships on key issues relating to conservation approaches and monitoring of impacts: in WWF’s case this includes a conservation partnership with Stora Enso. It would be fair to say that opinion about when and if plantations are a positive aspect of forest management remains a subject of debate even within some of the NGOs that are actively working on plantation issues.

There have been a number of attempts at setting best pracice

At the same time, many stakeholders have worked at developing best practice guidelines and models for plantations to address some of the criticisms that have developed, including FAO, CIFOR and many individual countries or regions (see section 1.11 for a summary and analysis of these).

The debate has by no means concluded and positions are still shifting; some NGOs that were originally supportive of the FSC for example are now more reluctant to be involved while in other cases former opponents of plantations have tempered their opposition.

Conclusions

It is worth noting in the current context that the opposition to plantations did not arise spontaneously or for no reason; there is a depressingly long list of plantation projects that have fallen far short of what society might expect in terms of both their social and environmental impacts. We believe that this includes some past project s in Uruguay, which continue to create problems for local communities and for the wider environment. It is a common belief amongst some members of the plantation industry that opposition NGOs are motivated solely by ideology or fallacy, but we do not believe that this is either a fair or accurate representation. Opposition has arisen in response to bad plantations. Some critics have moved from concern about individual plantations to opposition to the whole concept of large plantations. No resource management activity is cost free: plantations take up land that could be managed in other ways and foster intensive growth patterns that themselves put ecosystems under additional stress. As a result of past problems, the industry is regarded by suspicion by many people and is under growing pressure to prove that it can run plantations that do not cause unacceptable social and environmental costs.

In the following parts of this chapter, we summarise some of the global state of knowledge about environmental and social costs of plantations and their sustainability over time. This information provides a background to our analysis of the current project and our recommendations for its development. It draws from the latest research from around the world and in particular from a series of working papers developed for the study.

Environmental impacts of plantations

Plantations can impact on the environment mainly through their effects on water, soil, ecological interactions, pollution and biodiversity and their role in the carbon cycle. Impacts can be beneficial or detrimental, depending to a large extent on the quality of planning and management. Some interactions are complex and inter-related. Environmental issues related to plantations are summarised in Table ## below and then described in more detail in the following paragraphs, where references to the various impacts are also included. It will be noted that many of the issues identified can potentially have positive or negative effects depending on individual conditions.

Table ##: Summary of key environmental impacts of plantations

| Issue | Notes |
| --- | --- |
| Water |  |
| Water quantity | Planting some species, e.g. pine and Eucalyptus, can result in reduced surface and/or groundwater, if these are planted in recharge areas or near water courses. Sometimes this is used positively – e.g. to reduce salinisation – in other cases plantations have led to net reduction in water availability in nearby areas. |
| Water quality | Trees can stabilise soil and reduce runoff. Conversely if badly managed, drainage, ploughing and clearfelling can increase water turbidity, which can damage breeding success of fish and disturb other aquatic life. |
| Acidification | Plantations on base-poor soils increase soil and water acidification due mainly to trees scavenging air pollutants, which are washed to the ground |
| Soil |  |
| Soil | Plantations can stabilise soil in areas where erosion has been a problem. Deep ploughing and the use of heavy machinery can also lead to soil erosion and nutrient loss as can the impact of clearfelling |
| Ecological interactions |  |
| Changes to the fire ecology | Plantations can either increase the risk of fires through poor management, or result in artificial suppression of fires due to management practices. |
| Pests and diseases | Although exotic species are generally believed to suffer fewer pest and disease attacks, this situation is changing as pests spread around the world and in some cases move from plantations to native forests |
| Biodiversity |  |
| Habitat and species | In many places, plantations have been established in the place of existing natural forest, ancient heath, ecologically important grassland etc. This can lead to dramatic reductions in biodiversity although some species can become adapted to and eventually sheltered by plantations |
| Introduction of exotic tree species | Exotics sometimes compete with native species, and have occasionally hybridised with native species, leading to loss of local provenances. |
| Pollution |  |
| Agrochemical use | Pests and the pressure to maintain high levels of productivity lead to increased use of pesticides and soluble fertilisers which if applied incorrectly can have associated environmental effects |
| Carbon cycle |  |
| Carbon sequestration | Trees sequester carbon which can mitigate climate change, although the net effect depends on how long the resulting pulp or timber remains intact |

Not all of these issues will be directly relevant to conditions in Uruguay.

Plantation impacts on water quantity

The most consistent environmental criticism levelled at plantations relates to their impacts on water availability within a catchment. Shiva and Bandyopadhyay first outlined the case against eucalyptus from the perspective of water use in 1985, prompting a rapid response from FAO (Poore and Fries 1985) and since then much time and effort has been devoted to working out the impacts of afforestation on water quality in different ecosystems, planted with different tree species. A consensus about the direction, scale and impacts of plantations on water has gradually emerged amongst most forest hydrologists. However this has gone hand in hand with an acknowledgement that the complexity of factors involved means that plantations in different situations can have almost diametrically opposite effects, so that general predictions (and general management guidelines) are of little use. Experimental data can be found to support virtually any argument and it is important to view the available evidence in its entirety.

“It is now generally agreed that net water availability under trees is reduced compared to most other land uses, including pasture”

Do plantations reduce water availability?

It is now generally agreed that net water availability under trees is reduced compared to most other land uses, including pasture. This tendency has been well recognised for over twenty years and for instance a compilation of results from 94 paired catchments in 1982 found that afforestation always reduces water outflow (Bosch and Hewlett 1982). Although most work has been carried out in Australia, South Africa and the United States, studies from as far awat as Thailand (Chomitz and Kumari 1996) and Fiji (Gregersen et al 1987) found a similar pattern. Exceptions to this do occur, including a possible increase in dry season run-off, but the latter is commonest in cloud forest and semi-arid regions (Calder 2002) and thus unlikely to be found in Uruguay.

“Different site-specific and often competing processes, may be operating and the direction – let alone the magnitude of the impacts – may be difficult to predict for a particular site” – Prof. Ian Calder

Unfortunately, impacts cannot be generalized. Calder (2002) summarises the problems: “Different site-specific and often competing processes, may be operating and the direction – let alone the magnitude of the impacts – may be difficult to predict for a particular site”.

Nonetheless, many attempts have been made to model impacts on run-off, including development of various predictive models. For example, in 1987, Kuczera developed a model for catchment yield following growth of Eucalyptus reglans, which suggested that water was reduced most when trees were young and that the catchment in this case gradually started to recover hydrological equilibrium after about 12 years. This was backed by catchment research by Vertessey et al in 2001 which showed that water yield from an old-growth stand of Eucalyptus reglans is double that of 25 year-old trees.

Figure ##: A generalized model trend of catchment streamflow reduction as a result of the growth of the forest regenerated after the forest fire (Kuczera, 1987)

Vertessey (2000) argued that run-off was reduced most markedly under relatively wet conditions. This was confirmed by Zhang et al (2001), using data from over 250 experimental catchments worldwide to develop a hydrological model that relates mean annual evapotranspiration at the catchment scale to rainfall, potential evapotranspiration and soil available water capacity. The most comprehensive review to date, by Jackson et al (2005) analysed results from 504 catchments around the world, focusing mainly on carbon sequestration but also looking at water quantity. The results showed an average decrease of 227 mm per year in streamflow (in 52 per cent of the catchments studied), and complete drying up of streams in 13 per cent of the catchments studied. For example, studies of a particular stand of Eucalyptus grandis in South Africa found that streamflow disappeared after nine years and only returned five years after harvesting (Scott and Lesch 1997).

Probably more common than major changes of surface flow is a net reduction in rate of groundwater recharge, either because trees intercept more of the water before it seeps down or in some cases because their roots can reach shallow groundwater resources directly. This seems to be particularly the case with eucalyptus (Calder 1992). For example blue gum (Eucalyptus globulus, a commonly planted species) has been shown to reduce groundwater in South-Western Australia (Harper et al 2000). In these cases there may be no visible signs that hydrology is being impacted on the land surface but nonetheless major changes can be taking place to long-term water reserves.

It is noticeable that in the case of both direct impacts on surface water and, to an even greater extent, impacts on recharge of ground water, there may be a long lag time between plantation establishment and measurable impacts on water (Scott 2005). This poses a challenge for forest managers, because by the time serious problems are noticed there will already have been major investment and the assumption of a long-term project.

Different tree species have differing impacts on water (which can also vary dramatically within a species depending on water availability, climate etc). Canopy characteristics are important in determining differences in interception losses between species (Hall 2003). Despite the bad name associated with eucalyptus, most research suggests that pines actually extract more water. For example Vertessey (2000) found run-off reductions greatest under wet conditions and under pine. A recent evaluation of 145 catchments experiments indicated a 20-25 mm reduction in outflow at a 10 per cent increase in coniferous forest, a 6 mm decrease for eucalypt forest and a 17-19 mm decrease for deciduous forests (Sahin and Hall, 1996).

In general, plantations are likely to reduce water availability compared to grassland or agricultural crops, although in many cases this will only be a return to a hydrology similar to that found before land was cleared for agriculture. For example, afforestation in Australia has sometimes changed permanent streams back into seasonal streams, which would have been the case before clearance. Plantations are likely to cause problems in terms of water supply in conditions where water availability is marginal, during exceptionally dry periods and in cases where trees can tap directly into groundwater. Due to the complexity of forest-catchment interactions and the continued lack of models of the sophistication needed to make accurate predictions (or sometimes of data to help drive the models), plantations in many parts of the world are inevitably started without certainty about likely impacts on water.

Factors affecting water availability

The extent to which plantations intercept water is therefore affected by a range of factors relating to climate, geography and management; some key variables are summarised in Table ## below.

Table ##: Factors affecting changes in water availability as a result of plantation establishment

| Variable | Details | Source |
| --- | --- | --- |
| Climate |  |  |
| Rainfall | Generally the proportion of water lost is highest under wet conditions (although the impact is usually most noticeable when water is scarce) | Vertessey 2000 Zhang et al 2001 |
| Other climate variables | Water uptake is affected by rate of transpiration, which is likely to be greatest under sunny and windy conditions |  |
| Geography |  |  |
| Soils and geology | The nature of soils and bedrock have a major impact on rate of water movement and thus groundwater recharge | Keenan et al 2004 |
| Depth of water table | Some plantation species, including many eucalypts, can draw water directly from shallow groundwater source | Keenan et al 2004 |
| Nature of water flows | Factors such as the number of streams, degree of slope, presence of wetlands and extent to which soils and vegetation retain water will all have impacts on water removal by trees |  |
| Management |  |  |
| Percentage of trees in the landscape | Impacts are affected by density of tree in a catchment. Research in Australia found anything less than 20 per cent planted will have little measurable impact, but this may not apply in other conditions. | Brown et al 2005 |
| Density of tree planting in a given area |  |  |
| Rotation time and fallow period | Short rotation times can maintain the watershed in the period of near maximum water use by trees, thus not giving groundwater systems a chance to recover, |  |
| Thinning regime | Removing trees (usually at least 20 per cent of biomass for a noticeable impact) can lead to temporary reduction in water use, although this does not usually have noticeable impacts when water is scarce | Wheeler et al 2002 |
| Proximity to water sources | Trees planted close to streams, rivers and lakes tend to extract greater amounts of water |  |
| Species | Pine is generally thought to have a greater impact on water availability than eucalyptus, although a few experiments have found the reverse. |  |

For forest managers then, there is inevitably a measure of trade off. Scott (2005) sums it up: “Growing trees quickly, something that is implicit in economically successful plantation forestry, is going to cost water; you cannot have one without the other. In certain places this will cause conflicts”. The trick is to make the best possible predictions and identify steps to minimize the problems, including planning at both landscape and site scale.

Table ##: Scales and hydrological indicators for the maintenance of catchment health, thus contributing for the protection of water values and for the search of sustainable management of planted forests (modified from Lima & Zakia, 1998)

We need to fit this table in somehow…

| Scale | Water impacts | Possible causes | Indicators | References |
| --- | --- | --- | --- | --- |
| Macro | Available water | Afforestation / deforestation | Regional water balance | Versfeld, 1996 |
|  |  |  |  | Nambiar 1999 |
|  | Landscape values and biodiversity | Unplanned, large-scale plantations | Ecological zoning | Hartley 2002 |
|  |  |  |  | Andreassian 2004 |
| Meso | Catchment degradation | Destruction of riparian ecosystems | Conditions of riparian areas | Dias Jnr et al 1999 |
|  |  |  |  | Goncalves and Stape (eds.) 2002 |
|  |  | Inadequate roads | Road design | Girvetz and Shilling 2003 |
|  |  | Soil compaction | Infiltration | Boothroyd et al 2004 |
|  |  | Erosion | Soil conservation measures | Pennington and Laffan 2004 |
|  |  |  |  | Fernandez et al 2004 |
| Micro | Water quantity and streamflow regime | Change in forest cover | Stream discharge | Swank and Miller 1968 |
|  |  |  |  | Calder 1986 |
|  | Eutrophication | Fertilisation, erosion, lack of buffer | Concentration of P and N in stream | Fahey and Jackson 1997 |
|  |  |  |  | Scott and Lesch 1997 |
|  | Stream shoaling | Erosion and sedimentation | Stream turbidity | Huber at al 1998 |
|  |  |  |  | Sharda et al 1998 |
|  | Nutrient losses | Erosion, clearcutting | Stream conductivity, catchment biogeochemistry | Câmara and Lima 1999 |
|  |  |  |  | Vital et al 1999 |
|  |  |  |  | Câmara et al 2000 |
|  |  |  |  | Vertessey et al 2001 |
|  |  |  |  | Stott et al 2001 |
|  | Organic materials | Decomposition of residues in streams | Dissolved oxygen and colour in stream water | Mackensen et al 2003 |

Plantation impacts on water quality

Plantations can affect water quality in two main ways:

This is very incomplete…

- Through the application of fertilisers, herbicides and pesticides in the management of plantations, which may end up in water courses

- Through sedimentation of water courses as a result of soil erosion through planting and harvesting practices or from logging roads and tracks

In addition, choice of tree species can sometimes have a direct impact on aspects of water quality. For instance, a study in south west Scotland found a significant rise in surface water acidification in rivers found by pine plantations (Helliwell et al 2001). In conditions of high air pollution, some plantation species are also effective at capturing pollutants on their foliage, which is later washed into water-courses ().

Agrochemicals

Although fertilizer use in forests is generally less than in intensive crop production, research has frequently found elevated levels in surface and ground water near plantations, particularly in the case of nitrate. Elliott and Hodgson (2004) analysed 5227 water samples in Tasmania’s plantations over a ten year period. They tested concentrations of herbicides, pesticides and fertilisers applied and compared them to the recommended safe limits according to Tasmanian law. They found that “Most of the operations where guideline levels have been exceeded involved application of nitrogen fertilisers. High readings of nutrients in forest streams can occur due to several factors such as high natural background levels, direct deposition of fertiliser on watercourses, application of fertiliser to saturated soils, and runoff and leaching following significant rainfall after fertilising”. Studies of plantations in mid-Wales (Chapman et al 1999); shade grown coffee plantations mixed with Eucalyptus deglupta in Costa Rica (Renderos et al 2004) and eucalyptus in the Congo (Neal et al 2004) all found increased nitrate pollution in ground and / or surface water near plantations as compared with other vegetation. Research in north Queensland, Australia, found nitrate pollution from pine plantations was leading to pollution in the Great Barrier Reef (Faithful et al 2005). Losses were generally greatest in periods of high rainfall.

Level of impact depends on rate of application but also on a range of other factors including density of tree planting, soil fertility, rainfall patterns and the fertilisation history on the site (Renderos Durán, et al, 2002); Well designed and applied environmental management systems can substantially reduce rates of loss (Croke et al 1999) although when planting is taking place continuously these controls are more difficult to apply because many involve matching application to weather conditions and seasons.

It has also been suggested that nitrate losses may be proportionately higher in nurseries (Neal et al 2004).

High nitrate levels in water causes a range of problems including in particular eutrophication (a rapid increase in algae growth followed by oxygen depletion as they die and decompose). In terms of human health, elevated nitrate levels in drinking water are related to infant methaemoglobinaemia or blue baby syndrome and World Health Organisation safety limits are set to reduce risks of this potentially fatal disease. Methaemoglobinaemia is most likely to occur in conditions where a child is already weakened and drinking contaminated water. Blue baby syndrome is generally believed to be a greater risk in under-developed countries (Pretty and Conway 1988). Nitrate may also interact with organic compounds to form nitrosamines, which are known to cause cancer, although extensive epidemiological studies in human populations have failed to find evidence that high nitrate levels cause cancer (Dudley, 1985; find reference).

WE NEED SPECIFIC REFERENCE MATERIAL ON BOTH FERTILIZERS AND PESTICIDES IN FAST-WOOD PLANTATIONS

Harvesting practices

Harvesting practices affect water quality in a number of ways. Soils and other particles can be dislodged by heavy machinery, soils that are left bare in between rotations can be washed into water courses, particularly if there are heavy rains (El Swaify, 2002) and during harvesting soils and particles may be dislodged.

Conversely, in areas where soil erosion is already a problem, establishing plantations can sometimes help to reduce rates of soil loss and stabilise the surface. Eucalyptus plantations have been used for this purpose in China for example (Jiayu and Sinming 1996).

Plantation impacts on biodiversity

Establishment of plantations invariably changes biodiversity and in most cases reduces the variety or at least the quality of this biodiversity: plantation trees are a crop and a good manager will be actively trying to eliminate many natural species (e.g. herbaceous plants, other tree species, fungi and many insect species) as a necessary part of maximising productivity. Several studies (for example Helle and Mönnönnen 1990, Hunter, 1990, Baguette et al 1994 and Cannell 1999) have found that biodiversity decreased under plantations. There are exceptions, which will be described below. The impacts depend on the habitat being replaced and on management strategies within the plantation: most importantly of all they depend on management of land within the plantation estate that is not planted with trees. Cossalter and Pye-Smith (2003) state that “If a large swathe of natural forest is cleared to make way for a fast wood plantation, there will be a loss of biodiversity. Yet a similar plantation, established on degraded land, might bring about an increase in biodiversity”.

The impacts depend on the habitat being replaced and on management strategies within the plantation: most importantly of all they depend on management of land within the plantation estate that is not planted with trees

In Europe some of the more intensive forest management is found in the UK and the state forestry body has carried out a major research project looking at the role of plantations in biodiversity and best management practices (Humphrey at al 2003). The European Forest Institute coordinated a comparative study looking at planted and natural forests (Green 2001) and the Center for International Forestry Research and International Union of Forest Research Organisation carried out a joint study on biodiversity in production forests although here the focus was not specifically plantations (CIFOR and IUFRO 1999). Together these provide a picture of likely effects. The following provides a global picture; not all of factors will affect plantations in Uruguay.

Possible impacts of plantations on biodiversity

A plantation is never a functioning natural ecosystem with a full range of species, ecosystem processes, disturbance patterns and age distribution. This is not necessarily a problem if management at a landscape scale maintains sufficient natural habitats elsewhere and here plantations may be a better for biodiversity than many alternative land uses, such as intensive agriculture. A range of impacts are listed below, starting from the worst case scenario from a biodiversity perspective and progressing (approximately) to the best possible outcome.

- Reduced biodiversity both on and off-site: if plantations replace rich natural or semi-natural habitat and are then inappropriately managed they will reduce biodiversity onsite and can decrease biodiversity offsite. Impacts can include off-site pollution from fertilisers, pesticides and changes in stream sediment and sometimes damage as a result of increased human activity, such as illegal logging using plantation roads. For example, establishment of oil palm plantations in the Kinabatangan River in Sabah, Borneo, has drastically reduced native forest but also resulted in significant pollution of river, lakes and downstream mangroves (reference).

- Reduced biodiversity on site: off-site impacts are reduced by good management but the plantation replaces valuable habitat, such as natural forests, rich natural or semi-natural grassland, heath, moor or tundra. For example studies in New Zealand have shown pine plantations to be poor habitat for native birds, particularly for those which feed on fruit and nectar, nest in holes or feed on insectivorous species (Clout 1984).

- Release of invasive species, including pests and diseases: some plantation trees have themselves become invasive, but plantations have also led to introduction and spread of damaging pests and diseases. For example, in Kenya and Malawi, the indigenous Juniperus procera and Widdringtonia nodifolia are being damaged by a cypress aphid associated with introduced Mexican cyprus plantations (Ciesla 1991).

- Increased biodiversity on the site: if plantations are established on highly degraded land, they can sometimes increase biodiversity. For example in China, Jiayu and Siming (1996) report that the establishment of eucalyptus plantations on barren land brought back birds and other species, with 71 bird species living in the eucalypt plantations of Guiqi County. In most cases only commoner species are found in these situations (Bernhard-Reversat 2001).

- Increase in particular groups or species: some groups may be better able to adapt to plantations than others. Research in UK plantations (Humphrey at al 2003) found that while vascular plants, fungi and deadwood species all declined markedly in plantations diversity of some invertebrate groups (e.g. beetles and hoverflies) was not dissimilar to native woodland.

- Increase in specific, valuable species: some plantations can act as permanent or temporary habitat for particular valuable species (for instance endemic or endangered species). For example it is now known that the merlin (Falco columbarius) has adapted to roosting and breeding in spruce plantations in Wales, UK and plantations in Costa Rica have provided habitat for a rare parrot species (references). These specific cases are quite rare, and are also likely to be commonest in relatively slow-growing plantations.

- A role in landscape scale conservation strategies: by providing linkages (corridors, buffer zones and stepping stones), plantations enable animal dispersers (particularly birds and bats) to travel further between natural forest patches, thereby also dispersing more forest seeds (Carnus et al 2003). Kanowski (2005) argues that well-designed plantations can be beneficial to biodiversity in a degraded landscape. The role of plantations in providing temporary habitat for migratory birds in Costa Rica has also been well studied (Parrish and ). The position of the plantation in the landscape will be critical to achieve such a catalytic role (Montagnini, 2005).

Traps for mammals and reptiles in one of Stora Enso’s plantation sites in Uruguay as part of an ongoing biodiversity study, with armadillo found on the plantation.

The degree to which plantations can play a positive role in biodiversity conservation also depends on land use around and between plantations. There is evidence that plantations can avoid serious biodiversity loss if they include conservation of significant natural vegetation. For example the Klabin forest in Brazil has a a mosaic of land use, where eucalyptus and pine totalling 120,000 ha inter-mingles with natural forest of 85,000 ha (Brown, Palola and Lorenzo, 2006). This assumes that the company is prepared, and can afford, to set aside large areas of land specifically for biodiversity conservation and ecosystem services.

Planning plantations to minimise damage to biodiversity

Plantations are therefore not natural or immediate habitat for much biodiversity and any improvement in their role depends on careful planning at both landscape and site scale – one of the reasons for the current study. A number of steps can reduce impacts (examined in more detail in section ##):

- Avoiding establishing plantations on the most valuable sites

- Planning plantations at a landscape scale

- Planning plantations at a site

Ultimately, the value of the plantation for biodiversity will rest mainly on the land not planted within the plantation area and how this is managed. In cases where this land is managed actively for biodiversity, or restored, then net biodiversity within the site may even increase. Conversely, if set aside land is mismanaged or is subject to pollution and other disturbance from the managed areas, then it may provide few benefits.

Managing plantations to minimise damage to biodiversity

The extent to which plantations provide habitat for biodiversity depends partly on how intensively they are managed, as well as on what they are replacing. In Ethiopia, for example, where highly degraded soils no longer hold seeds from the original forests, plantations can play an important role in providing soil cover and perches for birds that may return seeds to the soils. A study in the Munessa-Shashamane forest showed that 82 per cent of the woody species in a neighbouring relict forest could be found recolonising under the plantation (Lemenih 2006). Clearly for this to happen, there needs however, to be sufficient forest near to the plantation.

Can plantations save natural forests by replacing timber?

A common argument is that plantations can halt deforestation and conserve forest biodiversity by providing cheap and efficient alternatives to logging natural forests. However, there are few mechanisms for ensuring that this happens. Timber extraction is not usually the only pressure on native forests and therefore, establishing timber or pulp plantations will not necessarily alleviate all the pressure or prevent the deforestation (Friedman in press). Other have argued to the contrary: that if natural forests are no longer considered valuable because plantations provide most of the valuable timber and fibre, then they may be degraded even more rapidly (FAO 2001). The “displacement theory” of plantations will not work in isolation, but only if establishment and management of plantations are part of an integrated landscape approach to protection and management of forest resources. A key problem in many countries is that land ownership patterns are at a site level, and ultimately this is what will influence management decisions (Carnus 2003). Need another key reference

Social impacts of plantations

Social impacts work both ways – plantations can bring jobs and money into an area but can also result in social disruption and loss of rights. In the last few years, the social impacts of plantations have become a focus of discontent, with an increasing number of campaigning groups opposing all plantation development. The global “anti-plantation campaign” has been spearheaded by the World Rainforest Movement, which is based in Montevideo, so that these issues will be brought to the fore in respect to any developments in Uruguay. There is little doubt that the critics of plantation social impacts have plenty of ammunition in the form of bad examples from around the world, many of which have had direct and detrimental impacts on human wellbeing:

Plantations are often an additional disruption to local communities that are already undergoing rapid social change – their role can be positive or negative depending on circumstances

- Loss of land tenure and governance: ranging from indigenous peoples and local communities being forcibly ejected from their land to more subtle changes in terms of increased land pricing

- Loss of political power: through governments and to an even greater extent large businesses gaining increased control over resources and on a local scale, loss of influence over access to and management of land

- Loss of forest resources and ecosystem services: particularly when natural forests, which frequently supply multiple benefits to local communities, are replaced by single species plantations that have fewer values (e.g. may supply fewer non-wood forest products)

- Changes in employment: although plantations are often sources of employment in frequently impoverished rural areas, there is evidence that in some cases they have created fewer jobs than hoped for local communities

- Social disruption: because employment is often seasonal or occasional (e.g. bursts of activity every few years) they often use casual labour from outside, which can increase crime, prostitution and alcohol abuse

As in the case of impacts on biodiversity examined above, not all the problems described in this section are likely to apply to Uruguay, but are summarised to give an overall picture of global issues. Others definitely will apply. Plantations are often one factor amongst many affecting rural communities, which are undergoing a period of rapid social and economic change in virtually every country in the world. A study for the International Tropical Timber Organisation pointed out that: ““The social and economic conditions that exist when a forest crop is harvested are seldom the same as those prevailing when a tree seedling first takes root, nor do the priorities of individuals remain the same” (ITTO 2002).

While natural forests provide a range of benefits to communities living in and around them, the number of goods and services generally provided by plantations is much more limited

Land tenure

Land tenure issues are often the most significant problem facing plantation companies (Evans and Turnbull 2004). Plantations require large areas of land: either obtained through purchase or long term lease. Because land has both an economic and an “emotional” (sometimes also spiritual) value to local communities, its takeover by large companies can cause significant resentment, particularly if the company is foreign-owned. These issues are not confined to developing countries but have been subject of debate for instance in New Zealand at the time of privatisation of state forests (Roche and Heron 1993).

Compulsory purchase of land to create state forests in Wales, UK, has led to long-term resentment from communities. The situation in many countries is far worse than this, with little if any compensation for loss of land or livelihood.

At an extreme, people have been forcibly displaced from their land to make way for plantations. In particular cases, (e.g. in Indonesia) the government may take land from communities in order to lease it to plantation companies, arguing that this is in the greater national interest (Cossalter and Pye-Smith 2003). Semi-legal processes are used, for instance where land rights granted to local communities were not confirmed with official deeds. Without documentation, communities are at risk of eviction from traditional homelands. In Brazil for instance, a number of indigenous peoples were displaced to make way for a plantation managed by Aracruz, leading to a conflict that has lasted over 30 years (De Nadai et al 2005).

Communities and individuals may be displaced through economic means. Large plantation companies can afford to pay more for land and quickly drive up the price when it is known they are interested in investing in a region (Carrere 2006). This phenomenon is not confined to developing countries but has also been seen for instance in Australia (Kelly and Lymon 2003).

Plantations tend to have to be large to be economically viable. In many instances this will impact on land ownership, creating an increasingly unequal distribution of land with large areas of land concentrated in the hands of a few plantation companies/owners. This can contribute to rural decline. Charnley (2006) describes the example of Kamataka state in India where the government leased out former common land (from which locals derived a range of benefits such as medicines, fuelwood and fodder) to large plantation companies. The result was that locals could no longer access benefits, forcing many to migrate to urban areas.

Political control

Plantation companies tend to be large corporations, which are perceived with suspicion by many local people. There is also often a perception that there is collusion among powerful decision-makers, i.e. donor and recipient governments, plantation companies and even in some cases big inter-governmental agencies like the World Bank or Asian Development Bank, leaving local communities isolated and without a representative voice [reference]. Kanowski (2005) points out that the cultural differences between the private sector profit-oriented mentality of plantation companies and rural communities can also cause problems.

Political control is often tied up with the economic incentives. In some cases considerable economic advantages have been given to plantation companies thus, distorting the market and negatively affecting smaller players. Governments are anxious to encourage investment in plantations to stimulate the rural economy, but end up supporting some of the richest companies in the forest sector, thus attracting criticism on social and political grounds. Incentives may be subsidies (e.g. in Chile or South Africa the plantation sector was maintained many years by large state subsidies), tax breaks or grants. This will be an issue of debate in Uruguay, with the Red Uruguaya de ONGs Ambientalistas de Uruguay claiming that government tax breaks to large plantation companies (exemption from land taxes for plantation areas, no tax on capital gains from tree plantations and duty free importation of equipment and supplies utilized in plantations) will give them a huge comparative advantage over smaller players and also reduce net income to the country by what they estimate to be US$97-120 million.

Forest resources and ecosystem services

While natural forests provide a range of benefits to communities living in and around them, the number of goods and services generally provided by plantations is much more limited. ITTO (2002) suggest that “multi-purpose trees may have an especially important role for local communities”. When plantations are established in areas of natural forests, they may increase the pressure local people place on remaining natural forests. This will not be the case in Uruguay. Potential reduction in availability in water also has consequences. In South Africa reduced availability of water due to plantations is forcing women to walk further to collect water (Mayers and Vermeulen, 2002). There is already a strong perception in Uruguay that plantations have reduced water availability, according to testimonials from local communities collected by the World Rainforest Movement. For example one community leader is quoted as complaining that “because of the eucalyptus trees the Arroyo Negro stream dried up, it used to be the town beach…” (Carrere 2006). At the moment there is little hard data to confirm or refute these claims, meaning that personal observations from local people provides the best evidence we have.

Employment

A common argument for plantation establishment is the provision of work, but in practice the issue is complex. New jobs are certainly created. In Vietnam for instance, The Food and Agricultural Organisation of the UN calculated that by 1991 the forestry sector had provided half a million households with jobs (FAO 1996). According to Cossalter and Pye-Smith (2003), in Congo, the establishment of the Unite d’Afforestation Industrielle du Congo’s eucalyptus plantations employed 9 people per 100 hectares (5000 people for 45,000 ha). The director of the Brazilian Pulp and Paper Association claimed that plantations created 1.5 million jobs in Brazil in a presentation ot FAO in 2005 (Leonel 2005).

However, net benefits are influenced to some extent by the land being displaced and the nature of employment. Jobs may be seasonal. On eucalyptus plantations labour tends to be needed only during plantation establishment and then again during harvesting. On pine plantations more work is necessary as the trees grow, as they generally require more active thinning and tending (Charnley 2006). The question of who gets the jobs is also critical, particularly whether they go to people from local communities or contractors from outside.

Labour conditions on plantations have not always had a good reputation. Some plantation companies have bad safety records, exacerbated by poor training and a lack of safety equipment. Contract labour and outsourcing is a particular concern and over the last decade there has been a general increase in such contractual arrangements. With multiple contractors, wages tend to drop, employment conditions worsen, and it becomes difficult for companies to drive improvements in health and safety. Contractors will require investment in training and careful supervision is environmental and social safeguards are to be maintained. This issue has been highlighted by local communities with respect to Stora Enso operations in Rio Grande do Sul (reference from Helsingen Sanomat)

Social disruption

Many critics of plantations argue that their establishment often has negative impacts on the social structure of the area; for example as a result of an influx of migrant workers which may, if not well managed, cause social tensions. Gender imbalance can occur as numerous young men are brought to an area to supply strong labour. An exercise by the US-based Consensus Building Institute, looking at questions relating to pulp mills in Uruguay, found concern about social change and prostitution arising from migrant workers to be high on the list of concerns of communities in plantation areas (CBI 2005).

Many of these problems are not restricted to plantations, but will affect any large development project; indeed issues of pollution and to some extent land tenure are likely to be more extreme in the case of large food plantations.

The impacts of intensification

The move from traditional plantations to fast wood plantations has increased pressure on natural systems. There are both positive and negative impacts (and many cases where impacts will depend on local conditions and on individual management choices). The table below makes a first attempt to summarise some of the costs and benefits

Table ##: Impacts of intensification on plantations

| Area | Type of impact | Pros and cons |
| --- | --- | --- |
| Energy |  |  |
| Solar energy | Positive: greater amount of solar energy captured by the plantation |  |
| Fossil fuel energy | Depends: more energy used because more frequent planting, harvesting, transport | Net impact depends on the balance between solar energy gain and fossil fuel loss – also latter non-renewable |
| Carbon |  |  |
| Carbon storage | Possibly positive: more carbon stored in higher productivity |  |
| Water |  |  |
| Water use | Possibly negative: more water used in total, risk of always harvesting during the period of maximum use so poor recovery | But modern cultivars are more efficient in terms of water use, impacts can be controlled by careful planning |
| Nutrients |  |  |
| Nutrient use | Hard to generalise: greater use because of faster growth rates | Some new cultivars are more efficient in terms of nutrient use |
|  |  | Depends to a large extent on whether residuals are left in the soil… |
|  |  | …but conversely they can also be used as an energy source |
| Fertilizer use | Usually negative: likely to be higher use, with potential side effects | Some new cultivars are more efficient in terms of nutrient use |
| Silviculture |  |  |
| Pesticide and herbicide use | Usually negative: likely to be higher, with potential side effects |  |
| Social impacts |  |  |
| Employment | Depends: high productivity increases need for labour | But intensification has also brought in mechanisation that can radically reduce labour needs |
|  |  | Some jobs associated with intensive plantations are not high quality |
| Transport | Negative: more movement of timber |  |

Section 2

Conditions in the plantation landscape

Physical environment

Biological environment

Social environment

Introduction

A key task of the analysis was to determine the physical, social, biological and cultural conditions in the area that Sora Enso wishes to plant. This was no easy task: the area is one of the least studied in the country and much of the information that we would have liked to obtain remains unknown. Accordingly, Stora Enso commissioned a collection of local experts from a wide range of disciplines (see Appendix ##) to provide landsape characterisations for the area, identify gaps in knowledge and suggest a number of indiciators for both planning and monitoring. Section 2 describes their main findings, starting with a summary of key points distilled from a wrap-up workshop in March 2007.

Data were collected by experts, who also came together in a cross-disciplinary workshop to discuss results, identify indicators and suggest next steps for the company.

Participants in the workshop on landscape characterisation and identification of indicators, March 2007, Montevideo

Project location area

The area where Stora Enso is seeking to buy land – the “plantation landscape” – covers 31,500km2 or 18 per cent of the surface area of Uruguay

Stora Enso has defined the area of land in which it is seeking to buy farms and other properties to establish plantations – the so-called plantations landscape. As presently delineated, the area of interest is located in the centre of Uruguay, mainly southern Tacuarembó and practically all the department of Durazno along with parts of eastern Paysandú and Río Negro and small areas of northern Flores and northern Florida. This area represents 18 per cent of the total surface of Uruguay (31,500 km2 approximately) and includes some or all of four basins: Queguay, Tacuarembó, Merin Lagoon and most of the basin of Río Negro (Achkar et al., 2004): Figure ## below shows the polygons currently being explored for possible plantation establishment.

The plantation landscape is still only approximately fixed and has changed during the developed of the analysis; it is possible that it will change again in the future, in which case the analysis in this report will no longer be complete.

Figure ##: Current definition of the plantation landscape

Not all the plantation landscape is suitable for trees: further refinement will result from analysis of soils, hydrology, proximity to settlements and availability of land. Refining the area of interest for Stora Enso is one of the aims of the current project. Constraints exist at a landscape scale – for example whole areas where soil or geography is not conducive to planting trees – and more local constraints even within individual properties. Several properties have already been purchased and most of these have been visited during the assessment.

The area includes seven of the eleven landscape units defined for the Uruguayan territory: basaltic hill ranges, crystalline-metamorphic hill ranges, sedimentary and basaltic hillocks, sedimentary, crystalline and basaltic hills and hillocks, as well as lakes and lagoons.

When individual sites have been bought, they are also mapped by the company

The Physical Environment

The success or failure of the plantation will depend largely on making sure that the environmental conditions are optimal; this will also affect to a large extent the degree to which the plantation has wider impacts on water quality and quantity and on the general conditions of the area. Critical issues relate to the geology and particularly hydrogeology and the soils. The research team therefore contracted a number of specialists to look into these issues in detail and report back on the strengths and weaknesses of the landscape under consideration and on any particular steps that the company needs to take to maximise chances of success. Their work in summarised in the section below.

Geology

Geology of the plantation landscape

The Northern Uruguayan Basin is found in the northwestern, northern, northeastern and central region of the study area. It is part of the Chaco-Paraná Basin, which covers 1,500,000 km2 in Argentina, Brazil, Uruguay, and Paraguay. Rocks from the Precambrian Era are found in the southwestern, southern and southeastern region of the study area, in the territories Piedra Alta and Nico Pérez, separated by the shear zone Sarandí del Yi-Piriápolis. Figure ## shows a geological map of the area (modified from Bossi et al 1998).

The temporal and geographical locations of the geological formations that make up the subsoil of the study area are briefly described in the box below.

Issues relating to site selection for plantations

Two issues appear to be of particular importance:

- Identification of areas of mining interest for example either for the exploitation of kaolin in the Formation Cordobés or collection of ornamental rocks and gravel from rock fragments in the crystalline basement. Potential mining areas may overlap with areas of interest for forestry: the government has right of compulsory purchase for mining so it may be worth avoiding planting in these areas.

- Identification of areas with calcareous sediments from Cretaceous materials, which generate soils with high contents of calcium carbonate (blanqueales - whitish lands) that may hinder or prevent the normal growth of the trees and again are to be avoided.

To check:

1. Why are high calcium soils bad for trees?

2. How many areas might there be a possible clash with mining?

Box: stratigraphy of the study area

Precambrian (south): made up of low-grade metamorphic belts, corresponding to hard, tough rocks found in rocky outcrops or covered by poorly developed soils.

Devonian Formations (southern-central and southeastern area): sediments up to 300 m thick and separated into three formations that reach the surface, described from bottom up:

- Cerrezuelo Formation: mainly coarse to fine grained arkoses and kaolinitic pelites

- Cordobés Formation: predominantly light gray to dark gray pelites

- La Paloma Formation: including pelites – more frequent at the base – and fine conglomerates at the top of the formation

Carboniferous-Permian Sequence (Carboniferous-Permian) (eastern and southeastern area): After a new erosive period in the Late Carboniferous, sedimentation began in the Northern Uruguayan Basin, represented by the San Gregorio, Tres Islas, Melo and Yaguarí Formations, mainly from the Permian and Late Permian Periods and the Buena Vista, Tacuarembó and Rivera Formations from the Triassic and Jurassic Periods:

- San Gregorio Formation: from a glacial environment, made up of resedimented diamictites, tillites, and in lower proportions, by clast-supported conglomerates

- Tres Islas Formation: making a transition to a deltaic and fluvial environment, represented by fine to very coarse psammites, sometimes slightly sabulitic, with significant although variable percentages of pelites and lower amounts of psefites

- Melo Formation: representing a new marine transgression, predominantly sandstones and heterolitic, with small amounts of politic and psefitic facies

- Yaguarí Formation: (massive or layered psammites and pelites – a lower deposit made up of fine materials and an upper one of sand grains

- Tacuarembó and Rivera Formations (Jurassic-Cretaceous) (south and north-south band in the central area): the first contains fine and very fine sandstones and pelites of light colors; the second eolic sandstone in dune structures.

- Arapey Formation (Upper Cretaceous) (west, northwest, north, south and center of the area): the sedimentary sequences are partially covered by deep basaltic defiles corresponding to the Mesozoic Magmatism, occupying most part of the study area.

- Guichón and Mercedes Asencio Formations (Cretaceous) (western area): thick layers deposited during the Cretaceous in the Northern Basin. The Guichón Formation reaches the surface in outcrops in a very limited pat of the central eastern area, made up of conglomerated to pelitic sandstones. The Mercedes Asencio Formation consists of granules that vary from fine sandstone to conglomerates, with cement usually clayey and calcareous with limestone lenses. Ferricrete (iron-cemented conglomerate) is present in a significant part of the area.

The Cenozoic cover is represented by the Fray Bentos Formation and by recent and current deposits.

- Fray Bentos Formation (Miocene) (western area): silty rocks with variations in the content of clay, fine sand and calcareous material

Key

Recent and actual

Fray Bentos (Miocene)

Mercedes-Ascensio (Cretaceous)

Guichon (Cretaceous)

Arapey (Cretaceous)

Tacuarembo (Jurassic superior)

Yaguari (Permian superior)

Melo (Permian)

San Gregorio (Permian inferior, Carboniferous superior)

La Paloma (Devonian)

Cordobes (Devonian)

Cerrezuelo (Devonian)

Terrencio Piedra Alta (Pre-Cambrian)

Centuron Arroyo Grande (Pre-Cambrian)

Cizalla Sarandidel (Pre-Cambrian)

Batolito de Illescas (Pre-Cambrian)

Valentines (Pre-Cambrian)

Rivera (Jurassic superior, Cretaceous inferior)

Hydrology

The study area includes part of the Hydrogeological Provinces Paranaense and Meridional. Both porous and fissured aquifers are found in the area. Each province is divided into sub-provinces, in accordance with the hydrogeological behavior of the subsoil materials. The map in Figure ## shows the productivities of the different aquifers, classified according to the specific flow (q) of the perforations, reflecting their importance in relation to their potential.

Issues relevant to plantation development

There are three critical issues from the geological point of view.

- Eucalyptus or pine plantations will reduce the net recharge of aquifers: both by interception of the recharge water by the vegetable mass and also because of increased water consumption in the plantation area. This is noticeable in free aquifers with saturation levels near the surface, and less evident in semiconfined aquifers; there is no noticeable effect in confined aquifers. In free aquifers, recharge areas are the most affected. Therefore, recharge areas need to be identified in order to avoid a negative impact on the water resource. In this case the most vulnerable aquifers are Cerrezuelo, Guaraní Aquifer System, and areas of fissured aquifers.

Potential indicators for planning plantations and conserving groundwater

The following elements contain potential indicators for the conservation of underground waters:

- Potentiometric map

- Monitoring of potentiometric levels

- Chemical monitoring

Soils

The soil type will help to determine the location of plantations. The government has classified all soils in Uruguay into three main classes depending on their proposed use. From the perspective of forestry, these are roughly equivalent to:

- Forest priority soils – where forestry is most encouraged; these are generally poorer soils less suitable for agriculture

- Flexible class – where forestry is not a priority but there is flexibility or afforestation if needed

- Agricultural soils – where forestry is not recommended (although there is no complete prohibition).

There is a debate about whether or not non-forestry soils should be used for forestry: in theory this could boost tree production because at present trees are kept to the poorest soils, reserving good soils for crops. Adherence used to be driven mainly because grants were only available for plantations on designated soil types. The situation is further complicated because soils that are good for agriculture are not necessarily always the best for trees.

Landscape characteristic of soil units in the plantation landscape

The different soil types in the plantation landscape are described, and for the purposes of analysis are ordered by means of two transects: crossing the department of Paysandú from west to east through the town Lorenzo Geyres and the department of Tacuarembó through the town of Clara; and the second crossing the department of Durazno from west to east from the Palmar Dam. Table ## below outlines the main soil types and their characteristics.

| Unit | Relief | Soils | Notes |
| --- | --- | --- | --- |
| Northern transect: from west to east, Paysandú-Río Negro-Tacuarembó |  |  |  |
| Algorta | Gentle slopes (up to 5 per cent) | In extended convex gentle slopes mainly Planosoles Dístricos and Argisoles Dístricos. In shorter convex slopes mainly Argisoles Dístricos. In extended and gentle slopes of higher areas mainly Brunosoles Subeutricos | Clear effects of past erosion from agriculture; moderate erosion risk |
| Bacacuá | Plateaus with gentle slopes and other well-defined steep areas, also rocky hillocks | In higher areas, soils covered by Quaternary formations are Brunosoles Subeutricos (sometimes Eutricos). Brunosoles Pseudolíticos and Litosoles are associated. Argisoles of the Unit Algorta are common in convex slopes. In stronger slopes, there are Argisoles of the Unit Chapicuy | Erosion risk is generally quite low except on steep slopes where it is severe |
| Chapicuy | Strong well-defined convex slopes and high concave valleys. | Most commong soils are Brunosoles Subeútricos with Argisoles Subeútricos. In the E and S Argisol Dístrico occurs. Well drained soils, characteristic red colour | High erosion risk due to slopes and degradation of the superficial horizon. Recently excluded from forestry priority |
| Curtina | Basaltic soils: no forestry priority soils |  |  |
| Paso Palmar | As Bacacuá, but for larger scarpes more resistant to weathering | Soils as for Bacacuá |  |
| Itapebí-Tres Árboles | Basaltic soils, no forestry priority soils |  |  |
| Queguay Chico | Basaltic soils, no forestry priority soils |  |  |
| Curtina | Basaltic soils, no forestry priority soils |  |  |
| Tacua-rembó | Sedimentary non-rocky hillocks, with convex slopes (up to 9%) | The deeper soils, sandy soils are Luvisoles Ocricos and Acrisoles Ocricos found in the hillocks and higher areas of the slopes. Associated soils found moderately deep in areas with well-defined relief are Inceptisoles and Litosoles, Planosoles in steep slopes |  |
| Rincón de Zamora | Landscape of well-defined hillocks, sometimes short-medium slopes (up to 10%) | Sandy soils in strong convex slopes and non rocky hillocks are Luvisoles Ocricos and Acrisoles in gentle and steep hillocks (up to 5%) Ocricos, Argisoles Dístricos; in steep slopes Luvisoles Melánicos and in steepish slopes (up to 6%) Brunosoles Subeutricos | Little effect of past erosion but a high erosion risk, mainly due to steep slopes, light texture and weak structure in superficial horizons. |
| Río Tacua-rembó | Landscape of plains in high, medium and low areas; Significant native riverside forest. | The predominant soils are Gleysoles Lúvicos and Planosoles Dístricos; also Solonetz Solodizados and Fluvisoles Heterotexturales. In the area of the mouth of the Río Tacuarembó and Río Negro, there are Arenosoles that form dunes. | No effect of past erosion and no erosion risk. No forestry priority soils and also identified as important to include in system of protected areas. |
| Southern transect: from Paso Palmar, crosses Durazno from west to east, to the west end of the department of Cerro Largo |  |  |  |
| Bacacuá | Large isolated plateaus with steep-sloped rocky scarps in the borders of large hills. | The soils above the rocky scarp are Argisoles Dístricos. When there is Quaternary formation, soils are Brunosoles Eutricos. In areas where the Quaternary formation is deep the Merino Unit occurs and no forestry priority | No effect of past erosion and low erosion risk in high areas, above the scarp and very high risk in the slopes below the scarp |
| Queguay Chico | Basaltic soils, no forestry priority soils |  |  |
| Baygorria | Basaltic soils, no forestry priority soils |  |  |
| Curtina | Basaltic soils, no forestry priority soils |  |  |
| Itapebí-Tres Árboles | Basaltic soils, no forestry priority soils |  |  |
| San Jorge | Strong slopes, and gentle slopes when there are extended watershed areas | The soils are Argisoles Dístricos in strong convex slopes and Argisoles Subéutricos in watershed areas | Little effect of past erosion, although large gullies occurin some areas due to a combination of geological process and overgrazing. High erosion risk. |
| Manuel Oribe | Landscape of non-rocky sedimentary hillocks, except in the north, where sedimentary hillocks are slightly rocky | The soils are Brunosoles Dístricos and Luvisoles Melánicos in long, gentle slopes. Luvisoles Melánicos and Inceptisoles Ocricos occur in stronger slopes and superficial Brunosoles Dístricos and Luvisoles Melánicos in slightly rocky hills | Low effect of past erosion. Significant degradation of superficial horizons of the soil, resulting in degradation of the vegetable cover. High erosion risk; conservation measures needed. |
| Blanquillo | Landscape of strong and gentle slopes | The soils are Brunosoles Subeútricos and Argisoles Subeútricos in strong slopes and Vertisoles Rúpticos in gentle slopes. | There is a slight effect of past erosion due to agriculture. High erosion risk. |
| Aparicio Saravia | Landscape of non-rocky hillocks and a few slightly rocky hillocks | The soils are Luvisoles Ocricos (hillocks nearby the Río Negro river); Luvisoles Ocricos and Acrisoles Ocricos (in slightly rocky hillocks between Blanquillo and La Paloma) and Luvisoles Melánicos (near Arévalo). | There is no effect of past erosion. There are some large gullies, but not caused by agriculture. Very high erosion risk |
| Tres Islas | Steep hillocks and strong slopes | The soils are Luvisoles Ocricos in the medium part of convex slopes, Litosoles in the high areas (usually with outcroppings) and Brunosoles Subéutricos in low areas of the slopes. | There is no effect of past erosion but a a very high erosion risk. |

The main soil types in the plantation landscape are outlined in the map in Figure ## below.

Figure ##: Map of the main soil types in the plantation landscape

Critical issues for soils under forest plantations

Forest soils face three main threats from plantations, i.e. risk of

- Increased erosion

- Loss of fertility

- Structural degradation of the soil

Soil pit dug for analysis in Stora Enso plantation in Rio Grande do Sul, Brazil

.

Nigel Dudley

Following the same two transets used in defining soil characteristics, critical threats are identified for each of the soil types in Table ## below.

Table ##: Critical issues for soils in the plantation landscape

| Unit | Critical issues |
| --- | --- |
| Northern transect |  |
| Young | Not forestry priority |
| Algorta | The physical properties of the soils unit make them suitable plantations but erosion risks remain and there has been moderate erosion in the past. Virtually the whole unit has been used for agriculture at some time. It will be necessary to adopt and implement soil conservation measures: contour planting, natural drainage areas covered with grass and terraces plus restoration of areas affected by erosion and gulleying plus restoration of corridors (firebreak lanes, extraction paths, etc.) to restore campos. Yatay capitata, the palm, is common in the area. |
| Bacacuá | The critical issues in this unit are the scarpes, unique geological formations that are outstanding landscape features.Yatay capitata palm trees grow in the scarp. The same considerations should be applied to this unit. |
| Chapicuy | Excluded from forestry priority lands but does contain some foretry priority soils. Slopes contain serious erosion including gulleys that prevent use of conventional machinery. Soil erosion is a critical risk and conservation measures will be needed. Yatay capitata occur. |
| Curtina | The critical issue at a landscape levels is the existence of excellent quality natural grassland with an important germplasm bank. |
| Paso Palmar | A forestry priority unit, covering 267,311 ha. Moderately suitable for plantations: soils have restrictions such as sandstones portions in the profile, horizons with high clay and presence of calcium carbonate, usually over 80 cm. Good water availability but also fissures leading to aquifers [recharge?] There is no forestry priority In the upper part of the unit but above the scarpe border there is a unit without forestry priority formed of agricultural soils Brunosoles Eútricos, which may be suitable for plantations. If large areas fall within plantation, they should be included as environmental units. Erosion risk is high in the slopes below the scarpe and very high when those slopes are concave. All planting should use contour lines with controlled drainage. Yatay capitata appear in the gorges of the scarpes or in the fissures of the stones where they are protected from grazing. It is important to take into account the landscape value of these plateaus |
| Itapebí – Tres Árboles | No forestry priority land |
| Tacua-rembó | The effect of past erosion is not a generalised phenomenon. A few deep gullies are found in former agricultural land, particularly crops like watermelon, pumpkin and potato. As the superficial horizon is usually over 60 cm deep, corrective measures such as contour planting can be used. Soils are acid and low fertility; management should "contribute" rather than "extract". It is not possible to use this unit without a strict soil conservation system. All crops must use contour planting with parallel lines, making corrections when there are changes in the slope. The area used for the corrections may be devoted to paths or free areas. In Luvisoles, with slopes above 8%, terraces designed for each situation must be used. The strip below the canal may be used for extraction paths, as it is drier. |
| Rincón de Zamora | All soils in the Unit have similar characteristics in the superficial horizon, with a different percentage of sand and are fragile, with superficial horizons of 30 cm on average, light textures and a weak structure. Therefore, erosion risk is high and soil conservation measures must be applied. Contour planting should be used and natural drainage areas be covered with grass. Furrows should not end with a transversal section close to the natural drainage area. When the slope is higher than 8%, use of terraces is recommended. |
| Río Tacua- rembó | The unit has no forestry priority. |
| Southern transect: units described in the section above will just be identified |  |
| Bacacuá | See above |
| Queguay Chico | Unit with much superficial basalt. No forestry priority land |
| Baygorria | Unit with much superficial basalt. No forestry priority land |
| Curtina | See above |
| Itapebí – Tres Árboles | See above |
| San Jorge | A critical issue is the high erosion risk resulting from light texture and weak structure in the superficial horizon. Therefore, when using the resource, contour planting and natural drainage areas with grass must be used. |
| Manuel Oribe | The critical issue is the low or very low fertility, although plantations do not usually require special care in this respect. Erosion, and therefore further loss of fertility, is a very important issue. If the erosion risk is controlled, the fertility loss risk will also be under control so that a strict soil conservation system, using contour plantation and drainage areas covered with grass, should be used. Soil protection should take place at the superficial horizon and vegetation cover. When erosion reaches underlying material, large gullies are produced. |
| Blanquillo | The soils have a high erosion risk and the same management considerations made for the soils in the Manuel Oribe Unit should be applied here. Most of the soils are very similar and the landscape is the same, therefore, the critical issue is the erosion risk. |
| Aparicio Saravia | The soils always have a layer of pebbles between the horizons A and B. Critical issues to be taken into account are erosion, low fertility and risk of structural degradation. These risks suggest use of soil conservation systems. Contour designs are also important in these soils to improve the storage of water in the profile. |
| Tres Islas | The critical issues are the erosion risk and the low fertility. Therefore, the crops must be planted using soil conservation systems, mainly contour lines and drainage areas covered with grass. |

Risk impacts from plantations and monitoring change

Transformations in the uses of the soil are important because they are permanent and irreversible. The resulting impacts on the human population in surrounding areas, or in more distant areas, should be taken into account. This effect may be beneficial, and efforts should be made to ensure that this is the case. Sustained production for forestry projects infers a system and an intensity intensity of use of either a natural resource or an ecosystem without degrading it, i.e. keeping its characteristics and values inact and functioning.

Impacts on soils and water resources are two of the risks associated with plantations that generate the highest levels of controversy. They are also areas where careful planning and management can mimimise risks, so a process to monitoring biophysical characteristics of the soils and water over time offers a way of both reassuring stakeholders and, if necessary, taking corrective action if problems emerge.

In order to have information on the progress of the project and to monitor the modifications on the maintenance of the ecosystem quality, it is necessary:

- To establish a net of sites to measure variations in the soil under different production systems (management of the soil resource under the project and under the current natural conditions).

- To establish a system to determine the productivity variations for each production system (maps based on remote sensors and field verifications to incorporate a GIS of the area of interest)

Both of these will require the identification and measurement of key indicators. Physical, chemical and biological indicators are considered.

- Physical indicators: indicators that refer to soil physical properties, mainly in the superficial horizon.

- The texture indicates the land use suitability, the water storage capacity, the erosion risk and the degradation risk.

- The structure provides information to assess the erosion risk and the time suitability for tillage.

- The assessment of the structural stability is a very versatile indicator and provides a measure that can be compared with exclusion situations. The analysis of this indicator implies a careful sampling programme inside and outside the project.

- The superficial horizon depth (when it is possible to compare with an ideal situation nearby) is an accurate indicator of the effect of past erosion. It may also participate in the assessment of the land use suitability and the water storage capacity

- Chemical indicators: parameters that affect directly the development of the project or the soil degradation risk should be taken into account as indicators.

- Soil acidity is a universal indicator. The crops indicate the acceptable range for a successful implementation and development

- The existence and depth of calcium carbonate may be an effective indicator, as it may limit the implementation of a crop or variety

- The exchange capacity is a very valuable indicator, as it indicates the profile fertility. The systematic use of this indicator implies having access to a laboratory with the equipment needed for this analysis in the long term

- Biological indicators: linked to the percentage of soil organic matter, a parameter that has a strong influence when establishing a plantation. Other biological indicators must be considered in the framework of multidisciplinary teams, as multiple organisms are part of the ecosystem at soil level

The Biological Environment

Issues relating to biodiversity and plantations are perhaps the most important of all in terms of global opinions but paradoxically the least of the concerns amongst many stakeholders within the country. This is by no means an unusual situation for as a global company with their home base in a country with a high level of awareness of conservation.

The following analysis starts by looking at the ecological history of Uruguay, to provide some background about the type of environmental conditions likely to be found and then proceeds to a series of landscape characterisations from the perspective of major flora and fauna groups. The study focuses on those plant and animal groups likely to be of most interest and also most representative of the ecosystem as a whole.

Senecio mattfeldianus

Ecological history of Uruguay

Given that plantations will cause major changes to ecology, it is important to understand the changes that have already taken place to the ecology of the country. Knowledge of Uruguayan ecological history remains sketchy and the following account covers the whole of the country.

As far as we can tell, most of Uruguay is naturally dominated by scrub and grassland ecosystems, with much smaller areas of high forest and wetland. Natural grasslands are diverse and ecologically important, although centuries of agriculture have changed their ecology and degraded their environment – the original ecosystem would have looked radically different to that found today. Although over-grazing is undoubtedly a major pressure on remaining natural grasslands, complete absence of grazing is also likely to lead to losses of diversity in herbaceous species.

Uruguay is naturally dominated by scrub and grassland ecosystems

Grassland

The campos grassland is a sub-humid sub-tropical grassland ecosystem with few large trees occurring naturally except close to rivers and streams

The campos grassland is a sub-humid sub-tropical grassland ecosystem with few large trees occurring naturally except close to rivers and streams. It lies between 24°S and 35°S; and includes parts of Brazil, Paraguay and Argentina, and all of Uruguay, covering approximately 500,000 km2. Most of the area falls within the Uruguayan savannah ecoregion, a 355,000 km2 area covering parts of southern Brazil, northern Argentina and most of Uruguay (Locklin 2001). In addition to the dominant grassland, the region contains a mosaic of gallery forests, palm savannas and out-cropping of sub-montane forests. Within the campos there are various different grassland habitats, determined by soil properties, water content and elevation (although the whole country is low-lying), with species and ecology now also heavily influenced by grazing pressure and, at least historically, probably also by the presence and frequency of fire. Generally, grassland today varies between tussock grass and short grass communities, with the latter generally being encouraged by heavy grazing. Different species mixtures are likely to remain depending on whether sheep or cattle are on the pasture. There is a marked difference in species between the north and south.

Rainfall is evenly distributed through the year but subject to marked annual fluctuations, with high humidity in the spring and autumn but evapotranspiration exceeds rainfall during the summer.

The Uruguayan grasslands support about 400 species of annual and perennial grasses, making the area exceptionally rich in grass species including some 4 per cent of the global total. There are also numerous herbaceous species. However historical grassland would not have looked much like the pasture found in most of the country but would instead have been dominated by shrubs growing up to 3 metres tall. The dominant shrub species return very quickly if grazing stops or is reduced and can be seen emerging quickly in abandoned fields and along roadsides, where they have to be controlled. The areas now dominated by tall or short grasses would in the past have resembled more closely a savannah as it is understood in Africa. Exceptions, where short grass species would have remained dominant, are found naturally on some particularly wet soils or very thin solids where larger shrubs have difficulty in establishing.

Woodland

Natural woodland now only covers 3.8 per cent of the country

Woodland is the other major habitat in the country, but is comparatively limited in extent. Much woodland has been lost as a result of human actions (Carrere, 2001) and natural woodlands now cover only 3.8 per cent of the country (670,000 hectares) according to a survey carried out in 2000 (FAO, 2000). Both natural woodland and palm savannah are protected, although the latter is currently suffering pro  blems from lack of regeneration due to cattle grazing (Locklin 2001).

There are 74 mammal species in Uruguay along with 404 birds, although both groups tend to be concentrated in wetlands and river forests (Nores et al, 2005); around 80 species of birds are found in grassland (Locklin, 2001).

Ecological history

Comparatively little is known about the ecological history of Uruguay prior to the introduction of cattle and horses at the beginning of the 17th century. Reports by travellers such as Charles Darwin suggest that already by the beginning of the 19th century there were few forests except along rivers and the country was dominated by a savannah landscape with occasional trees and shrubs.

A limited amount of pollen analysis has been carried out within the wider ecoregion, suggesting that the ecology is naturally dominated by grassland, albeit now heavily modified by human impacts. A study in the lowlands of Rio Grande do Sul in southern Brazil, in a similar habitat to that of the grasslands of Uruguay, concluded that that the Campos region was naturally covered by savannah grassland during the full- and late-glacial periods, during which time riverside forests were absent except for a few isolated trees along rivers, and in the post-glacial period. Riverside forests started to spread during the late Holocene, both along rivers and also in some wetter, lowland areas, reaching a maximum extent around a thousand years ago. But by then there were also humans living in the region and records suggest a marked increase in fires at this time, which were probably anthropogenic, although an increase in both temperatures and in frequency of dry periods during this time would have also favoured more fires (Behling et al, 2005).

Land use

About 80 per cent of Uruguay is used for cattle ranching on both natural and increasingly also on artificial grasslands. In 1996 there were an estimated 13 million cattle in Uruguay (Pallarés et al, 2005). Natural grassland (neither cultivated fields nor artificial grasslands) occupy approximately 14 million hectares (Biodiversity Planning Support Unit, 2001). Increasingly these are being replaced by exotic species and artificial grasslands increased by 32 per cent between 1980 and 1990 in Uruguay (Republica Oriental del Uruguay et al, 1992). Natural grassland that has retained a wide range of species is particularly valuable from a biodiversity perspective. There have, as yet, been few studies comparing richness of animal diversity with naturalness of grassland. Exotic plants, including many from Europe, have also been introduced and in some cases these have changed the nature of the grasslands.

The evolution of the Uruguayan campos since the days of the first European settlements is said to involve 3 main stages (Panario and Bidegain, 1997):

- 'hardening' as a result of the introduction of livestock

- 'refinement' by the action of fire and overgrazing

- further degradation, including preferential development of warm-season species, due to persistent overgrazing

Over-grazing has changed the ecology even in many apparently natural grassland habitats. In addition it was estimated that 53 per cent of agricultural land received application of fertilizers in 1992 (Republica Oriental del Uruguay et al, 1992) and application of pesticides is also increasing.

Restoration of original habitats

Given the continued lack of detailed knowledge about vegetation history, full restoration is not currently possible. However, several experimental studies have excluded grasslands from grazing for a period of years and the impacts have been recorded.

The bushes that would naturally dominate much of the campos spring up wherever there is no grazing pressure

An area of grassland was fenced off in 1984 to exclude grazing from land that had previously been grazed for centuries. Short grasses declined and taller bunch-grasses emerged along with shrubs. The continued exclusion of grazing led to increased litter accumulation, which changed the moisture retention capacity of the soil and, coupled with the effects of taller grasses, modified the microclimate (Pallarés et al, 2005). A nine-year exclusion project on natural grassland found similar changes towards tussock vegetation and also an increase in annual species, while there was also an increase in annual herbaceous species and nitrogen-fixing species. However, general species richness and diversity decreased over the period (Rodriguez et al, 2003). These results were later confirmed by comparing with natural and artificial grazing (Altesor et al, 2005). A survey of seven continuously grazed and ungrazed paired plots included analysis of impacts on soil, fauna and flora. Species numbers were lower in ungrazed plots (with 70 species in enclosures and 105 in grazed areas). Grazing considerably altered species composition, with an increase in prostrate, warm season species and a decrease in shrubs with grazing. (Altesor et al, 2006).

Virtually all the grassland in Uruguay has been changed by cattle and sheep grazing; in natural conditions there would have been far more scrub and bushes.

Although virgin campos (grassland which has never been under crops) is identified as a distinct habitat type, it is said to be indistinguishable from former cropland once the latter has been through a ten year recovery period, suggesting a high level of resilience within the system (Boggiano per comm. 2007)

Uruguayan scientists are building up an understanding of the changes that are likely to have occurred through introduction of cattle (which have to some extent displaced natural grazers such as deer) and of optimal levels of grazing to maintain species diversity. A full understanding of ecology before European settlement is by now probably impossible and in any case it seems likely that humans have been present since current climatic conditions existed so that a “pristine” environment without human influence has probably never existed in the form it would today.

Protected area network of Uruguay

The protected areas system is under-developed with a reported 0.4 per cent of land in some kind of protected area (Earthtrends 2003), although it is not clear if this figure includes proposed protected areas as well. There is currently a commitment by the Government to development the protected areas system, which is included in its goals under the Millennium Development Committee, and an implementation grant from the Global Environmental Facility has been received.

Existing and proposed protected areas

Many of the current protected areas are small, with the exception of a large Ramsar site to the far east of the country, which is under multiple use practices. Most protected areas have not been assigned an IUCN category of management objective (there are six categories distinguishing management types ranging from strict protection to multiple use) and details on the World Database on Protected Areas remain sketchy. Uruguay has protected areas listed under the Ramsar Convention (for important wetlands), UNESCO Man and the Biosphere programme and UNECO World Heritage sites. There are also a number of state and private protected areas. No major protected areas fall within the plantation landscape. Details of existing protected areas are summarised below.

Ramsar: there are two wetland siteslisted under the Ramsar Convention.

- Bañados del Este y Franja Costera in Rocha covers 407,408 ha, sharing a border with Brazil; a vast complex of coastal wetlands including lagoons and parts of several rivers. It is important for locally nesting shorebirds and migratory shorebirds. It was added to the Montreux Record, indicating that the Ramsar Committee regards it as being at risk, on 4 July 1990. The site is also the only UNESCO Man and the Biosphere reserve in Uruguay.

- Esteros de Farrapos e Islas del Río Uruguay covers 17,496 ha in the lower Uruguay River. The site consists of alluvial areas on the river's eastern bank and 24 islands, which are occasionally flooded. The system is important for the control of floods and erosion. Elevated sand bars support forests and permanent and intermittent freshwater pools. The larger part of the area is owned by the State, and its most abundant use is extensive summer cattle grazing, although there is coal, horticulture and citrus production as well.

World Heritage: there is only one World Heritage site in Uruguay, the old town of Colonia, which is a cultural site and therefore not of relevance to protected areas in the sense of biodiversity conservation.

Table ## overleaf summarises details of most of the current protected areas in Uruguay, excepting arboreta and historical sites

The World Database of Protected Areas also lists fifteen more “recommended” sites that have not yet been designated; some of these are quite large and two – Bosques y Banales del Yaguari and Cuchi de Haedo – may fall within the plantation landscape. In addition, it has been proposed that islands and sand dunes in the Rio Negro could be possible candidates for protection, some of which are within both the wider landscape and lands that Stora Enso currenty owns.

Table ##: Protected areas in Uruguay

| Name | Size (ha) | IUCN category |
| --- | --- | --- |
| National parks |  |  |
| - Anchorena | 1,370 |  |
| - Arequita | 1,000 | II |
| - El Potrerillo de Sta. Teresa | 715 |  |
| - Fortaleza de Sta. Teresa | 3,290 |  |
| - Franklin Delano Roosevelt | 1,492 |  |
| - Fuerte San Miguel | 1,553 | VI |
| - Isla de San Gabriel | 24 |  |
| - Islas Costeras | 70 |  |
| - Islas de Río Negro | 1,859 |  |
| - Lacustre | 15,250 |  |
| Wildlife reserve |  |  |
| - Laguna de Castillos | 8,185 |  |
| Reserve |  |  |
| - Isla de las Gaviotas |  |  |
| Protected natural area |  |  |
| - Quebrada de los Cuervos | 366 |  |
| Parks |  |  |
| - Andresito Park | 162 |  |
| - Bartolome Hidalgo | 188 |  |
| Multiple-use area |  |  |
| - Lagunas de José Ignacio, Garzón y Rocha | 15,250 |  |
| Flora and fauna reserve |  |  |
| - Potrerillo de Sta. Teresa | 715 |  |
| Forest reserves |  |  |
| - Arboretum de Lussich | 300 |  |
| - Cabo Polonio y Aguas Dulces | 6,000 |  |
| Municipal reserve |  |  |
| - Quebrada de los Cuervos | 400 |  |
| National forest |  |  |
| - Islas Fiscales del Rio Sta Lucia | 550 |  |
| - Islas Fiscales del Río Uruguay | 6,600 |  |

Private reserves

The idea that private reserves, including those on company-owned land, could be more fully integrated into national protected area systems is one that IUCN The World Conservation Union is anxious to explore.

The IUCN definition of a protected area explicitly recognises the role of protected areas outside state control, including private reserves.

The IUCN definition of a protected area – an area of land and/or sea especially dedicated to the protection and maintenance of biological diversity, and of natural and associated cultural resources, and managed through legal or other effective means (IUCN 1994) – explicitly recognises the role of protected areas outside state control including private reserves, community conserved areas and co-managed areas. The number of company-owned protected areas is increasing rapidly, in part because of obligations by some of the forest certification systems to maintain part of land under protection. Options for including set-aside areas of plantations within protected areas in Uruguay is explored later in the report.

Flora

We have already noted that historical management has created major changes in the ecology of Uruguay. Despite this, a rich flora remains in remnant patches of semi-natural habitats and in the large areas of campos. As part of the assessment, specialists on forests, wetlands and grassland identified key issues of importance.

Needed: All the flora references from the consultants

Background – the flora of Uruguay

The vascular flora of Uruguay includes 2500 species grouped in 811 genera and 150 families (Marchesi 2005). Uruguay has a medium to low richness of endemic species.

Grasslands occupy 70 to 80 per cent of the country; forests and woodlands, 4 per cent and other vegetation types even less. Only around a hundred plant species are classified as endemic to Uruguay and many of these are in fact probably present undetected in neighbouring countries. Although some authorities classify Uruguayan grassland with the pampas found in Argentina, others separate Uruguay and southern Brazil into a different campos sub-region (e.g. Grela 2004), differentiated in part by the presence of more trees and shrubs and also different grass species. Furthermore, within the campos sub-region, Soriano et al (1991) distinguish between the subtropical flora in the north, which are gradually replaced by flora of colder and dryer areas in the south. Grela (2004) suggests a different division based on distribution of woody species, distinguishing the western area along the Rio Uruguay (occidental flora) and the eastern and north-eastern area of the country (oriental flora).

Identifying hotspots for flora

Some initial efforts have been made to identify biodiversity hotspots (Myers et al 2000) using flora. The analysis is primarily based first on the presence of tree and shrub species, because their phytogeography is relatively well understood. From this perspective, the Uruguayan woody flora is a southern extension of the Paranaense Province in the east and a south-western extension of the Chaqueña Province in the west: in both cases being at the limit of the typical species of these phytogeographic provinces. Three forest areas can be identified (see Figure ##):

Chaqueña-Paranense transition

Paranense province

Area of cerrado influence?

Figure ##: Main forest areas in Uruguay

Forests to the west are typical of woodland extending into Argentina, SE Paraguay and southern Bolivia and are thought to indicate a more arid climate in the past.

- To the east in the “domain of seasonal forests” are mainly river forests and Uruguay is again at their southern limit.

- Less well known, is the woody flora of the cerros chatos (flat hills) in Rivera, which may be part of the flora of Cerrado Province in Brazil.

Drawing on data for woody species and inferring a corresponding richness in other groups, the following hotspots have been suggested (Grela, 2004) – see Figure ## below. Using this analysis, it seems that the most important hotspots from the perspective of woody species are not included in the plantation landscape. However, this part of Uruguay is also one of the least studied from a botanical point of view and some of the typical species of areas of endemism may yet be found, e.g. “monte capón” (isolated forest not associated with any water body), riverside forests, etc. Figure ## shows the origin of herbaria collections of woody species and grasses in Uruguay and the lack of information from the centre of the country is very evident. The Stora Enso plantation landscape is also in a transitional area between the flora of the sub-tropical north and the cooler south.

Figure ##: Main concentrations of woody species with restricted distribution (areas of endemism), plus high biodiversity of other species – which can be considered as hotspots (adapted from Grela 2004)

Figure ##: Graphic representation of sampling effort through the number of grass species reported from different areas of Uruguay in herbarium samples. Collection bias of is evident given that this type of species is present in the whole country

For this reason, hotspots are still not a comprehensive tool for planning in Uruguay and decisions about floral richness need to be made on a case by case basis.

Main vegetation habitats in Uruguay

Grassland

Botanists draw an important distinction between grassland that has only ever been used for grazing and grassland that has at some time been ploughed for crops.

All campos is marked by the presence of few trees or shrubs, although shrubs are usually only repressed by grazing. Botanists draw an important distinction between grassland that has only ever been used for grazing and grassland that has at some time been ploughed for crops. Campo virgen pastoreado (virgin pasture) has never been used for crops. The structure and composition of the flora depends mainly on soil type and previous grazing. Annual or perennial exotic weeds are absent or very scarce, sometimes associated with roads, paths or salt feeding places. Rosengurtt (1975), identified several indicator species of virgin campos, as they disappear after the disruption caused by agriculture, e.g. Agenium villosum, Anemone decapetala, Discaria longispina, Dorstenia brasiliensis, Eryngium sanguisorba, Geranium albicans, Pavonia hastata, Psidium luridum, Shizachyrium imberbis, Trixis brasliensis, Criscia stricta. Long-lived perennial species dominate, forming intricate arrangements of tillers and stems. The conservation and management of these habitats is important because they include a significant floral diversity and a great genetic variability within populations of the most important productive species. The proportion of this type of campos has decreased over time. However, a number of transition phases are identified between abandoned crop-land and virgin pasture depending on time since cultivation:

Fields gradually recover natural vegetation patterns a number of years after crop planting ceases

- Stubble: immediately following crops

- New field from stubble: one or two years after crops, when grazing has reduced annual weeds and short-cycle perennial species start to predominate

- Rough field: 4-6 years after crops, long-cycle perennial species have colonised the area. Short-cycle perennial species are still abundant

- Established field: after 10 years it is hard to distinguish from a virgin field (usually by non-biological indicators e.g. ploughing ridges or plot markers) and grazing has reduced or eliminated short-cycle perennial species.

Within grassland habitats a number of varieties are distinguished depending on substrate:

- Campos pedregosos (stony fields) occur where rocky outcrops cover a significant part of the surface, ranging from bare or lichen-covered rock to a stable vegetation cover with a thin soil layer. Stony fields are relatively unstable habitats and liable to evolve into another kind of vegetation types dominated by trees, shrubs, cactus and bromeliad species.

- Campos arenosos (sandy fields) develop on soils with abundant sand, varying from dunes to soils with a relatively high content of organic matter.

- Campos de bañado (wetland fields) are found in flooded areas soil with high permanent or semi-permanent moisture content and include uliginose grassland with deep soils and water available all the year and paludose grassland with high soil water content and longer flooding periods

Forest

As with grasslands, different classification systems have been suggested and the following typology is not fixed as overlap occurs between types.

- River forests or gallery or riparian forests: Strips of woodland associated with rivers and streams. Trees are tall and thick, and the canopy completely covers the ground, sometimes with more than one layer.

- Serrano forests or mountain forests: associated with rocky hills and varying from complete cover to isolated small groups of trees on slopes. Trees tend to be stocky, without defined strata.

- Gorge forests: large trees, several strata, and usually a diverse herb layer, particularly of ferns.

- Maritime forests: xerophytic forests, very stocky and thorny, dominated by short species including some of those present in serrano and river forest. Found on the coastal strip of the Atlantic Ocean and the De la Plata River.

- Mesquite forests: thorny forest with a high layer of leguminous species, plus other trees, shrubs and palms (usually thorny) and usually a large herb layer.

- Palm tree groups: grassland or shrub habitat usually with one species of associated palm. There are two main species: Butia capitata and B. yatay. B. paraguayensis also occurs on small hills and sandstone hillocks.

To check:

The key does not match the text, maybe a translation problem

- Capon forests: small forests located in low areas forming "islands" of forest surrounded by grassland, associated with small ravines or swampy areas. Floral diversity may be very rich.

- Forests of flat hills: associated with sandstone cornices forming flat hills in Rivera and Tacuarembó, trees are usually stocky but not thorny. Butia yatay may be present.

Figure ## shows the area where the main types of woody vegetation are present.

Gallery forest

Gallery forest

Mountain forest

Capon forest

Maritime forest

Forest of rocks and cornices

Forests of flat hils

Whitish lands

Figure ##: Location of main woodland areas in Uruguay. Not shown are riverside forests (present in all the country); groups of palm trees; and small areas of serrano forests associated with crystalline basement outcropping in the centre and south-western areas. Shaded areas indicate places where the vegetation represented is present; it does not mean it predominates. Main plantation area in the box (approximate)

Other vegetation types in Uruguay

- Wetlands and marshes: found mainly in lowland areas with permanent or at least semi-permanent flooding. They are usually highly productive ecosystems with specialised vegetation and their soils have poor aeration and high organic matter content. Much smaller (and also less well studied) areas, which are nonetheless very important from the floral point of view, occur in low sandstone areas in Rivera and Tacuarembó. Small acid wetlands or marshes are formed next to water courses, with the only Uruguayan location of several species more typical of warmer areas of the Amazonic Domain. In addition to a range of vegetation assemblages (many of which will be single-species stands) wetlands also merge into transition habitats..

Vegetation of sandy inland sites is quite specialised and still poorly understood

Pajonales: forming intermediate habitats between campos and riverside forests or wetlands, dominated by large tufted grasses. These areas are frequently burnt to create new shoots for cattle, without apparently causing major damage to the system.

- Herbaceous and shrubby formations: intermediate between grassland and shrubs, common on the flat hills of Rivera and Tacuarembó, often with rocky outcrops. There is more bare soil due to the greater presence of tufted grasses and other upright species with little ground cover.

- Vegetation of sandy sites: sand dune complexes on the banks of the Rio Tacuarembó, Negro and Yi are unusually large for inland areas of Uruguay and contain specialised species with deep roots, stolons and rhizomes. Clear successions can often be seen as vegetation stabilises and changes the soil. Inland dunes remain poorly studied.

- Vegetation of rocky ground: variable habitats with changing micro-climatic conditions and specialised vegetation. Examples of this type of vegetation are the cornices of different geological origins in Río Negro and Paysandú (Cretaceous), Durazno and Cerro Largo (Devonian and Permian), and Tacuarembó and Rivera (Jurassic). These areas are often important from a conservation perspective as they may include field species that have disappeared from other sites as a result of management.

Flora in the Stora-Enso plantation landscape

The area of the proposed plantation is not well studied. What follows is an overview of what is known, followed by some informed speculation about what may be expected within the area but has yet to be recorded.

Main grassland types in the Stora-Enso plantation landscape

Because most vegetation is either unsuitable for trees or protected by law, the main impacts of the plantation, in terms of replacing natural or semi-natural habitats, will be on campos. This is therefore described in greatest detail. The following types occur, described by their CONEAT number based on soil type.

Question

Is this list useful?

- CONEAT Group 1: Superficial basalt fields: the first grasses to colonize here are those that resist the acute summer water deficit, e.g. annual winter grasses and low productivity summer grasses with low dry matter production.

- CONEAT Group 12: Deep basalt fields: vegetation in deep soils is dense and thick, including productive perennial species with little uncovered soil. Summer species cover up to 80 per cent of the ground. Common hard grasses coexist with high quality grasses, requiring care to avoid overgrazing the most palatable species.

- CONEAT Group 10: Deep crystalline fields: common productive grasses dominate in campos on crystalline ground in the centre of the area. Hard grasses are frequent and may predominate. Other herbaceous species are relatively important, but leguminous plants are less common.

- CONEAT Group 5: Superficial crystalline: the most superficial soils have naked soil areas and a high number of winter annual grasses. Sellaginela sp (Sellaginelaceae-Briophyta), Chloris grandiflora, Microchloa indica and Tripogon spicatum are typical species of the initial stages of soil formation.

- CONEAT Group 2: Sierra des Este fields: vegetation is varied, with thin summer grass predominating and shrubs, serrano forest, xerophyte communities and sometimes riverside forest associated. The canopy is open.

- CONEAT Group 7 and 8: Sandy fields: vegetation is mainly composed of perennial summer grasses and some winter species and native legumes.

- CONEAT Group13: Yaguarí heavy field soils: summer grasses dominate in campos on the north-east. Winter species are frequent in slopes with heavier soils. Campos is sometimes are covered by small shrubs or even native trees.

- CONEAT Group 9.6: Fields on cretaceous sediments: Summer species dominate. Despite this, winter forage production is quite high, due to the contribution of winter Cyperaceous, e.g. some Carex species

- CONEAT Group 11: Natural fields with park vegetation: High quality, tender, winter and summer species are both present, combining excellent production of natural grassland with the protection offered by trees. Winter species become more dominant as the internal drainage and fertility increase.

Other types of vegetation in the plantation landscape

Besides campos, the following habitats occur and need to be considered in forest management. Vegetation is highly associated with geology and geomorphology. A good interpretation of geological characteristics will help understanding of flora. The main habitats of interest are listed in Table ## below, along with their locations in the region.

Although the plantation will be established on grassland, conservation of other vegetation types inside the plantation estate will also be important.

| Habitat | Location and notes |
| --- | --- |
| Riverside forests | Found throughout. Those associated with the Rio Negro, Tacuarembó and Yí are the most developed and extended. The springs of the Queguay Grande and Queguay Chico rivers (tributaries of the Rio Uruguay) and the Salsipuedes, Malo and Cordobés streams (tributaries of Rio Negro) are other watercourses with important forests. Other specific forests associated with smaller water courses may also have interest due to particular characteristics (topography, development and conservation status, etc). |
| Serrano forests To check This table conflates 2 in the report – is it OK? | Scarcely represented, may be present in some areas of granitic outcrops in the southern part of the study area (Durazno, Florida, Flores) and towards the east (Cerro Largo). |
| Gorge forests | Occur in Tacuarembó |
| Algarrobales or park forests | Probably present in the eastern side of the study area (Rio Negro and Paysandú) associated with areas of cretaceous sandstone outcrops, although the plantation landscape does not include the largest areas of this type of vegetation. |
| Mesquite forest | The main areas of occurrence are Rio Negro and Paysandú |
| Butia yatay palm tree groups | This particular vegetation formation may be present in establishments located in the western limit of the study area, particularly around Guichón (Paysandú) and also in Rio Negro |
| Vegetation of small hills and rocky ground | Difficult to represent in maps due to the small individual areas involved, but found in areas covered by sandstone formations in the study area in Durazno, Cerro Largo and Tacuarembó (Permian and Triassic sandstones) and Río Negro and Paysandú (Cretaceous sandstones). These areas have high species richness and are of special interest for conservation. It must be noted that the flora associated with each of the two types of sandstone is very different. |
| Capon forests | Similar restrictions apply as above. They are mainly found in the area of Permian sandstones of San Gregorio-Tres Islas, eastern Durazno, Cerro Largo and southern Tacuarembó and are areas of particular interest for conservation. |
| Wetlands and marshes | Wetlands may occupy large areas in lowlands associated with the main water courses already mentioned, or smaller areas associated with natural drainage areas, river and stream-sides or capón forests. |
| Sandy sites | These are mainly associated with the final segment of the Rio Tacuarembó, and the Rio Negro and Yí. They are not always found next to the current banks, but can be large deposits somewhat distant, maybe as a result of paleobeds or mobile dunes. These communities have been little studied from a floral and ecological point of view. However, they are very fragile and therefore have a high conservation value |

Likely additions to the flora

Because the area has been so poorly surveyed for botany, the experts also made some informed speculation about what is likely to occur there but has yet to be recorded.

Most of the area of interest for the project of Stora Enso is outside the main areas of phytogeographic endemism of woody species. However, there are areas of endemism nearby and given the lack of research to draw upon; it is likely that rare and endemic species will be found within the plantation landscape. The following series of paragraphs makes some informed speculation about what is likely to occur there.

- The eastern limit of the “Chaqueña” flora, including Butia yatay palms, reaches the western part of the plantation landscape. Park forests (for instance Prosopis affinis, P. nigra, Aspidosperma -quebracho-blanco-, Geoffroea decorticans, Trithrinax campestris, etc.) and other associated herbaceous species could be present. Surveys carried out in the west have detected rare or restricted distribution species, both in areas of “blanqueales” (alkaline soils) or cornices of cretaceous sandstone, where the flora is completely different from that associated with the sandstones of San Gregorio-Tres Islas, Tacuarembó, and Rivera.

- The northern limit of the plantation landscape reaches the southern limit of the gorges and flat hills of Tacuarembó. Several species typical of the north-eastern sub-area of endemism may be present, for example woody species such as Ilex paraguariensis (Yerba mate), Banara umbraticola, Ocotea pulchella, O. puberula and Quillaja brasiliensis. Personal communications (not recorded in herbarium samples) mention the presence of at least two rare or very rare species of ferns: Dicksonia sellowiana (arborescent fern included in the Appendix I of CITES) and Niphidium crassiflorus of which we have detected only two populations in Uruguay (Sierra de Ríos in Cerro Largo).

- The flat hills in Tacuarembó apparently have a lower diversity than those of Rivera; but they have been less surveyed and some species thought to be exclusive to Rivera may also be present. In this common area between the basalt and the sandstones, the landscape factor must be considered, given that hills and cornices, small gorges, are typical elements of the area.

- Finally, to the east, the area reaches the limit of the gorges and capon forests. Although gorges are mainly present in the granitic areas (outside the study area), these species may occur in the isolated capon forests and gorges in the landscape of western Cerro Largo and north-eastern Durazno (beside the Arroyo Cordobés stream) associated with sandstone formations (particularly San Gregorio-Tres Islas).  These environments are favourable for very diverse vegetation due to the abundant water content. They have not been studied thoroughly from a floral point of view, and many rare or infrequent species (particularly ferns) may be present.

The specialists have surveyed the estates already bought by Stora Enso and mapped the flora, as here at Las Tias

Mammals

A survey was made of the diversity of large terrestrial mammals recorded in Durazno, Tacuarembó, Río Negro and Paysandú, in order to identify hotspots of species richness (Myers et al 2000) and potential indicator species.

It drew on bibliographic material from the Institute of Biological Research Clemente Estable, University of the Republic and the National Museum of Natural History, Uruguay (Achaval et al., 2004, Mones et al., 2003). A diagnosis file was created online for each species in order to update data, including geographical distribution, existing records, and conservation status of the species relating to the IUCN Red List of Threatened Species, CITES and national legislation. the area of the study, morphological features, habitat, and conservation status.

To check

Is this summary right – do I have all the sources?

Distribution of mammal species in the landscape

Of the 119 mammal species recorded in Uruguay, 68 per cent are terrestrial, 5 per cent aquatic and the remainder marine: 60 per cent are classified as large mammals with a body weight greater than 1 kg. In total, the study examined 34 large terrestrial and aquatic species (Achaval et al 2004), all but four of which are native. All the large mammal species are found in the study region and their distribution is outlined in Table ## below.

Table ##: Historical distribution of large mammals in the plantation landscape

| Species To check Is this right? Not all species are present now? | Durazno | Tacuarembó | Río Negro | Paysandú |
| --- | --- | --- | --- | --- |
| D. albiventris |  |  |  |  |
| D. hybridus |  |  |  |  |
| D. novemcinctus |  |  |  |  |
| E sexcinctus |  |  |  |  |
| M. tridactyla |  |  |  |  |
| T. tetradactyla |  |  |  |  |
| A. axis |  |  |  |  |
| B. dichotomus |  |  |  |  |
| O. bezoarticus |  |  |  | Unconfirmed |
| M. gouazoubira |  |  |  |  |
| S. scrofa |  |  |  |  |
| C. brachyurus |  |  |  |  |
| C thous |  |  |  |  |
| P gymnocercus |  |  |  |  |
| P cancrivorus |  |  |  |  |
| N. nasua |  |  |  |  |
| L. longicaudis | Unconfirmed |  |  | Unconfirmed |
| C.chinga |  |  | Unconfirmed |  |
| G. cuja |  |  |  |  |
| P concolor |  |  |  |  |
| O. geoffroyi |  |  |  |  |
| L. pardalis |  |  |  |  |
| L. wiedii |  |  |  |  |
| L. braccatus |  |  |  |  |
| H.hydrochaeris |  |  |  |  |
| S. spinosus |  |  |  |  |
| M. coypus |  |  |  |  |
| L. europaeus |  |  |  |  |

The dark blue color indicates the presence of the species by department: red indicates historical presence but that the species is no longer found there (these data are not available for every species).

According to the IUCN Red List, seven species are Near Threatened; one Vulnerable; and one Endangered. Eight species are listed on Appendix 1 of CITES (Convention on International Trade in Endangered Species), meaning the trade is prohibited because the species is in danger of extinction. Six more are listed on CITES Appendix II, which prohibits trade because the species may become threatened unless trade is controlled. Table ## shows the habitat of the species, whether it is native or exotic and the conservation status in the IUCN Red List and under Uruguayan legislation. The Law 9.481, from July 4 1935, bans the hunting of all mammals except the feral pig (Suis scrofa). A hunting license is needed for the following species: axis deer (Axis axis) and hare (Lepus europaeus) (MGAP, 2007).

Table ##: Habitat, origin and status of large terrestrial mammals in Uruguay

| Species To check Is this right? Seems to contradict some of text. We need to add English / Spanish names | Habitat | Native / Exotic | Conservation Status |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IUCN Red List | CITES | Status in Uruguay |
| D. albiventris | F | N | LR/lc |  | ANRH |
| C. tatouay | F | N | LC | III | P |
| D. hybridus | P | N | NT |  | ARH |
| D. novemcinctus | P | N | LC |  | P |
| E sexcinctus | P | N | NT |  | P |
| M. tridactyla | F | N | LC | II | P |
| T. tetradactyla | F | N | LR/lc | II | P |
| A. axis | F / P | E | LR/lc |  | ARH |
| D. dama | F / P | E | LR/lc |  | ARH |
| B. dichotomus | W / P | N | Vu A4acde | I | P |
| O. bezoarticus | P | N | NT | I | P |
| M. gouazoubira | F | N | DD |  | P |
| S. scrofa | F / P | E | LR/lc |  | ANRH |
| P. tajacu | F | N | LR/lc | III | P |
| C. brachyurus | W / P | N | NT | II | P |
| C thous | F | N | LC | II ?? | P |
| P gymnocercus | P | N | LC | II | P |
| P cancrivorus | F / P | N | LR/lc |  | P |
| N. nasua | F | N | LR/lc | III | P |
| L. longicaudis | A | N | DD | I | P |
| P. brasiliensis | A | N | En A3ce | I | P |
| C. chinga | F / P | N | LR/lc | II | P |
| G. cuja | F / P | N | LR/lc |  | P |
| P concolor | F / P | N | NT | I | P |
| O. geoffroyi | F | N | NT | I | P |
| L. pardalis | F | N | LC | I | P |
| L. wiedii | F | N | V ? | I | P |
| L. tigrinus | F | N | NT |  |  |
| L. braccatus | W / P | N | --- | II | P |
| H. hydrochaeris | W / P | N | LR/lc |  | P |
| S. spinosus | F | N | LR/lc | III | P |
| A .paca | F / P | N | LR/lc | III | P |
| M. coypus | W / P | N | LR/lc |  | P |
| L. europaeus | P | E | LR/lc |  | ARH |

References:

Habitat: F = forest; P = pasture; W = wetland; A = aquatic

Origin: N = native; E = exotic

Red List Categories: LR/lc = Lower risk – least concern; Vu = Vulnerable; En = Endangered; Lc = Least concern; NT = Near Threatened; DD = Data Deficient

CITES (“Convention on International Trade in Endangered Species”): - Appendix I: Trade prohibited (species threatened with extinction); Appendix II: Trade prohibited (species may become threatened if trade is not controlled); Appendix III: Species listed under request of a member country

Status in Uruguay: ARH = Allowed Regulated Hunting; ANRH = Allowed Non-Regulated Hunting; P = Protected

Distribution of mammals in the plantation landscape

The ecosystem with the higher diversity of large mammals is native forest, in particular riparian forest. These support gray brocket deer (M. gouazoubira), crab-eating fox (Cerdocyon thous), crab-eating raccoon (Procyon cancrivorus), coati (Nasua nasua), lesser grisson (Galictis cuja), Geoffroy's cat (Oncifelis geoffroyi), ocelot (Leopardus pardalis) and margay (Leopardus wiedii). In the north, there are records of coendu (Sphiggurus spinosus) associated to "Serrano" forests.

Many species associated with wetland ecosystems are rare in Uruguay, such as the marsh deer (Blastocerus dichotomus), the maned wolf (Chrysocyon brachyurus) and the "pajonal cat" (Lynchailurus pajeros). Aquatic species include the neotropical otter (Lutra longicaudis), capybara (Hydrochoerus hydrochaeris) and coypu (Myocastor coypus).

Status of mammal species in campos

Around half the species in the region use campos some or all of the time, with five species being listed purely as grassland species, eight as inhabiting forest and grassland and a further five wetland and grassland. Four of these are exotic. Status of native campos species is examined in Table ## below.

Table ##: Status of campos species found in the landscape

| Species | English name | Red List Status |
| --- | --- | --- |
| Pasture |  |  |
| Dasypus novemcinctus | Nine-banded armadillo | Least concern |
| Euphractus sexcinctus | Six-banded armadillo | Near threatened |
| Pseudalopex gymnocercus | Pampas fox | Least concern |
| Ozotoceros bezoarticus | Pampas deer | Near threatened |
| Pasture and forest |  |  |
| Procyon cancrivorus | Crab-eating racoon | Lower risk |
| C chingai |  | Lower risk |
| Galictis cuja | Lesser grison | Lower risk |
| Puma concolor | Puma | Near threatened |
| Agouti paca | Paca | Lower risk |
| Pasture and wetlands |  |  |
| Blastocerus dichotomus | Marsh deer | Vulnerable |
| Lynchailurus braccatus | Pajonal cat | Not listed |
| Hydrochoerus hydrochaeris | Capybara | Lower risk |
| Myocastor coypus | Coypu | Lower risk |

The most vulnerable campos species, and therefore the most likely species to be affected by the plantation, are therefore the deer species and puma. The six-banded armadillo is listed as near threatened by CITES but is widely distributed throughout Uruguay. The puma has been recorded in Tacuarembó and in two other regions of the country: Artigas and Lavalleja. It is a rare species, due to hunting because of the threats it poses to cattle production. It is listed as Appendix I by the CITES. The puma is also wide-ranging and unlikely to be seriously affected by plantations; indeed they may provide safe cover.

The most vulnerable campos species and thus most likely to be affected by plantations are the deer species and the puma.

There are only three Neotropical deer species in Uruguay: pampas deer (Ozotoceros bezoarticus), gray brocket deer (Mazama goauzoubira) and marsh deer (Blastocerus dichotomus) (Weber & González, 2003). No viable populations of this last species have been detected in the last 50 years. There is only a record of hunters in the department of Río Negro in 1990 (Pinder & Seal, 1994, González, 2004).

The pampas deer (O. bezoarticus)) was once found widely in the region but has since become locally extinct. It is the most emblematic herbivore of Uruguayan grasslands, but has suffered a dramatic decline probably linked to the alteration, fragmentation and reduction of its natural habitat caused by agriculture and cattle production. This has occurred mainly since the 19th century. Only two pampas deer populations are known in Uruguay today: El Tapado (approximately 1000 individuals) in the department of Salto, and Los Ajos (with 400 individuals), in the department of Rocha. Both of them are listed by the IUCN as "species threatened with extinction". In Uruguay the species has been declared Natural Monument (decree 09/85).

The marsh deer (B. dichotomus) today occurs in southeastern Peru, Bolivia, Paraguay, Argentina, Uruguay, and Brazil at the south of the Amazonian rain forest. Populations are located in the river basins of the main South American rivers to the south of the Amazon: the Río de la Plata basin, the Amazon River and the Río San Francisco River although the current status of the species in the last of these is unknown. The deer prefers floodable areas where the water depth is less than 60 cm, with low vegetable cover. In the State of Río Grande do Sul in Brazil, some individuals from a residual population used fields where rice and corn was planted. The species is threatened by habitat fragmentation (González et al 2001), and from diseases introduced by cattle (Duarte, 2001).

The gray brocket deer (M. gouazoubira) is a species of generalist habits, with low impact regarding hunting and environment modifications (Redford & Eisenberg, 1992), although it suffers a high hunting pressure in all the distribution range. It has been studied in Argentina (Weber and González, 2003), Brazil (Duarte 1998), Bolivian and Paraguayan Chaco and Peru (Weber and González, 2003), and in Uruguay (González, 2004), although there is not enough information to assess its conservation status.

Using mammals as indicators

Large mammals are good indicators of biodiversity. The populations of many species of deer and carnivores have decreased due to the effects of habitat fragmentation and urbanization. As carnivores are at the top of the ecosystem, changes in the population of these species may reflect alterations in the previous levels of the trophic chain, e.g. an increase in the number of rodents. However, they are difficult to observe and often live at low densities.

Fecal DNA isolation

Two non-invasive methods have been used in Uruguay. In order to assess the presence of the two species of fox in Uruguay – crab-eating fox (Cerdocyon thous) and pampas fox (Pseudalopex gymnocercus) – a molecular taxonomic system was designed using non-invasive sampling techniques. The method is based on the DNA extraction of stool. A fragment of mitochondrial DNA from a control region is then amplified and analyzed by PCR-RFLP.

Deer species have been mapped by collecting tissue and stool samples in the following locations: Río Negro, Rivera, Rocha, Maldonado, Tacuarembó and Salto. It was determined that the samples belonged to the species Mazama gouazoubira (González et al 2004a,b) – see Figure ## In addition, a Trail Master remote camera was placed in San Miguel-Rocha National Park and in Valle Edén-Tacuarembó. No photographs of the species were obtained in San Miguel. The presence of a female was registered in Valle Edén, and confirmed by molecular analysis.

Deer are candidate indicator species, particularly Mazama gouazoubira, which is likely to be present in native forests in the plantation landscape. Pampas deer would have been an excellent indicator in pastures, but its presence has not been recorded there for several decades. Carnivores, particularly foxes in the Canidae family and the neotropical river otter (Lutra platensis) in the Mustelidae family are excellent indicator species candidates.  Lutra platensis, in particular, lives in clean waters and is very sensitive to environmental changes.

Mazama gouazoubira

Ozotoceros bezoarticus

Figure ##: Distribution of deer species located using non-invasive indicators

Birds

Uruguay has a relatively rich bird fauna that has on the whole been able to adapt to the changes brought about by management, although loss of habitat has made some species far less common. Today birds face fresh pressures as a result of intensification of agriculture and in particular an increase in pesticides, which are still largely uncontrolled.

Methodology

Data were drawn together on birds of the proposed plantation landscape, based on existing literature, collection data (particularly in the Museo de Historia Natural de Montevideo, and Facultad de Ciencias collections), previous surveys on behalf of Stora-Enso and data from the expert consultants.

Overview

A full list of bird species recorded from the area is given in appendix ##. The following section provides some key statistics.

9%

Common

Less common

Rare

Resident birds: a total of 267 bird species were registered in the study area (75 per cent of the non marine bird species of Uruguay). Of these, 194 (63 per cent) were common species, 54 (28 per cent) less common, and 13 (9 per cent) rare.

Migratory birds: 63 migratory birds have been recorded in the study area including 52 summer migrants and 11 winter visitors: 53 are considered to be common, 18 less common and 10 rare.

The main habitat preferences were concentrated in open landscapes ranging from campos to park-like grassland with scattered trees,. Table ## below summarises information on habitat preferences. If grassland (21 per cent) and park habitats (19 per cent) are amalgamated, it can be seen that more bird species prefer this type of ecosystem than any other.

Table ##: Habitat preferences of birds in the proposed plantation landscape

| Habitat preferences | Number of species |
| --- | --- |
| Grasslands | 55 |
| Park-like grasslands | 50 |
| Water and shores | 81 |
| Woods | 80 |
| Associated with humans | 2 |

Most birds are insectivorous and a breakdown is given in Table ## below.

Table ##: Trophic preferences of birds in the proposed plantation landscape

| Habitat preferences | Number of species |
| --- | --- |
| Insectivorous | 129 |
| Granivorous | 42 |
| Piscivorous | 27 |
| Small vertebrales | 25 |
| Herbivorous | 13 |
| Frugivorous | 13 |
| Invertebrates | 12 |
| Carrion | 5 |
| Nectar | 3 |

The major habitats for birds in the plantation landscape

Three particular landscape types are worthy of note:

The sierra of the south-eastern border of the landscape

This sierra is limited by the National highway number 7 and consists of an undulating landscape with a North/South orientation. The productivity of the land is poor and it is called campos sucios ("dirty lands") with prevailing Eupatorium buniifolium. The main bird species are the Turkey Vulture (Cathartes aura), the Lesser Yellow-headed Vulture (C. burrovianus) and the Black Vulture (Coragyps atratus), the Dusky-legged Guam (Penelope obscura) and the Red-legged Seriema (Cariama cristata), along with some Passeriformes.

Campos

Various grasslands dominate the region. The road network is important in the area. The main landscapes are mosaics, made up of small reservoirs, rice fields, plantations, forage cultivated areas and natural and artificial grasslands.  The main bird species include the Greater Rhea (Rhea americana), Spotted Tinamou (Nothura maculosa), Red-winged Tinamou (Rynchotus rufescens), Southern Lapwing, Rufous Hornero (Furnarius rufus), Field Flicker (Colaptes ampestris), Picazuro Pigeon (Columba picazuro), Shiny Cowbird (Molothrus bonariensis), etc

High Rio Negro and Rio Tacuarembó fluvial plains

The plains are very important from the biodiversity perspective. They arise from a sedimentary system contributed by the two major rivers, including large gallery woods, wetlands, and some sand dunes. The main vegetation are trees and shrubs. The principal bird species of note include the Rufescent Tiger-Heron (tigrisoma lineatum), Purple Gallinule (Porphyrula martinica), Blue-black Grassquit (Volatinia jacarina), and wood bird species like Diademed Tanager (Stephanophorus diadematus), etc.

The potential impacts of plantations on birds of the region

Approximately 40 per cent of the bird species registered in the study area relate in one way or another to grassland habitat and are thus potentially impacted by plantations.

One of the best known species dependent on grasslands is the Greater Rhea, which prefers open grasslands; rhea are omnivorous, preferring vegetables, seeds and fruits but occasionally observed eating small vertebrates (amphibians, small birds or reptiles). Reduction of "natural" grasslands would have negative effects on this species, although by maintaining some open areas and natural corridors the effects could probably be minimized.

Other examples of open grassland species include the Red-winged and Spotted Tinamous. The Red-winged Tinamnou prefers dense grassland areas, being more solitary. The reproduction time in Uruguay extends between September and February and food of choice is insects. On the other hand, the Spotted Tinamou, prefers shorter grass, sometimes being seen in cultivated areas and roadsides. They also eat insects and seeds. Classical plantation approaches can certainly impact adversely on tinamous but impacts can be minimized through the implementation of a forest mosaic approach, maintaining some open areas.

Most birds or prey rely on open territory to hunt and several falcons could be affected. The White-tailed Kite, which inhabits grasslands, cultivated areas, and also urban areas and eats small vertebrates or insects, needs open areas. The Long-winged Harrier is usually seen near water bodies, where they hunt for amphibians, small mammals, birds and eggs. The Cinereous Harrier is considered a "rare" species in Uruguay and their detection in planned plantation areas would necessitate some conservation plans. The Sharp-shinned Hawk is also considered as "less common" in Uruguay with an important link in the grassland ecology (Bildstein & Meyer, 2000), having major impacts on small mammal populations. The Savannah Hawk is also is considered "less common" with distribution is in central/northern/eastern Uruguay. In all these cases, maintenance of open areas is important for survival.

A number of campos bird species, including the greater rhea, will need special attention in management if damage is to be avoided.

Some important migratory shorebirds also pass through the area and rely on coastal lagoons and, of relevance here, inundated grasslands during their stay in Uruguay. The Golden Plover migrates from North America to Uruguay and Argentina and prefers inundated and open grasslands. It is a currently a "common" species in Uruguay and maintenance of these ecosystem types should help to maintain populations. The Tawny-throated Dotterel is considered "less common". Individuals of this species utilise open areas, feeding on insects and invertebrates. The Upland Sandpiper is also a summer visitor to Uruguay from North America and considered as "less common". This species faces some conservation problems principally from loss of habitats, urbanisation and changes in landscape structures (Houston & Bowen, 2001). The Buff-breasted Sandpiper is also a summer visitor from northern Alaska and Canada and is considered as "almost threatened" in the Red List of the IUCN (IUCN, 2006) and is also listed in the Appendix II of the Convention of Migrating Species. During their stay in Uruguay, they prefer coastal shores, lagoons, pools, inundated grasslands, feeding principally on insects and spiders.

The Short-eared Owl is considered as "less common" in Uruguay and being reliant on open grassland and cultivated areas. The Passeriformes considered as "less common", such as the Hudson's Canastero, the Black-and-white Monjita, the four species of Pipits, the Double-collared Seedeater,  the Chestnut seedeater and the Lesser Red-breasted Meadowlark and the White-browed Blackbird, are all from open grasslands and need shorter or higher prairies for their biology.

Indicator species

In Uruguay knowledge of indicator species is low. Characteristic species of different environments could be considered as indicators, for example the Spotted Tinamou, the Pipit species or the Greater Rhea, but we think that indicators will probably have to be defined in every case, after the definition of the areas.

Reptiles and amphibians

Based on current knowledge, there are 48 amphibian species and 67 species of reptiles in Uruguay (Achaval, 2007; Maneyro & Carreira, 2006; Maneyro & Beheregaray, in press; Carreira & Lombardo, in press). The study area includes at least three of the five main taxonomic groups of amphibians (Maneyro et al., 1995; Maneyro & Beheregaray, in press), but reptiles have generally been less systematically studied, with records indicative of casual encounters and clearly associated with main national roads or specific collecting activities.

Continental reptiles and amphibians tend to have a low ability to move between sites, which makes them particularly sensitive to environmental changes at local scale (habitat impoverishment and fragmentation, capture of specimens, etc). This fragility is further increased by global scale factors (global warming, thinning of the ozone layer, etc.). Together, these factors make the taxa especially vulnerable. This is reflected in a particularly dramatic way in endemic species, where reduction of a single population can jeopardize the survival of the species.

The survey was based on the existing bibliography on the reptiles and amphibians of Uruguay, adapting it to the scale appropriate for the study area, drawing particularly on Carreira et al. (2005) for reptiles and Nuñez et al. (2004) for amphibians. Knowledge about distribution of amphibians and reptiles in Uruguay is based mainly on records provided by the official national collections: the Museum of Natural History and Anthropology (MUNHINA), of the Ministry of Education and Culture and the Vertebrate Zoology Collection (ZVC) of the School of Sciences, University of the Republic. This has inevitable limitations: sampling is not systematic, the ability to detect species is not constant and in some cases we do not know whether current distribution patterns have changed compared to historical distribution. Lists therefore contain both confirmed species and the possible presence of species based on habitat and expected distribution.

The plantation landscape contains three quarters of the reptiles in Uruguay and two thirds of the amphibians.

The plantation landscape contains most of the amphibians and reptiles known in Uruguay: 50 reptile species and 33 amphibian species: i.e. three quarters of reptiles and two thirds of amphibians. A thorough review of all species is beyond our scope, but general comments follow, which help outline the importance of the area and particular species of note. A full list is in appendix ##.

Amphibians of the study area

- Chthonerpeton indistinctum is the only legless amphibian species recorded in Uruguay, and all the existing data come from coastal areas, except for one specimen recorded in the department of Durazno.

- Among anurans, three species of particular interest belong to the genus Chaunus. C. dorbignyi and C. fernandezae are widely distributed but hybridization makes it difficult to map them at species level. C. achavali was recently described and has only been recorded in Uruguay and southern Rio Grande do Sul, Brazil. The type locality (Pueblo Valentines) is within the boundaries of the study area.

- The genus Melanophryniscus is represented by two species: M. atroluteus and M. sanmartini, although other species of this genus may be expected.  M. sanmartini is an endemic species of Uruguay living in hilly areas and faces conservation problems at population level.

- The family Hylidae is the most numerous in Uruguay, including 16 species. Eleven of them may be found in the study area. References to Dendropsophus minutus, Scinax aromothyella and Scinax fuscovarius were first made in Uruguay in locations near the plantation landscape. The type location of Scinax uruguays is also within the area of interest. Phyllomedusa iheringii is the only anuran species in Uruguay that builds a nest, using trees next to lotic water bodies.

- The Leptodactylidae family is represented by four of the seven species in Uruguay, all in Leptodactylus. They have complex reproductive behaviour, such as nest building and parental care, implying the presence of environments such as floodable grasslands.

- Elachistocleis bicolor, the only Uruguayan microhylid species, is widely distributed, but nonetheless its population dynamics may be affected by forestry, as its diet is based on ants (Berazategui et al in press).

- Six species of the Leiuperid family, belonging to three genera, and two species of the Cycloramphidae family occur. The Leiuperidae group includes four species of the genus Physalaemus (all species from this genus require semi-permanent aquatic habitats to build their foam nests). Two species, P. henselii and P. riograndensis, are strongly associated with hilly environments. Pleurodema bibrioni is little known and possibly endemic in Uruguay. L. macroglossa is monotypic (the only species in its genus) and found in hilly habitats hroughout whole country.

Reptiles in the study area

- Four of the five species of continental turtles recorded in Uruguay are present in the landscape. The most frequent are Trachemys dorbigni, Hydromedusa tectifera and Phrynops hilarii. These species live in lowland environments, near watercourses and lagoons. Although there are no records in the study zone, the fifth species Phrynops williamsi may also be present: it is a little known, with records in Salto, Artigas, Rivera and Cerro Largo.

- It is possible that the only crocodilian species in the country, Caiman latirostris (yacaré; broad-snouted caiman) is in the plantation landscape. There are records in the Río Tacuarembó, as well as observations made by staff of the rural establishment "Los Argentinos".

- There are ten lizard species recorded in the area. It is worth noting the presence of Liolaemus wiegmannii near Zapicán (Lavalleja) as this is outside its expected distribution pattern. Other species are Tupinambis merianae, Cercosaura schreibersii, Mabuya dorsivittata, and Teius oculatus. Cnemidophorus lacertoides and Homonota uruguayensis (the only autochthonous Gekkonidaeare), which are more restricted to hilly areas. The same is true for Stenocercus azureus, a very scarce and little known species, listed as vulnerable in Uruguay. There are also two of the three species of legless lizards recorded (Ophiodes aff. striatus and Ophiodes vertebralis).

- Although the presence of Anisolepsis undulatus has not been confirmed in the study area, it is likely to occur. The Uruguayan territory includes most of its known distribution range and it is listed as vulnerable at global level by IUCN (Hilton-Taylor 2006). Its ecology is little known and it uses low vegetation in rocky hills. Ophids represent 60 per cent of the reptiles expected or confirmed in the study area. Nineteen of these 30 species are recorded in national collections. According to Morales Fagundes & Carreira (2001), Leptotyphlops munoai, Boiruna maculata, Liophis almadensis, Liophis flavifrenatus, Phalotris lemniscatus, Philodryas olfersii olfersii and Pseudablabes agassizii are classified as having conservation problems within the country. Within this group of reptiles, there are four species considered dangerous for humans. Only Bothrops alternatus (crucera) and Bothrops pubescens (yara or yarará) have caused accidents in recent decades in Uruguay.

- Among widely distributed species, Liophis anomalus, Liophis poecilogyrus sublineatus, and Philodryas patagoniensis are the most numerous. The first two consume significant numbers of amphibians. Leptotyphlops munoai is found in hilly environments. It is the smallest ophid in the country and feeds only on ants and termites.

Important habitats

The area that includes the basaltic and crystalline-metamorphic hills is the most diverse within the study area. Most of the species that are rare, endemic and/or face conservations problems are found there. For many taxa, this environment is the southern limit of their distribution (i.e. some of the amphibian species of the so-called "fauna from Rio Grande").

Most of the rare or endemic species are found in the basaltic and crystalline-metamorphic hills in the region.

Indicator species

A number of initial suggestions for indicator species are included. Helicops infrataeniatus, a snake, seems to be sensitive to environmental conditions and might be a suitable indicator although there have as yet been no detailed studies and the suggestion is based on personal observation). Some lizard species could also be considered. Species of the genus Melanophryniscus are suggested in the amphibian group: their sensitivity to habitat fragmentation and loss has been documented. In addition the Hylidae family (e.g. Phyllomedusa iheringii), might be suitable due to its rather specialized hilly, lotic habitat.

Conservation implications and monitoring

A number of conservation requirements have been identified.

Research: clearly more research is required, because at present lack of knowledge means that it is quite difficult even to define a baseline and many species doubtless remain unrecorded or with their distribution, status and needs only very poorly understood. Permanent surveys and monitoring of key areas would both be beneficial. Such control areas have a double purpose. They allow the follow-up of the impact of any environmental changes on populations at ecosystem level, and generate basic information on the natural history of the species (diet, reproductive dynamics, etc.)

Control of agrochemicals: some taxa use ant nests to deposit and develop their eggs. Although there are bibliographical references on these aspects (Vaz-Ferreira et al., 1970; -1973), the impact of the use of biocides to control ants on reptile populations is unknown. As stated above, there are amphibian species (i.e. Elachistocleis bicolor) and reptiles (i.e. Leptotyphlops munoai) that feed on ants and termites respectively. The impact that some insecticides (i.e. carbaryl and malathion) and herbicides (i.e. glyphosate) have on aquatic communities is known (Relyea, 2005a; Relyea, 2005b). The need for responsible management of these substances in the Uruguayan forestry model has been pointed out by Geary (2001). Experimental research studies on the use of biocides and the impact they could have on the populations of amphibians and reptiles would be a useful element of any monitoring system.

Insects

As with other aspects of biodiversity, knowledge about insects of the region is limited. The experts therefore did not attempt to compile information on everything – an impossible task with such a large and diverse group – but instead focused on those species that were likely to provide the most useful data for monitoring change. This section therefore starts with a discussion of the use of insects in monitoring and then goes on to discuss make some specific proposals for the monitoring system.

Insects as indicators

Even though the commonest choice of indicator species are usually plants (Pharo et al 1999) and some groups of vertebrates (Garson et al 2002), it is now no longer unusual to use invertebrates to assess biodiversity (Hull et al 1998; Palmer 1999; Sluys, 1999). In fact, studies using insects as indicator taxons, such as the group Cicindelidae - Coleoptera (Pearson and Cassola 1992), butterflies (Ricketts et al 2002) or other groups of invertebrates are increasingly common.

Coleoptera: beetles

Taxonomically, Coleoptera account for over 40 per cent of the diversity described for Hexapoda at the global level with about 350,000 species described to date. Within that Order of insects, several features support the use of dung beetles (Scarabaeoidea) as a group to be considered when carrying out biodiversity studies in the short (rapid ecological assessment) and long term (monitoring:

- They are easy to capture and sampling protocols easily standardized

- They are a species-rich group that plays a significant role in the functioning of ecosystems by recycling nutrients and seeds. Scarabaeidae are sensitive to resources availability and changes in the vegetation (Fávila and Halffter 1997)

- They have a manageable taxonomy, worldwide distribution and the different aspects of their biology are well known

- They have a dramatic response to the modification of natural environments by human action (Halffter and Favila 1993; Hammond 1995; Favila and Halffter 1997)

Dung beetles have been used as possible indicator groups of biodiversity changes at a specific level and at landscape level, due to anthropogenic changes (Halffter & Arellano, 2002). Most of the studies, however, aim to determine what happens at a specific level when vegetation is destroyed or drastically reduced in extension (Klein 1989; Shure and Phillip 1991; Majer and Beeston 1996; Halffter, 1998). The different activities of beetles may be related to soil structure and supply of resources, in particular the amount of food and the variety of types of dung, which plays a major role in the maintenance of dung beetle populations (Astrid et al 2003; Escobar 2000; Morelli et al 2002)

Aquatic insects

The disproportionate loss of aquatic habitats has raised interest in the use of Coleoptera as indicators of a range of freshwater habitats. All freshwater bodies are good habitats for aquatic Coleoptera. From an ecological point of view beetles, and particularly their larva stage, are particularly useful indicators as they are so diverse, taking part in multiple food chains where they act as predators, detritivors, herbivores or periphyton eaters. Finally, these insects may act as allochthonous organic matter transformers and integrators (i.e. leaves, seeds, branches, fallen trees) (Araya 2000).

Aquatic Coleoptera are therefore good candidates to be used as biodiversity indicators in aquatic environments. Due to the diversity in number of species, size variation and ecological conditions, they are an ideal group for environmental impact studies, conservation proposals and biodiversity studies in a broad sense and can help to detect spatial and temporal changes (Bournaud et al 1992; Richoux 1994; Millán et al 2001a and b 2002; Pearson 1994; Halffter et al 2001).  Within benthic macroinvertebrates, Elmidae are particularly sensitive to pollution (Figueroa et al 2003).

They are a simple and low cost tool to identify priority conservation areas, as the diversity, rarity and threaten patterns of this group are apparently strongly linked to the pattersn in other organisms usually used to select protected areas, such as vertebrates or plants (Ribera et al 1993; Abellán et al 2004)

Lepidoptera: butterflies and moths

Lepidoptera are indicators of the conservation status, degree of endemism and biogeographic affinity of an area, as there is a significant correlation of larva and imago due to their herbivore habits and nectar requirements respectively, and their high sensitivity to environmental changes and variations in native vegetation. They also fulfill the characteristics of an indicator group and their use in monitoring and environmental assessments is been increasingly accepted. It is possible to identify common, migrating, key, rare and endemic species. A decrease in diversity, together with a significant reduction or increase of some species populations, indicates an environment alteration. Nymphalidae is the most studied family as indicator of environmental quality, mainly because of the important data already existing on that family.

Survey of existing information for the plantation landscape

The research team used bibliographic information to identify the list of species of Coleoptera and Lepidoptera in the area studied, which due to limitations of information was confined to Tacuarembó, Durazno and Paysandú. Data were drawn from a range of published species’ lists and from collections in three academic bodies in Montevideo.

Coleoptera

According to sources consulted, there are records in Uruguay of 63 families of Coleoptera, including 607 genres and 1,615 species. Of these, 16 families and 81 genera have been found in the area studied. The Cerambycidae family has the most species, with 34 species recorded in the area. Then follow the families Tenebrionidae (9), Carabidae (8) and Rutelidae (5), although with significantly lower diversity and the remaining families each have less than 5 species recorded.

Based on the concepts explained above and in section ##, species from the families Scarabaeidae, Aphodiidae, Trogidae and Geotrupidae are suggested as possible indicator groups for monitoring terrestrial ecosystems and species of aquatic Coleptera for monitoring watercourses.

Lepidoptera

According to the sources consulted, there are records in Uruguay of 43 families of Lepidoptera, including 737 genres and 1,241 species, of which there are 31 familias and 348 genres in the area studied.

The Noctuidae family is the one with most species, with a total of 61 species recorded in the area, followed by the families Nymphalidae (45), Hesperidae (41), Pieridae (24), and Sphingidae (20). The rest of the families have less than 20 records: Geometridae (18), Arctiidaae(18), Riodinidae (15), Crambidae (15), Lycaenidae (14), Saturniidae (12), Ctenuchiidae (11), Pyralidae (10); including 10 families with less than 10 record, and 8 with 1 record.

Butterflies have some advantages as indicators, being relatively easy to identify and also possible to identify in many cases from photographs. From the perspective of work in Uruguay, some are also indicators of former agricultural land, thus identifying site where biodiversity loss is not likely to be a problem. Some possible lepidopteran indicators are outlined in Table ## below.

| Family | Species | Observations |
| --- | --- | --- |
| Nmphalidae | Biblis hyperia | Understory forest, open areas spp. |
|  | Dryadula phaetusa | " |
|  | Phoebis philea | " |
|  | Anteos menippe | " |
|  | Adelpha sp. | " |
|  | Euptychia sp. | Primary forest |
|  | Ithomia sp. | " |
|  | Lycorea sp. | " |
|  | Anaea sp. | " |
|  | Agrius sp. | " |
|  | Prepona sp. | " |
|  | Ccaligo sp. | " |
|  | Eryphanes sp. | " |
|  | Opsiphanes invirae | " |
|  | Heliconius sp. | Fragmented secondary forest |
|  | Hamadryas sp | " |
|  | Ithomia sp. | " |
|  | Dryas julia | Indicators of cattle production area |
|  | Aphrissa statira | " |
|  | Dione juno | " |
|  | Dryadula phaetusa | " |
|  | Anartia jatrophae | Very degraded areas |
|  | Junonia evarete | Very degraded area (grassland) |
| Noctuidae | Thysania agrippina | Primary forest (Threatened for Rio Grande) |
|  | Thysania zenobia | Secondary forest |
| Papilionidae | Pterourus hellanichus | Rare species (red list) |
| Nymphalidae | Morpho catenarius | Forest of Coronilla emerus |

Recommendations

The insects of the plantation landscape are seriously under-surveyed…

According to the bibliographic survey and the study of specimens collected more than 40 years ago, the area studied has a record of only 28 per cent of the Lepidoptera species and 5 per cent of the Coleoptera species found in Uruguay, suggesting that there is currently serious under-reporting. The surveys were made in one or two locations in each of the three departments considered and in most cases during a single season. It would therefore be essential to carry out an updated survey of the Coleoptera and Lepidoptera orders in the area to use them as bioindicators. As the Coleoptera order is so diverse, it is recommended to focus on the study of soil and aquatic species. Soil Coleoptera, predators and decomposers, are represented mainly in the families Carabidae, Staphylinidae and Scarabidaeidae, among others. These functional groups are indicators of the diversity of other groups, as well as physical-chemical changes in the soil and the functioning of the terrestrial ecosystems. On the other hand, aquatic Coleoptera are important describers of spatial, temporal and physical-chemical changes produced in fluvial systems (mainly Elmidae family). They are indicators of the richness of other groups and allow the identification of priority conservation areas.

Socio-economic context

A large project will impact on the economy in more ways than just employing people; knock of effects could relate to secondary employment, overall infrastructure and concerns about social change.

The plantation project will be a major economic force in the region, which like much of rural Uruguay is currently rather under-developed, with high levels of poverty. The primary concern for many local communities is about the extent to which the project may be able to offer jobs and money, how secure employment might be and who will get the jobs. A large project will impact on the economy in more ways than just employing people; knock of effects could relate to secondary employment (contractors but also restaurants, hotels, shops and do on), overall infrastructure (maybe better roads or maybe roads that are just more heavily used) and the concerns about social change brought about by an influx of new people. To understand what may happen, it is important to know where we start. The following section of the report therefore analyses the current socio-economic situation in the region in and near where Stora Enso is buying plantations, both to help understand what will happen and to help set a baseline for future monitoring.

The main aim is to analyze and characterize the territories of Durazno, Tacuarembó, the east of Paysandú and the east of Río Negro, from a social and economic point of view. The intention is to obtain a first diagnosis regarding the main strengths and weaknesses of the area.  Two main methods have been used:

- Literature survey of existing research on forestry issues relevant to the selected provinces

- Gathering and analysis of secondary information sources such as: population and housing census, agricultural and livestock census, home surveys, reports from international organizations, maps and legislation

The following section starts by analyzing the general characteristics of the area of study in relation to the rest of the country and then looks in more detail at the social and demographic characteristics of the different localities.

Social and demographic information

Uruguay is an increasingly urbanised population; Montevideo contains 41 per cent of the population while the rest of the country (99.7 per cent of the total area) contains 59 per cent of the population and less than 45 per cent of the GDP, The four 4 provinces under consideration represent 29 per cent of the country, and only 10 per cent of the population (316.581 people), being some of the provinces with relatively low population density (except for Paysandú).

The general conditions in the country are mirrored within the micro-region under consideration, with 88 per cent of the population living in the city (280,174 people) even in an area that is considered to be very rural. Paysandú contains most of the population (36 per cent), followed by Tacuarembó (29 per cent), Durazno (19 per cent) and Río Negro (17 per cent).

If we take the micro-region, 28 per cent of the population is under 14, 61 per cent from 15 to 64 and the remaining 12 per cent is over 65, the percentage of children being considerably higher than for the country as a whole. Gender ratio is 50:50. Within Uruguay, 174,393 people live in shanty towns, including 1,252 homes in the micro-region representing 3 of the population and 1 per cent of the homes. Tacuarembó has no shanty towns, Paysandú has 3 per cent of the population and Durazno and Río Negro have 2 per cent of the population in shanty towns. With respect to agricultural holdings, 57 per cent of the producers live on the holding and 43 per cent away from it.

Human Development Index

Up to 2001, Uruguay had the second place among the Latin American countries and the Caribbean in term of Human Development Index; although in 2002 it dropped to sixth as a result of a crumbling of the GDP resulting in an economical crisis, being overtaken by Argentina, Barbados, Chile and Costa Rica. However, since the 1990s, Uruguay has become part of the group of countries with greater rates of development in the region.

In general terms, the Human Development Index in the four provinces means that they are classified has having a high human development (over 0.8), having evolved positively each year from 1991-2002.

If the rate of attendance in primary and secondary school is analyzed, according to age group, the rate is approximately 100 per cent for primary schooling in Durazno and Tacuarembó and 99 per cent in Paysandú and Río Negro. This rate is retained for the latter for secondary schooling, but drops considerably in the Other provinces: 86 per cent in Durazno, 84 per cent in Paysandú and 91 per cent in Tacuarembó.

Conditions for the rural population

Although the land is dominated by crops and pasture, employment in the sector only affects a tenth of the population.

Although the land is almost completely dominated by crops and pasture, employment in this sector only affects a tenth of the population and many of these are family members who are not paid in a formal sense. The total rural area of the plantation landscape is 4,857,560 hectares which represents 99 per cent of the whole area. A total of 9,454 businesses were registered in the census and the agricultural population was estimated at 33,053 people (10 per cent of the total population registered in the 2004 Census), 88 per cent of whom are included in the working population. Table ##, from the Agricultural and Livestock Census by the Ministry of Cattle Agriculture and Fisheries in 2000, summarises information.

Table ##: Basic agricultural and livestock characteristics

| Concept | Tacua-rembó | Durazno | Río Negro | Paysandú | Subtotal |
| --- | --- | --- | --- | --- | --- |
| Total amount of exploitations | 3.327 | 2.364 | 1.361 | 2.402 | 9.454 |
| Total rural surface (hectares) | 1.472.806 | 1.093.060 | 947.055 | 1.344.639 | 4.857.560 |
| Agricultural population | 10.796 | 6.618 | 5.971 | 9.668 | 33.053 |
| Total population of the Province (2004 census) | 90.489 | 58.859 | 53.989 | 113.244 | 316.581 |
| Working population | 9.314 | 6.499 | 5.326 | 7.855 | 28.994 |
| Hectares per exploitation | 443 | 462 | 696 | 560 | 540 |
| Residents per exploitation | 3,24 | 2,80 | 4,39 | 4,02 | 3,61 |
| Hectares per person | 136 | 165 | 159 | 139 | 150 |
| Workers per exploitation | 2,80 | 2,75 | 3,91 | 3,27 | 3,18 |
| Hectares per worker | 158 | 168 | 178 | 171 | 169 |

Holdings tend to be reasonably large. The average number of hectares per holding of the region is 540 hectares, even higher in Río Negro and Paysandú , and in all cases higher than the average for the rest of the country.

Labour

With respect to the permanent labour force, it should be noted that 55 per cent of the workers are not paid in a formal sense (55 per cent of these are the producer or a partner themselves, 41 per cent a producer’s relative and 4 per cent others) and the remaining 45 per cent are paid workers (of which we can distinguish 5 per cent professionals/technicians, 16 per cent administrators/foremen and 9 per cent machine or tractor operators with most of the rest classified as manual workers or ranch hands). This breakdown appears to be similar throughout the landscape.

Land tenure

Regarding the land tenure regime, in the micro-region 60 per cent are owners representing 54 per cent of the land surface, 15 per cent tenants, 9 per cent owners / tenants and 10 per cent of the holdings operate under some other tenure system. Tables ## and ## below summarise the land tenure system in the region.

Table ##: Number of holdings according to land tenure regime

| Tenure | Tacuarembó | Durazno | Río Negro | Paysandú |
| --- | --- | --- | --- | --- |
| TOTAL | 3.327 | 2.364 | 1.361 | 1.911 |
| Owners | 2.170 | 1.355 | 731 | 1.097 |
| Tenants | 364 | 360 | 300 | 289 |
| Sharecroppers | 14 | 4 | 23 | 17 |
| Occupants | 212 | 132 | 26 | 57 |
| Owners /Tenants | 236 | 264 | 103 | 193 |
| Owners /Sharecroppers | 3 | 1 | 16 | 30 |
| Tenants /Sharecroppers |  |  | 21 | 28 |
| Other means | 328 | 248 | 141 | 200 |

Table ##: Surface area of holdings according to land tenure regime

| Tenure | Tacuarembó | Durazno | Río Negro | Paysandú |
| --- | --- | --- | --- | --- |
| TOTAL | 1.472.806 | 1.093.060 | 947.055 | 1.334.352 |
| Owners | 878.066 | 566.486 | 475.439 | 705.157 |
| Tenants | 149.718 | 127.109 | 82.513 | 154.325 |
| Sharecroppers | 2.377 | 392 | 14.883 | 5.964 |
| Occupants | 16.480 | 10.753 | 2.860 | 4.035 |
| Owners /Tenants | 200.856 | 243.177 | 142.306 | 228.092 |
| Owners /Sharecroppers | 86 | 62 | 8.859 | 51.140 |
| Tenants /Sharecroppers |  |  | 17.095 | 14.489 |
| Other means | 225.223 | 145.081 | 203.100 | 171.150 |

90 per cent of the land surface in the plantation landscape is owned by Uruguayans and used for agriculture.

The large majority of the operations (82 per cent) are single person businesses, with 8 per cent joint venture and 9 per cent legal partnerships. Regarding the nationality of producers, 96 per cent of agricultural businesses belong to citizens of Uruguay, representing 90 per cent of the surface and 4 per cent are foreigners including mainly Argentineans and Brazilians along with a few other nationalities. Around 4 per cent of the surface is currently in the hands of people of these “other nationalities”.

Education levels within the agricultural community

Level of education varies widely amongst those owning or running agricultural enterprises in the region. Only 3 per cent have had no education but 17 per cent more did not complete primary schooling; at the other end of the scale 16 per cent of people have had a university education (12 per cent completing). It is worth noting that 40 per cent of land is managed by people with a university education, suggesting that larger land owners tend to benefit from a better education. In Table ## the different education levels from the different regions are summarised.

Table ##: Education levels within the agricultural community

| Educational level | Tacuarembó | Durazno | Río Negro | Paysandú |
| --- | --- | --- | --- | --- |
| TOTAL | 100,0 | 100,0 | 100,0 | 100,0 |
| None | 4,5 | 2,5 | 1,2 | 2,6 |
| Primary completed | 32,4 | 34,4 | 24,0 | 29,2 |
| Primary non completed | 24,1 | 13,1 | 11,1 | 15,7 |
| Secondary completed | 10,0 | 16,7 | 17,7 | 14,5 |
| Secondary non completed | 8,4 | 9,3 | 11,2 | 11,2 |
| Technical completed | 4,5 | 5,1 | 9,7 | 7,2 |
| Technical non completed | 1,3 | 1,4 | 1,9 | 1,8 |
| University completed | 9,1 | 11,5 | 16,9 | 12,0 |
| University non completed | 3,3 | 3,6 | 4,4 | 3,6 |
| Other | 1,0 | 1,1 | 0,7 | 0,6 |

Jobs and income level

With respect to the latest report by the Encuesta Nacional de Hogares Ampliada [Extended National Domestic Survey], the Activity Rate for all the country in October 2006 is 60.6 per cent, although information is not available at provincial level. In rural areas this rate has remained stable overall, but has increased slightly in places with 5,000 inhabitants or more and declined in smaller localities and rural areas according to INE [National Institute of Statistics]. The trend for jobs and people to gravitate towards urban areas appears to be continuing. The lack of provincial data is addressed to some extent by a survey carried out in the regions: however the survey was of a fairly small urban based sample and must be treated with caution. From the people surveyed in the four provinces, it appears that employment rate exceeds 50 per cent in every case if those under 14 are eliminated from the calculations. The unemployment rate is below the national average. However, in all four provinces wage levels are also slightly below the annual average of $5.383.

The role of agriculture in economic activity

The contribution of agriculture and livestock to the economy has declined in the region in ecent years.

The provinces under consideration remained relatively stable during the period of economic downturn as compared to some other parts of the country, but in general the agriculture and livestock contribution to the economy has decreased over the last few years although it now shows some signs of recovery. Considering the country as a whole, in 2003, the primary sector represented 13.58 per cent, the secondary sector 28.32 per cent and the tertiary sector 58.1 per cent of the economic activity. The role of the primary sector increases slightly in rural provinces, but remains less than the tertiary sector even here, the latter reaching almost 50 per cent in all of the four provinces.

Land use

As regards land exploitation measured through the exploited surface of the soils, we gather that in the micro-region, 74% of the land is used as pasture, with other significant uses being 6 per cent of the land in plantations and 6 per cent in planted grassland. Table ## below outlines more precisely the breakdown.

Table ##: Breakdown of land use in hectares

| Land use | Tacuarembó | Durazno | Río Negro | Paysandú |
| --- | --- | --- | --- | --- |
| TOTAL | 1.472.806 | 1.093.060 | 947.055 | 1.344.639 |
| Natural woods | 74.650 | 12.609 | 33.807 | 54.073 |
| Artificial woods | 94.553 | 38.449 | 70.523 | 91.448 |
| Citrus fruits | 94 |  | 321 | 8.960 |
| Other fruit trees | 7 | 3 | 88 | 296 |
| Vineyards | 54 | 99 | 2 | 181 |
| Orchard crops | 1.882 | 76 | 498 | 453 |
| Industrial and cereal crops | 17.978 | 5.352 | 79.868 | 56.069 |
| Annual fodder crops | 13.474 | 15.603 | 42.192 | 35.406 |
| Ploughed soil up to 6/30/00. | 1.460 | 3.292 | 12.522 | 16.024 |
| Stubble soil | 3.267 | 2.665 | 14.566 | 13.149 |
| Artificial prairies | 41.118 | 46.174 | 110.367 | 79.085 |
| Natural fields seeded under shelter | 17.082 | 74.274 | 11.219 | 9.842 |
| Fertilized natural fields | 6.376 | 5.066 | 11.075 | 10.191 |
| Natural fields | 1.186.493 | 884.061 | 552.242 | 957.971 |
| Non productive land | 14.318 | 5.337 | 7.765 | 11.491 |

Table ##: Breakdown of land use as a percentage

| Land use | Tacuarembó | Durazno | Río Negro | Paysandú |
| --- | --- | --- | --- | --- |
| TOTAL | 100,0 | 100,0 | 100,0 | 100,0 |
| Natural woods | 5,1 | 1,2 | 3,6 | 4,0 |
| Artificial woods | 6,4 | 3,5 | 7,4 | 6,8 |
| Citrus fruits | 0,0 |  | 0,0 | 0,7 |
| Other fruit trees | 0,0 | 0,0 | 0,0 | 0,0 |
| Vineyards | 0,0 | 0,0 | 0,0 | 0,0 |
| Orchard crops | 0,1 | 0,0 | 0,1 | 0,0 |
| Industrial and cereal crops | 1,2 | 0,5 | 8,4 | 4,2 |
| Annual fodder crops | 0,9 | 1,4 | 4,5 | 2,6 |
| Ploughed soil up to 6/30/00. | 0,1 | 0,3 | 1,3 | 1,2 |
| Stubble soil | 0,2 | 0,2 | 1,5 | 1,0 |
| Artificial prairies | 2,8 | 4,2 | 11,7 | 5,9 |
| Natural fields seeded under shelter | 1,2 | 6,8 | 1,2 | 0,7 |
| Fertilized natural fields | 0,4 | 0,5 | 1,2 | 0,8 |
| Natural fields | 80,6 | 80,9 | 58,3 | 71,2 |
| Non productive land | 1,0 | 0,5 | 0,8 | 0,9 |

As Figure ## shows below, the dominant economic activity remains cattle farming

Existing plantations

Taking the micro-region as a whole, 6 per cent is already devoted to plantations, of Durazno is below this average percentage with only 4 per cent of plantations, and Río Negro and Paysandú already have 7 per cent of the area afforested with plantations. The large majority of plantation area (82 per cent) is established within the framework of the Law for Promoting Forestry, as summarised in Table ## below. More than 98 per cent of the plantations are in plots covering over 100 ha, and 83 per cent over 500 ha.

Table ##: Relationship of plantations with the Law Promoting Forestry

| Size of exploitation | Total micro-region |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total |  | Within the Law for Promoting Forestry |  | Outside the Law for Promoting Forestry |  |  |
|  | Ha | (%) | Ha | (%) |  | Ha | (%) |
| TOTAL | 294.906 | 100,0 | 242.927 | 82% |  | 51.979 | 18% |
| Less than 3 | 2.393 | 1% | 1 | 0% |  | 2.392 | 5% |
| From 3-0 | 5.732 | 2% | 37 | 0% |  | 5.695 | 11% |
| From 11-20 | 3.953 | 1% | 210 | 0% |  | 3.743 | 7% |
| From 21-50 | 7.064 | 2% | 902 | 0% |  | 6.162 | 12% |
| From 51-100 | 6.700 | 2% | 2.640 | 1% |  | 4.060 | 8% |
| From 101-500 | 48.816 | 17% | 38.289 | 16% |  | 10.527 | 20% |
| Over 500 | 220.248 | 75% | 200.848 | 83% |  | 19.400 | 37% |

There was a peak of planting in the mid to late 1990s, followed by a rapid falling off due to changes in the general economic situation and the specific support for forestry, as Figure ## below outlines:

To check:

Is this correct relating to forest support?

Figure ##:

Roads and other infrastructure

Around 29 per cent of agricultural and livestock holdings have acess by using a permanent road or route, 46 per cent use improved roads and 26 per cent by earth road. Tacuarembó and Durazno have comparatively large amounts of earth road access (35 per cent and 31 per cent respectively). Around 43 per cent of holdings have a telephone, 61 per cent electricity, 14 per cent an administrator, 30 per cent technical assistance and 40 per cent maintain management records.

In summary, although agriculture and livestock tend to generate the highest incomes, the rural parts of the four provinces also rely on forestry, fruit growing, horticulture and dairy production. In Durazno and Tacuarembó, agricultural and livestock activity is still the most important primary industry within the provincial gross added value, with cattle highest, followed by agriculture. Gradually, other activities such as poultry, beekeeping, viticulture and more recently forestry are starting to have a greater effect. In Río Negro is the major contributor to the gross value of the agricultur  al and livestock production in the province, representing 23 per cent of the fertilized area in Uruguay, and the situation is similar in Paysandú.

…although agriculture and livestock tend to generate the highest incomes, the four provinces also rely on forestry, fruit growing, horticulture and dairy production.

Social and demographic characteristics of the four provinces

In addition to the general survey, more details were compiled for the four provinces under consideration, both to supply background to help planning the project and to provide a baseline against which monitoring and assessment can take place. The following areas were considered:

- Durazno: every locality

- Tacuarembó: every locality

- East of Paysandú: Piedra Sola, Arbolito, Tiatucura, Morató, Merinos, Piñera, Guichón

- East of Río Negro: Sarandí de Navarro, Rolón, Grecco

Durazno

From the 15 localities registered by the Census (excepting the capital city), only one has a population greater than 2.000 inhabitants (Sarandí del Yí and el Carmen; six more have a population of 1,000-2.000 (La Paloma, Santa Bernardita, Blanquillo, Cerro Chato, Carlos Reyles and Centenario). For the rest, San Jorge has a population slightly greater than 500, and the remainder are below 250 (Baygorria, Aguas Buenas and Ombúes de Oribe), or below 100 (Feliciano, Rossell y Rius and Pueblo Álvarez).

Regarding population distribution by age, the highest proportion of elderly people are found in Rossell y Rius and Pueblo Álvarez with 25 per cent of inhabitants over 65 years old. Proportions of elderly people of 10 per cent or more are found in Sarandí del Yí, Carmen, La Paloma, Santa Bernardina, Blanquillo, Cerro Chato, Carlos Reyles and Centenario. In every case except for Rossell y Rius children make up more than 20 per cent of the population. Both these statistics have implications for availability of labour. In nine places in the province less than 60 per cent of the population is from 15 to 64 year olds, although it is higher in Durazno, Santa Bernardina, Blanquillo, Centenario, San Jorge, Rossell y Rius and Baygorria, where it reaches 76 per cent. Around 98 per cent of the population lives in private houses and 2 per cent in collective houses; some 17 per cent of houses are currently empty although this average rises considerably higher in economically depressed areas such as Baygorria and Feliciano.

Tacuarembó

Tacuarembó has more towns and villages, reaching  25  in addition to its capital city. Paso de los Toros is a city of over 10,000 people and two towns with over 2,000 inhabitants (San Gregorio del Polanco and Ansina). There are ten places with 100-1000 people and eleven with fewer than a hundred inhabitants: in part this is a result of the large area covered by the province. Perhaps because of this, it has 16 per cent of rural population which is slightly higher than in Durazno.

With respect to age distribution, the most densely populated settlements tend to have more than 10 per cent of elderly people (65 or over), for example in Tacuarembó, Paso de los Toros, San Gregorio del Polanco, Ansina, Achar and Curtina. Proportion of children under 14 varies from 25-38 per cent depending on locality with a provincial average of 28 per cent. More than half the towns and villages have less than 60 per cent of the population between 15 and 64 years old and thus eligible to work (San Gregorio del Polanco, Ansina, Curtina, Achar, Tambores, Paso Bonilla, Clara, Pueblo del Barro, Piedra Sola, Sauce de Batoví, Chamberlain, Cardozo, Cerro de Pastoreo and La Hilera). The only localities that show percentages higher than 70 employable are Balneario de Iporá, Rincón del Bonete and Laureles. Similar to Durazno, 85 per cent of houses are occupied although there are variations between settlements.

East of Paysandú

In Paysandú, out of the seven localities included in the study, only Guichón has a population of over 5,000 people, the rest have populations below 1,000 and in Villa María there are less than a hundred people.

Regarding age distribution, there are four localities with an elderly population above 10 per cent of the total: as Guichón, Piñeira, Piedra Sola and Villa María. All localities have more than 50 per cent of the population between the ages of 15 and 64 years and therefore eligible to work.

Although 85 per cent of houses are inhabited this disguises major differences within the part of the province within the study. There are only two localities (Paysandú and Guichón) where the rate is above 80 per cent and the remaining five all have more than 25 per cent uninhabited houses.

East of Río Negro

In Río Negro, the two localities covered by the census, are in the east of the province: Grecco and Sarandí de Navarro, with populations of 726 and 269 people respectively. In Grecco 30 per cent of the population are children and 54 per cent adults between the ages of 15 and 64. In Sarandí de Navarro, the relation is similar, 34 per cent children and 50 per cent are adults.

Regarding the number of houses that are inhabited the latest figures are 83 per cent in Grecco and 68 per cent en Sarandí de Navarro.

Table ##: Summary of key socio-demographic indicators for the four provinces

| Variable | Tacuarembó |  | Durazno |  | Paysandú |  | Río Negro |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total population | 90.489 |  | 58.859 |  | 113.244 |  | 53.989 |
| Proportion living in urban areas | 84% |  | 88% |  | 92% |  | 87% |
| Ration of men to women | 50%-50% |  | 50%-50% |  | 50%-50% |  | 51%-49% |
| Working population (15-64 years) | 61% |  | 60% |  | 60% |  | 61% |
| Viviendas / hogares | 32.298 / 28.054 |  | 20.778 / 17.779 |  | 38.133 / 33.691 |  | 18.431 / 15.786 |
| Tasa de Desempleo (ENHA – Oct. 06) Inf. para el total del país. | Total for the country: 9,7 % Montevideo. 9,7% - Interior of the country: 9,7% Hombres: 7,3% - Mujeres 12,6% |  |  |  |  |  |  |
| Human Development Index | 0,828 (lugar 7) | 0,837 (lugar 8) |  | 0,831 (lugar 11) |  | 0,837 (lugar 12) |  |

Local stakeholder opinions regarding the proposed plantation

Plantations are controversial. A large series of plantation projects bring major changes to rural communities, which may already be experiencing considerable social upheaval. The promise (from some peoples’ persective the threat) of a pulp mill adds to the questions and fears that local communities have about the impacts of the project. The fact that in this case it is foreign-owned raises additional concerns. Many people are happy to speak on behalf of local communities but in this study we made an attempt to give people a chance to speak for themselves by using a team of anthropologists to carry out a series of structured interviews in ten communities within the plantation landscape. Below is a summary of thoughts and opinions from the region, followed by a more detailed breakdown of some of the commuities involved.

The following communities were involved:

- La Paloma – Durazno

- Blanquillo – Durazno

- Villa Carmen – Durazno

- Pueblo Centenario – Durazno

- Paso de los Toros – Tacuarembo

- Clara – Tacuarembo

- Zamora – Tacuarembo

- San Gregorio – Tacuarembo

- Grecco – Rio Negro

- Guichón – Paysandú

In all, 83 interviews were carried out – a full list of the people involved and their positions is given in appendix ##.

Forestry and cultural change: Building a new work identity

Large-scale forestry in a traditionally agricultural country implies a substantial cultural change in rural areas. It is not only a new production system but also requires new behaviours and new collective imagery. Although the production model is similar to the traditional ranching model as regards workers depending on employers, certain changes are foreseen.

The relationship between the farm labourer and the owner is substituted by that of a forestry worker and a “faceless” company. Workers, who in the past dealt with the foreman, the skilful right-hand man of the owner, now deal with contractors, who are often outsiders – in many cases from Rivera (one of the 19 administrative districts of Uruguay, on the border with Brazil). For workers, forestry work is equated with contractors. Slthough they know they are separate companies, to date forest companies in general, and especially Stora Enso, have been absent.

Changes in employment status are also significant. Forestry is characterized by seasonal patterns whereas farms are a source of stable work. Moreover, the eight-hour shift in forestry contrasts sharply with the round-the-clock availability of farm workers. Inclusion of seasonal and permanent workers in the social security system is an improvement brought about by forestry and highly valued by local communities. Testimonies also mention other benefits, including transport to and from the work place, food, and safety gear for crews.

Traditional rural workers enjoy a certain degree of independence, knowledge and expertise are handed down from generation to generation or learnt by observation, and performance is evaluated on the basis of how the task is performed rather than the amount of work done. Forestry crews, however, are assessed on the basis of quality and quantity of tasks accomplished forcing rural workers to readapt to this new set of criteria.

Inexperience and the fear of losing opportunities push rural workers to attend training courses; yet another change since now they require formal knowledge to perform tasks.

Workers, the community at large, and educators in particular, appreciate the fact that forestry workers can return to their homes at the end of their shifts instead of staying overnight on the farm or plantation as per the usual practice.

In short, the changes mentioned confer dignity on rural work, make workers feel cared for and that their basic needs are being fulfilled.

From the anthropological point of view, the substitution of agriculture by forestry can be interpreted as a loss of the ranching work identity and/or as processes of cultural change and emerging concepts of the “rural being”.

Economically depressed and with previous experience in failed development projects, these communities feel the pressing need for new projects and sources of employment. In the absence of local and national proposals, forestry would be quickly accepted without concerns regarding lifestyle changes in relation to rural work or deep analysis of potential benefits versus liabilities connected to forestry.

Gender issues

Female employment in forestry, whether in nurseries or plantations, is one of the main changes brought about by forestry in the area.

For the women interviewed there is a “before” and “after” feeling. Female employment opportunities in the towns studied were almost nonexistent, with the exception of traditional low-paying female work such as cooking and house cleaning. In the communities where women have already started working in nurseries or in plantations, there is a feeling of pride on breaking into this traditionally masculine line of work. Furthermore, in some tasks, such as herbicide application and operating the harvester, women have equalled or outperformed men.

Female harvester operator, southern Uruguay

Nigel Dudley

Changes in the family structure, in power relations between spouses and in gender relations outside the family are a consequence of female employment. Yet another emerging factor in local social structures worth noting is the potential economic independence of women.

Lastly, women have made huge inroads into the rural labour market, which puts them one step higher in a social sense, giving them a new status within the community and producing changes in social and family situations

History – decline of the railway system and emergence of forestry

Accounts and documents reveal the construction of the railway system by British companies resulted in a chain of settlements and towns springing up throughout the country. The genesis of most towns was closely connected to the arrival of the railroad lines that brought jobs and communication with the rest of the country. The deterioration of the railway system that began in the 1970s resulted in social and economic decay of railway stations and surrounding towns (in Paso de los Toros, for instance, the number of workers dropped from 600 to 100 at present). Town dwellers reminisce about those by-gone times and feel “forsaken” and “forgotten”. The road network is no substitute for the railway system because it fails to provide protection and organization to locals who miss the orderly speeding of trains along the lines.

Ever since, a number of replacement projects of varied scope have been proposed, but in the end all turned out to be unsustainable. In the 1960s for example, Metzen y Sena started exploiting the kaolinite quarries near Blanquillo. Throughout that decade, the quarries were the main source of employment, but soon mechanization replaced workers. Other projects in tourism, crafts, etc., have generated expectations but failed to deliver. Although one of the aims of the study was to identify new projects, none was found.

Stora Enso, a well-established, internationally renowned forest company based in Europe, is often considered the sole alternative for community development in view of local history. Almost all sectors of society believe Stora Enso has the “power” to give life to towns and villages that will derive benefits from the company’s project. There is a “before and after” of Stora Enso and other forest companies (for instance in Blanquillo). In other towns, where forest companies are yet to arrive, expectations are high (for instance in La Paloma). The arrival of crews or workers seeking jobs in the forest industry has resulted in a population increase, reminiscent of the golden days when towns and villages were founded by railway employees.

Almost everyone believes Stora Enso has the “power” to give life to rowns and villages…however locals know booms are not long-lasting.

The region flourishes with the hustle and bustle of the new founding period. Large projects offering plenty of jobs attract workers that flock to the area. Locals are starting to believe once again that “it’s all for the better”, in the words of an interviewee in Clara.

However, locals know booms are not long lasting. They ask themselves incessantly how long the forestry bonanza is going to last, a concern voiced by several interviewees, including those that express the staunchest support for forestry. Community action and communication strategies should include expectation management aspects.

New conflicts of conscience: more jobs verses environmental uncertainties

Debates about environmental issues connected to forestry are, to a certain extent, a novelty in farming. The concept of sustainability dating back to the 1980s was part of the discourse of agricultural engineers and government officials. Now for the first time in the country’s history, an environmental debate rages over a rural issue. In the mid 1980s, rice and the associated water requirements that posed a threat to wetlands in eastern Uruguay were debated. Although progressive shrinking of fresh water bodies could be considered a global issue, the debate was within Uruguay’s borders and had to be solved in Uruguay. Environmental issues and threats connected to forestry reach Uruguay through international and foreign mass media and the cause is quickly embraced by national and international environmental groups.

How do towns and villages experience the phenomenon? According to interviews, in general forestry is perceived as a job generator. Even for the few interviewees who mentioned that seasonal employment is a critical issue, lack of other options makes forestry “better than nothing”. However, varied levels of uncertainty permeate the welcome of forestry in general and of Stora Enso in particular. Interviewees in general, with the exception of those in specific production sectors, mention social benefits and ask questions about environmental issues without taking radical stances. Testimonies within this same group that do not fall in this category were exceptional, such as the teacher in La Paloma.

Water as a resource is one of the main concerns voiced by interviewees. Information sources from abroad denounce desiccation of certain ecosystems due to Eucalyptus plantations and local knowledge has been acquired by observation and personal experience of farmers or rural workers.

Forest fires are another source of concern, especially in towns or villages that are surrounded by trees (Grecco for example), whereas agrochemicals and erosion do not appear to be a widespread concern.

Landscape changes due to plantations are not considered a problem in general, though interviewees mentioned local issues and voiced strong personal opinions. Some interviewees in Blanquillo and Clara for example, consider that the surrounding plantations make the place “look like a European country”, as if it were “the first world”. A livestock farmer in Zamora, however, says “I feel as if I were fenced in”.

Plantations bring dramatic visual changes to a flat, open landscape like the one in Uruguay.

Certain animal species considered as pests associated to woodlands are a considerable source of concern that will be dealt with further on.

Socio economic benefits outweigh possible environmental damages and environmental groups opposed to forestry seem to have little influence in shaping public opinion. Nonetheless, in Uruguay forestry and connected environmental issues have been seriously questioned and information is often contradictory. Testimonies give evidence of an unsolved conflict owing to a lack of clear answers to questions about forestry as a new production system.

Forestry from the point of view of other sectors

Persepectives were collected from a range of people who will not be working directly in the plantation or relying on it for their income.

Fishing communities

Fishing is a way of life and part of the culture in Paso de los Toros, San Gregorio de Polanco and Centenario. Individuals as well as formal spokesmen of cooperatives, such as Copehum in San Gregorio, believe forestry and fishing are not incompatible. Forestry is not seen as a threat in itself to fish, a natural resource being exploited for economic purposes. As fishermen, they voice their concerns regarding agrochemicals that will eventually leach into the Río Negro basin, contaminating watercourses and therefore destroying the habitats of fish species.

Some interviewees mentioned they would rather combine fishing and forestry. Fishermen in San Gregorio, who are grouped in a cooperative, mentioned they would be interested in neutralizing the seasonality of both jobs, planning forestry peak seasons and fishing peak seasons so that they do not overlap, allowing them to move from one job to the other. Fishermen in Paso de los Toros and Centenario are yet to organize themselves, but they perceive forestry along the same lines as in San Gregorio. Seasonality characterises both fishing and forestry. Fishermen envisage a way to combat and minimize seasonality using watercourses for log transport, profiting from their in-depth knowledge of the river. According to their own testimonies, they would be following the steps of countries with logging tradition and enough available watercourses suitable for log transport. In summary, fishermen seek to become partners of forestry instead of confronting it.

Tourism

In towns with a strong inclination towards tourism, such as San Gregorio, the population at large, the tourism sector and connected sectors (commerce, handicrafts, real estate agents, etc.), were concerned about the proposed location of the mill. Locals oppose the building of the mill in the vicinity or upstream of the Rincón del Bonete dam, on the grounds that it is a tourist destination heavily reliant on natural beauty. In Paso de los Toros, however, these concerns were not voiced. Employment is a pressing concern and tourism is not sufficiently developed so forestry is viewed positively as a potential source of jobs.

In general, forestry is not viewed as threat to tourism. Although far from unanimously, the changes in landscape are met with approval. Areas that were once vast, empty plains are now covered with trees.

Farming communities

The deepest questioning of forestry comes from livestock farmers and ranchers who face pressing conflicts: demand for land and labour. This sector has felt the previously mentioned cultural changes more sharply than any other one and resents the state benefits granted to forestry. Nevertheless, interviewees cannot ignore the upsurge in forestry projects and are beginning to think of forestry as a partner. This partnership involves in-depth study of the possibilities offered by agroforestry, breeding mares for the pharmaceutical industry and planting some of the low profitability land within their ranches with trees, on their own or through contractors. Ranchers interviewed showed a cautious acceptance of plantations and moderate expectations as to the compatibility of plantations and traditional livestock farming.

Opinions differ within the sector. According to qualified interviewees, older traditional ranchers are the most doubtful as to the benefits of forestry, whereas younger generations seem more or less open to making the two sectors compatible.

Wine and vineyard sector

This sector has played an important role in towns with a strong wine making tradition, such as Villa Carmen, where inhabitants feel a historical link to wine making. This link is felt most strongly among older generations who remember the golden years of vineyards. At present, forestry is not seen as an obstacle to new projects in this sector. Interviewees, including those directly involved in present wine and vineyards projects, see forestry as well established in the surrounding area.  Forestry has had a strong impact on the town’s everyday life, creating jobs and thereby promoting the financial well-being of its inhabitants and distributing income in the area. Besides, local vineyards produce wine on a small-scale for commercialization in local shops.

Beekeepers

Eucalyptus species being planted are controversial. According to some beekeepers, these are not good sources of nectar while others state that plantation management does not take into account nectar gathering cycles, preventing beekeepers from reaping profits. From the social point of view, forestry is considered positive. In some specific cases, however, such as in San Gregorio, interviewees believe forestry is not compatible with traditional agriculture. In Villa del Carmen, on the other hand, beekeepers believe honey opportunities have sprung up with the arrival of Eucalyptus. Consequently they consider forestry brings not only social benefits, but also benefits for beekeepers.

Conclusions

The study was conducted in ten towns comprising eighty eight testimonies and provides a general diagnosis of the current situation of towns and villages in relation to forestry. Designers of company social policies to be implemented will find in this document ample reference to the socio cultural reality of the area under study.

Future research issues arise from the field work carried out. Among these it is worth mentioning in-depth analysis of ranching, fishing and forestry labour identities in rural areas and their complementarity; changes in gender, family and poverty issues caused by forestry; historical building of towns after Stora Enso’s arrival; the local cultural construction of “environmental issues” and how existing productive systems impact on them, among others.

National stakeholder opinions regarding the proposed plantation

A series of interviews were carried out during March 2007, in a process that aimed at identifying and then talking with major stakeholders interested in forestry and plantation development to gauge opinions, hopes and fears in this sector. In all, 31 national stakeholder groups were identified and selected individuals approached for interview – a full list is available in appendix ##.

To check:

The translation (or perhaps the original text) was extremely unclear – I had to guess in places, so  this needs checking with the original and possibily going back to the authors

The Interview guidelines were drawn up bearing in mind major research issues and also the main arguments discussed in the media, with the aim of identifying current issues and recognising the different positions of the national stakeholders (see summary of main questions in appendix ##). In all, 16 interviews with national authorities, representatives of producers associations, academics and provincial stakeholders of Durazno and Tacuarembó were carried out from March 7th to 20th.

Opinions regarding national forest policy

There is a broad consensus among all the stakeholders interviewed that Law 15.939 – promulgated in 1987 to encourage forest development – had a positive impact on the country. The acceptance of current State policy on forestry by all the major political parties marks a rare political consensus in Uruguay. Historically, the country has focused on livestock, although there have also been considerable efforts to promote agriculture. Development of plantation forestry was only possible due to the incentives offered to the sector. Nowadays, several different production models can be identified in terms of final products from plantations: the south of the country tends to produce pulp; the littoral and centre produce a mixture of pulp and sawnwood, while the north has focused mainly on sawnwood.

Following these basic agreements, opinions start to differ about the importance of forestry for the country although there is agreement that the change of government in 2005 also marked a distinct change of direction in forest policy.

- National authorities generally regard government policy as having fostered plantations and helped to create a critical momentum towards plantation within the sector, but with weak integration with other sectors

- From the academic field, producers associations and provincial stakeholders, it is acknowledged that the policy was successfully implemented to generate enough raw materials to help start an industry. However, some stakeholders emphasized that the model promoted forestry based on direct foreign investments, which is regarded as negative

In general, the main concerns about the impact of the model are:

Key concerns relate to land tenure, risks of displacement, presence of outside workers, poor dialogue with local communities and farming community and lack of information

- Impacts on land tenure

- Displacement of the rural population

- Hiring of outside workers

- Poor dialogue between plantation companies and local residents

- Lack of interest in developing outgrower schemes and other collaborations with small producers

- Scarcity of information and public research on environmental impacts of plantation forestry

- Lack of contact between livestock farmers and the forestry sector.

Interviewees tried to identify some common forestry policy guidelines from the new government, but although there was consensus in identifying concrete measures, there was lack of agreement about what direction it should be taking. Some important legal and policy steps are outlined in the box below.

Do we need this?

Box: Recent legislaton and policy initiatives affecting the sector

- Repeal of decree 333/90 (accessory lands)

- Repeal of decree 330/90 on subsidies

- Re-classification of forestry soil which included quality species

- Tax amendment given to pulp plantations

- Decree on previous environmental permit by DINAMA on surfaces above 10.000 ha

- Guidelines for forestry transportation

- Repeal of indirect tax refund to log exports to be sawed (under development)

- The third parties and Corporations Law, while covering more than plantations, is recognized as having been inspired by this sector

Although these policy initiatives have certainly helped to set the direction for forest policy there is little consensus about where it will go from here, either inside or outside the government. It is recognized that subsidies are no alonger needed to promote the sector, although also that their removal may damage small producers, who are once more excluded from this sector. The national authorities believe that the current dominant forestry model will continue to play a leading role, although they want to introduce measures the better to promote integration of national producers into the sector and to create better working conditions. They stress the need to develop a strategy to add more value to the pulp industry and to improve integration of the forestry and livestock sectors.

The national authorities admit there is some degree of internal disorganization with respect to national forestry policy and there is clearly sometimes a lack of internal coordination and of joint vision. Workers in the academic and research fields are trying to keep up with changes in policy, with major developments relating to species diversification, silviculture projects and implementation of the clean development mechanism. Producers associations see lack of coordination and gaps in the current model of forestry development which cause uncertainty due to the fast rate of policy change and lack of stakeholder consensus.

In general, there is agreement on the need to move towards a more integrated approach in which the plantation industry would be one more part of a broader approach to development, with higher-value products and better integration between producers, including the small producers. However, there is disagreement about the possibility of ths: some stajeholders believe it is possible or even in progress while others think that the market-based model will continue to be dominated by a few large players.

Specific aspects of forest policy

Changes in the tax law

Most interviewees thought that eventual removal of subsidies to the sector had been planned even when the original law had been set, abd the government’s actions had simply accelerated a process that was going to take place anyway towards 2007. There was some feeling that there was still insufficient clarity about the period of time in which subsidies could be kept and a general recognition that default on subsidy payments had created problems. No-one thought thar removing the subsidies would affect the development of the sector, because the sector is already running and new companies do not regard it as a requirement. Some believe that the subsidy was never really needed to promote the sector or attract investments. Acaddemics argued for special subsidies to help small producers although this was not mentioned by producers associations.

Tax exemptions on forestry soils

Currently forestry operations do not have to pay tax to local authorities. There are two clear opinions about this: national stakeholders regard it as unimportant because they think that forestry will bring economic dynamism to the area, while local authorities disagree and believe that the national government is reducing regional autonomy. Even though the latter acknowledge the positive socio-economic impact the sector, they have concern that loss of tax revenues will make it more difficult to maintain and repair provincial roads. Indeed the issue of the impact that forest development will have on roads has been discussed since the 1990s without clear policies emerging. Infrastructure has historically been one of the main election issues at a local level but only has a minimal impact on voting at a national level and there is concern that as a result the national government does not take it seriously. National authorities believe that a concrete tax can be negotiated to support road infrastructure but there remain disagreements on the ways in which this might be collected or implemented. There ic currently no discussion about reviving the rail network.

In general, there continues to be only a weak relationship between government and companies: the latter fear that the government has changeable policies while the former regard their relationship with companies as being based on lobbying with a relatively narrow vision and little opportunity to build the sector together.

State capacity to monitor the forestry sector

Monitoring of the forestry sector is identified as a serious weakness

All the interviewees were concerned that the state did not have the capacity to monitor the economy in general and particularly the forestry sector. In general, it is considered to require important changes in approach by the government, where a clear strategy with respect to monitoring and assessment needs to be defined, ensuring that mechanisms to collect and assess relevant information are in place. Moreover, it will require the creation of integrated data management systems.

Labour market

There is little consensus on net labour gains or even which parts of the plantation process create the most jobs and lack of research to help clarify the situation, although work in different stages of the pulp process are identified:

- The evolution of tree nurseries is generally believed to have been positive for Uruguay because prior to this, companies imported the plants

- Plantation work is recognised as cyclical – the establishment phase uses the most people although maintenance is generally better paid

- The wood industry, pulp mills etc are generally thought to produce more constant and permanent employment, located in urban centres, but it is still only poorly developed

- Services for the sector is an important additional dynamic factor for the economy

There is concern about the quality of employment, particularly in earlier times and perhaps caused by the very rapid growth of the sector. Those improvements that have taken place are believed to have been mainly as a result of regulation and enforcement, sometimes driven by outside forces such as certification requirements and sometimes by contracting companies themselves. In general, contracting companies are singled out as a very different type of entity within the field, requirigin different approaches. There was no clear agreement about the extent to which regulation of this sector is needed, some people believing that it stifled investment while others felt it was essential to maintain workers’ rights.

Many interviewees stressed the unique nature of the forestry sector but also that it had to develop and grow, for example through agreement of stronger codes of practice etc. Formation of syndicates is regarded as one way of helping to address these problems. Lack of training is identified as a problem although one that many people felt should be addressed first by the companies themselves.

Vision of the forestry sector in relation to other sectors

There is concern that the forestry sector is isolated from other sectors, including particularly livestock and agriculture

Everyone appears to think that the forestry sector is relatively isolated from other productive sectors in the country. However, while the government beloieves the forestry sector isolates itself, other stakeholders think the reverse, i.e. that forestry is being isolated by other sectors of the economy, particularly livestock and agriculture, which have a powerful lobbying capacity. Nowadays, the forestry sector appears to be gaining ground in this respect according to the interviewees. However, at a production level, complementarity among the sectors can be observed, promoted both spontaneously and through actions by the government. Underlying this is recognition that plantation forestry is a growing sector of the economy, driving development and with clear strategies for development and increase in export markets.

Within the very forestry sector itself, there appears to be an established relationship between big companies and small producers, albeit with some confrontation, based mainly on the disadvantages that small producers face regarding the access to land and the restrictions promoted by the big companies, which leave many of the small producers behind. At present there are two opinions about the way forward: on the one hand a suggestion that large companies should integrate smaller ones into their operations by means of contracts or agreements; on the other that small companies should unite themselves, although this option would require state support and there are no clear policies for this as yet.

Assesment and future vision of the sector

Interviewees agree that the view of forestry reported by the media originates more from opinions within Montevideo, but the one closer to reality comes from the rural population. A similar situation occurs with respect to the agricultural and livestock sector. However, there is throughout a tendency to have a rather cool and initially negative opinion about plantations, which has been greatly influenced by current debates on the installation of the pulp mils; by ideological issues regarding foreign investments and by environmental factors. Some of the antipathy has deeper roots in a country that is based around the livestock and agricultural sector and has probably been intensified by lack of information from the plantation sector and inappropriate handling of some controversial issues.

In summary, there is consensus that the forestry sector – all the way from government institutions to vompanies – has not led the development process in a systematic, consistent or consensual way, nor has it provided sufficient information about planned activities.

In fact, all interviewees were concerned about the small amount of research that the sector has generated. In general terms, gaps in information relate to technologies that ensure the maintenance of the environmental resources; general research on the production process and industrial development; management; and the wood industry. There is recognition of the need to develop this sector within Uruguay. Whilst it is acknoweledged that companies pay for much useful research, this is generally not communicated from the private to the public sector. Interviewees identified a number of important research elements, outlined in the box below.

There was general consensus that for more research was needed within the sector and a list of topics could be identified

Box: Research needs identified by stakeholders

- Monitoring and evaluation and use of results

- Provincial information systems regarding labour and company statistics

- Inventories of forestry sites

- Social impact of the plantations with respect to workers and local communities

- Characteristics, technology and use of woods (for example ethanol production and sawnwood drying by means of ultrasound)

- Best practice with a variety of species

- Environmental impacts on soils (soil degradation); water (impact upon the quantity of surface water and groundwater, also on its quality); biodiversity (impact upon biodiversity in general and in relation to the campos)

Assessment of the forestry sector

There was a marked difference of opinion between interview groups regarding opinions of the sector, sumamrised in Table ## below

Table ##: Opinions regarding development of the plantation sector

| Stakeholder | Positive opinions | Aspects to be improved |
| --- | --- | --- |
| OPYPA-MGAP: Planning and Agricultural and Livestock Policy Bureau- Ministry of Cattle, Agriculture and Fisheries | Creates dynamic expectations | Highly centered model, does not integrate small and medium companies and complementary activities |
|  | Diversifies the Uruguayan economy (agricultural, livestock, industry, exports). | Better State regulatory and managerial capacity, both regarding environmental planning and assessment |
|  | Generates direct and indirect employment |  |
| DGF – MGAP: General Bureau of Forestry- Ministry of Cattle, Agriculture and Fisheries | Generates local employment | Invested capital was above the decree law and environmental issues |
|  | Develops workers’ organisations | Existence of subsidies has skewed the sector |
|  | Rise in the macro economic rates. | High concentration of land ownership |
| MTSS-IGT: Min. of Labour and Social Security- Labour General Inspection | Possibility of creating good jobs | Permissive state attitudes helped create bad working conditions |
|  |  | Environmental suspicion concerns our people |
| MTSS – DINAE: Ministry of Labour and Social Security- National Bureau of Employment | Generates employment in the provinces in harsh economic periods | Jobs have low levels of secuirity |
|  | Regenerates the social tissue and economic distribution. | Predisposition to generate “slave” labour. |
|  |  | Predisposition of contractors to avoid their responsibilities. |
| MIEM – Mesa Tecnológica: Ministry of Industry, Energy and Mining- Technology | Generates employment | Economic and political interests could divert attention. |
|  | Generates export exchange for the country | Environmental concerns |
|  | Incorporates technology | Migration of rural population due to land concentration |
| DINAMA – MVOTMA: Nat. Bureau of Envt- Min of Housing, Territorial Organization and Environment | Improves the living conditions in the countryside and in the country as well. | Concentration of land ownership |
|  | Promotes employment in the countryside. |  |
|  | Improves conditions regarding livestock in biodiversity. |  |
| F. AGRONOMIA: School of Agronomy | Successful | More training at an intermediate level |
|  | Attractive internationally due to the conditions of the country | Develop incentive for high quality management. |
| INIA: Nat. Bureau of Ag. & Livestock Research | New economic area | Difficult to reach consensus in a newly developing area |
|  | Research opportunities are attractive since it is a new field |  |
| CIEDUR: Centre of Interdisciplinary Studies on Development | Long term coherence. | Environmental impact |
|  | Specific policies are formulated in the management of the countries | New state policies are required |
|  | New investment opportunities for some parts of the country | Foreign investment creates enclaves |
| CLAEH: Latin American Centre of Human Economy | Generates employment and dynamic economic conditions | Disorganized early stages with lack of clarity about where products would be used |
|  | Chance to promote the integration of different local capacities. | Social impacts of increased displacement |
|  | Generates services and conditions which did not exist previously | Events happening too fast without enogh discussion |
| AIAU: Uruguayan Association of Agronomists | Successful project, aims were settled and accomplished | Integration with agricultural and livestock sectors, promotion of mixed models |
|  | Increased value of rural land | Predominance of big foreign companies |
|  | Develops industrial base in the rural areas | Aim at more valuable species and processes |
| SPF: Uruguayan Association of Forestry Producers | Generates employment in the rural sphere | Lack of integration with agricultural and livestock |
|  | Alternative options for low profit livestock areas | Communication from the sector to the rest of society |
|  | Good planning and land organization | Mistakes due to lack of research |
| Asociación Contratistas Forestales: (Forestry Contractors) | Generates employment in the countryside | Causes increased inequality in rural areas |
|  | Sustainable production with positive environment balance |  |
|  | Generates exchange |  |
| ADT: Diagnostic Analysis beyond frontiers | Economic development and growth of service sector | Rapid, monoculture-based process |
|  | The big companies have set positive regulations | Inequality due to support for large companies |
| IMT: Borough Hall of Tacuarembó | Generates employment | Government subsidies and rural contributions in particular |
|  | The sector contributes to provincial & national economy | From the environmental point of view, it remains unclear what happens once the woodlands are exploited |
|  | Productive sector, attractive to foreign investment. |  |
| IMD: Borough Hall of Durazno | Generates employment and wealth | Tax exemption is too large & interpretation of law is unclear |
|  | Early vision for plantations | Bad working conditions |
|  | Environmental reserve created by plantation woodlands | Non- discriminated exports of non-industrialized wood. |

Future developments

There is consensus that the current model will continue and some major growth is expected in the next 10-15 years. Greater attention needs to be paid to the social aspects of the development and particularly promotion of complementary activities among the different productive sectors. In general there is a hope that in future there will be a better balance between large and small companies although the latter are still expected to lead the process.

Some issues to be considered by the company

Getting stakeholders to discuss concrete actions was difficult and tended to create tensios within the interviews, but the following points were generally identified:

- Promote social dialogue with stakeholders and local authorities, remaining open to criticism, and social consensus

- Define clear communication strategies that explain what is happening to local stakeholders

- Promote forestry training and options for gaining local qualification

- Make activities known including holding meetings with the local people in order to inform them of the impact the company will have on the area

- Promote codes of social and company responsibility

- Decentralize the company regarding decision making

- Promote research together with other national or provincial stakeholders to foster a proactive attitude of the company and its commitment to the development of the sector in the country

- Promote dialogue instances with other sectors, through existing producers associations

The Historical Environment

The following overview of the historical importance of the proposed plantation areas is based primarily on bibliographical and cartographic information and analysis of satellite photographs, as well as studying archaeological collections from the area. To help set the context, the box below outlines the main historical periods in Uruguayan history and prehistory: photographs and collections are described overleaf.

Box: Timeline of history in Uruguay

- Paleo Indian or First American Period:  12,000–9,000 Before Present (BP). Indicator: presence of “Fell I” or “fish tail” projectile points. Environmental characteristics: end of the glacial period, significant modification of the environment and landscape, extinction of pleistocenic megafauna. Numerous records on littoral sites of Middle Negro River and Tacuarembó River.

- Archaic Period: 9,000–5,000 BP. Indicators: high stylistic variability of projectile points (sizes and shapes.) Polished artifacts (rompecabezas – a weapon with two iron or lead balls on a short, flexible handle), mortars, etc.  More complex artefact inventory. Environmental characteristics: interglacial conditions. The period ends following a peak in temperature (sea level 5 metres.) Temperature and humidity are higher than now. Numerous records on littoral sites of Middle Negro River and Tacuarembó River.

- Post-Archaic Period: 5,000–2,500 BP. Indicators: some researchers believe the surge in mound construction (mounds without pottery) occurs in this period. Environmental characteristics: climate nearing current conditions with periods of aridness. Human groups have to respond culturally to the change in environmental conditions.

- Incipient Formative Period: 2,500–400 BP. Indicators: a surge in pottery.  Mound construct ion found in the extreme south of Brazil and in the east and northeast of Uruguay. Pottery from the Middle Negro River region shows distinctive characteristics with decorations defined as “pointed” incisions. From this period, the surge in rock paintings (pictographs) is notable. Environmental characteristics: current conditions, with more extensive highland and riverside wooded areas and abundant brush vegetation.

- Hispano-Indigenous Contact Period: 16th and 17th centuries. Indicators: the introduction of livestock (horses, cattle and later sheep) and other non-native species (e.g. canines) transform the landscape, progressively pushing out native fauna. Native flora contracts to highlands and river areas. Large portions of the territory are deforested, leading way to the prairies of today.

- Colonial Period and Beginning of Independent Uruguay Period: 18th and 19th centuries. Indicators: occupation of territory and consolidation of cattle raising, developing from the “cowshed of the sea” (Jesuits) to the fencing in of the fields. Land registries of the 19th century reflect various technological changes in livestock practices, land tendencies and demographic changes. Noteworthy are the constructions with dry stone technology:  corrals, fences, housing, etc.  Battles leave archeological records in various battlegrounds.

Aerial photographs

Detecting the prehistoric evidence of the area using satellite photography is not efficient, with the exception of the “cerritos indios” (Indian mounds). In this last case, a 1:20,000 (SGM) photograph was taken successfully: nonetheless, the procedure is not able to record low-lying mounds. However, research into the historical evidence of narrowing chutes, corrals and stone fences can accomplished quite effectively by this resource. Aerial photographs 1:20,000 (SGM) and high-resolution satellite images (pixels equaling at least 20m of surface area) are appropriate for detection of these structures. Processing of digital images through use of specific programmes (e.g., IDRISI) may also be useful to facilitate the detection of prehistoric and historical structures over wide surface areas.

Private collections

Our in-house analysis revealed the existence of numerous private collections of prehistoric archeology of the Middle Negro River, with few references.  In five days, 13 private collections of the area were categorized. Respectful personal relationships with collectors led to enriching exchanges. Hundreds of archeological artefacts were photographed, owners interviewed and information on original locations recorded and entered into the GIS digital mapping system. Valuable empirical observations from the collectors were gathered regarding the various archeological sites and possibilities were discussed for collaboration in the development of local museums.

Private archaeological collection in San Gregorio

.

Arturo Toscano [?]

Table ##: Private collections in the region that supplied information to the study

| Collection | Location | Inventory | Marked | No. of pieces |
| --- | --- | --- | --- | --- |
| Cayetano Álvez | Durazno | No | No | 750 |
| Rivera House Museum | Durazno | Yes | Yes | 1,000 |
| Séptimo Bálsamo | Paso de los Toros | No | No | 1,500 |
| Washington Aizpún | Paso de los Toros | Yes | Yes | 2,498 |
| Julio Bálsamo | Paso de los Toros | No | No | 300 |
| Muga | San Gregorio de Polanco | No | No | 3,000 |
| Julián Machado | San Gregorio de Polanco | Yes | Yes | 500 |
| Julio Santos | San Gregorio de Polanco | No | No | 50 |
| Juan Pusillo | San Gregorio de Polanco | No | No | 250 |
| Luís Rodríguez | San Gregorio de Polanco | No | No | 20 |
| Indian Museum | Tacuarembó | Partial | Partial | 2,500 |
| Milton Coore | Tacuarembó | No | No | 750 |
| Omar Michoelsson | Tacuarembó | Yes | Yes | 500 |
|  |  |  | Total | 13,618 |

Summary of findings

To check:

We need the references

For the purposes of analysis and to help make recommendations regarding conservation of historical areas and artefacts, the archaeological record has been differentiated into two periods: prehistoric and historic.

Prehistoric records

Four main centres of activity can be identified, each with distinctive features:

- Littoral sites: Middle Negro River and Tacuarembó River: settlements often took place along or close to rivers. For example the Middle Negro River has significant prehistoric littoral archaeological deposits: example, thousands of projectile points and boleadoras (a type of sling).

- Lowland hill mounds: East of the Tacuarembó River. Indian earth mounds are cultural manifestations of the human groups occupying an extensive lowland territory for several millennia in the southern state of Rio Grande do Sul (Brazil), and the Uruguayan border zone. Excavations in the Yaguarí valley found a mound over 3000 years old, measuring 32 m in diameter and 1.8 m  in height (Taddei date) along with ceramics, food remains, bonfires and with a complete human burial site. Over 600 mounds have since been identified from the same valley (López et al 2004) demonstrating the long and complex maintenance of monumental spaces by this culture. Unfortunately hundreds or even thousands of mound structures have been destroyed by agriculture and livestock management and particularly the expansion of rice.

Rock art sites: Southern Department of Durazno. There is a significant amount of rock art associated with granite outcrops in the central-south region of Uruguay. These are usually on isolated rock surfaces in the open and consist of geometric abstracts and drawings, ranging from a few centimeters to over one square metre, and drawn in red pigment (Fe2O3 hematite) covered by a thin translucent silicate layer that preserves them. They are often covered by lichen, which makes detection difficult, and many undoubtedly remain to be discovered. To date, the rock art sites are the only prehistoric archaeology remains ro have received legislative protection as historical monuments (Resolution 842/995.)

Rock art sites have legislative protection as historical monuments.

- Stone piles: It is common to find various classes of “artificial piles” of stones in the hills and elevated areas in Uruguay; these were first described by Darwin at the Sierra de las Animas in 1832 (Figueira 1982). Two types have been distinguished: cone-shaped and ring-shaped piles either isolated or in groups and both can be seen at a same hill. Little has been determined about their function or chronology but they have generally been attributed to initiation ceremonies or death rituals, although there has been speculation that the ring-shaped piles might be animal traps. In the Department of Tacuarembó, findings at Charrúa Hill have been reported, but a large part of the elevations of the department have not been surveyed. There is sometimes confusion between prehistoric piles and European dry stone construction.

Historic records

Since European settlement, two main types of records exist:

- Rural architecture: a study of transformations in rural architecture reflects the socio-cultural processes occurring over four centuries of occupation, although records are very incomplete and there is much to be learnt. Rural architecture associated with livestock activity shows various construction technologies: (1) dry stone (corrals, fences, narrowing chutes, housing and stands); (2) mud construction (straw and sod); and (3) rubblework with plaster or mortar (tubes, bathrooms, sheds, housing, burial sites, etc.) Following a recent visit to El Arazá establishment and others in the area, the following states of conservation of dry stone corrals, narrowing chutes and fences could be evaluated on a preliminary basis: (1) original: original function maintained, including maintenance (construction technology maintained or not.); (2) modified: functionality maintained, in association with modern post and wire fences; (3) abandoned: loss of functionality, progressive deterioration with invasion of trees and shrubs and partial dismantling; and (4) dismantled: some support ashlars, because of extraction to reuse stones.

Historical walls and buildings are found on several of the properties bought by Stora Enso

- Battlegrounds and other historic events: in the area of interest, we found at least 20 sites where episodes not only left their historical memory, but also show related material. When there are documents or remnants from wars, solid references to buildings, notable geographical features (passages, trails, hills, etc.) are explicit. In other cases, when document references are scarce, archaeology can offer the appropriate methodology to identify material vestiges of these events. Precise geographic location of battles is often difficult as many sites are controversial or have never been precisely identified. Following bibliographical research a database has been created and geo-referenced in GIS, classified as (1) reliable / consensus exists; (2) vague or controversial information; and (3) tentative / generic information. The following chronological periods can be distinguished:

- Colonial Period (17th century – 1811)

- Artigas Revolution – Portuguese Invasion (1811-1820)

- Founding of the Uruguayan State (1830-1839)

- Great War (1839-1851)

- Caudillo Period (1851-1875)

- Military Period (1875-1890)

- Revolutionary Period (1897-1904)

- G. Terra Dictatorship (1933-1938)

Table ## below outlines some of the main battle sites in the region under study.

Table ##: Known battlefields in the plantation landscape

| Battle | Year | Date | Location | Department |
| --- | --- | --- | --- | --- |
| Puntas del Yí | 1702 | 6 February | Puntas del Yí |  |
| Guayabos | 1815 | 10 January | Guayabos Stream | Paysandú |
| Cordobés | 1816 | November | Cordobés Stream |  |
| Tacuarembó | 1820 | 22 January | Tacuarembó Chico Stream | Tacuarembó |
| Matanza de Salsipuedes | 1831 | 11 April | Salsipuedes Stream |  |
| Tupambaé | 1832 | August | Tupambay Stream | Cerro Largo |
| Yarao | 1834 | 14 May | Near Tacuarembó Chico Stream | Tacuarembó |
| Carpintería | 1836 | 19 September | Carpintería Grande Stream | Durazno |
| Yí | 1837 | 21 November | Yí River | Durazno |
| Molles y Sauce | 1844 | 24 January | Molles y el Sauce Stream | Durazno |
| Hecatombre de Quinteros | 1858 | 28 January | Mouth of Rolón Stream | Río Negro |
| San Fructuoso | 1863 | 31 July | Tacuarembó Capital | Tacuarembó |
| Pedernal | 1863 | 9 September | Pedernal Hill |  |
| Tres Árboles | 1897 | 17 March | Tres Árboles Stream | Tacuarembó |
| Paso Rubí | 1897 | 25 March | Las Cañas Stream – Paso Rubí | Durazno |
| Las Palmas | 1904 | 17 January | Las Palmas or Paso Billar | Durazno |
| Cerro Dos Hermanos | 1904 | 17 January | Dos Hermanos Hill | Durazno |
| Cordobés | 1904 | 18 January | Pass at El Cordobés Stream | Durazno |
| Tupambaé | 1904 | 23 June | Tupambay Stream | Cerro Largo |
| Picada de Ladrones | 1935 | 4 February | Negro River coast | Durazno |

In summary, the plantation will need to consider many specific archaeological sites, most of which will not yet have been described or identified, and also a range of more historical sites and landscapes, a proportion of which remain as obscure as those of prehistory.

Location of historical “hotspots” with Geographic Information System (GIS)

Spatial location of hotspots was performed by GIS. ArcView 3.2 software was employed to geo-reference sites with their respective databases and basic location references. The GIS was organized into two folders: (1) geographic database: hydrography, main and secondary roads, departmental borders and localities; and (2) archeological indicators: littoral sites, mounds, rock art, historical sites and battlegrounds. The locations were marked through “points.” However, these represent variable expression polygons on the terrain and must be taken simply as an indication. For improved information management by people unfamiliar with GIS, the project is presented in a self-running executable (Flash) through the ArcEXPLORER 2.0 program (free software.) The program is an easy-to-use digital cartography browser and a user manual is included on the CD (.pdf format.) Specialized usage is possible by copying files D:\fscommand\Sitios (.shx, .shp, .dbf) on the CD for use in GIS programs compatible with ArcView

Figure ##: GIS reference on archaeological and historical sites

Some possible indicators

To improve management of the area of study, locations characterized by the presence of relevant evidence material are proposed for archeological indicators.  These include:

- Prehistoric archeological finds (camp sites, shop sites, rock art, mounds, stone pilings, etc.)

- 17th – 19th century historic housing and constructions (ranch house and stand foundations, stagecoach posts, general stores, burial structures, etc.)

- Historic narrowing chutes, corrals and stone fences (dry stone)

- Historic sites where notable events took place of regional or national interest (battlegrounds, meeting places or historic proclamations.)

With these four indicators, diverse archeological remains can be organized and identified. Nevertheless, it will also be necessary to evaluate the relevance of each site or particular remains. This will need analysis of the value that various stakeholders attribute to particular archaeological remains, drawing on opinions of:  (1) local community; (2) national community; (3) the State through departmental and national legal offices; and (4) archeology as a specialized discipline.

Likely impacts of plantations on archaeological and historical heritage

Forestry can have two types of impact on archaeology: direct effects on archeological sites and effects on the cultural environment or landscape.

- Direct impacts: intensive forestry can potentially destroy or irreversibly alter archeological remains. Plantations imply various interventions on the land that impact in a differentiating manner according to the types of archeological evidence. In the case of prehistoric remains, the preparation and sowing of seedlings can in some cases cause deep cuts in the earth, which in turn would cause partial or total destruction of an archaeological site. Meanwhile, tree plantations may cover and seal archaeological sites making their identification and scientific knowledge difficult (e.g., prehistoric littoral sites on meadows.) In the case of historic records, the conditioning of specific infrastructure (foundations, sheds, etc.) can mean the dismantling or destruction of buildings and other structures of historical value. For an area such as the one under consideration, with poor knowledge of archaeology and no systematic surveying, archaeological studies are therefore necessary prior to forestry activities.

- Impacts on the cultural environment or landscape: UNESCO (1999) has recognized the value of the landscape: “cultural landscapes often reflect specific, viable land utilization techniques having accounted for the characteristics and limits of the natural environment where they are located, as well as a specific spiritual relationship with nature…” (Article 38.) For example, rock art sites require preservation of their surroundings, including geological outcroppings and native forests, as an indivisible unit that together with art forms the cultural landscape. In 1998, the Heritage Commission established an 800-m radius of protection for pictographs. Similarly, battleground scenes are relevant where the demarcation of the area is complex due to the size and vagueness of the location. The alteration of the landscape in these historic locations could imply a deterioration of heritage.

Policy, legal and administrative context

MORE TO COME HERE

To do:

None of the consultants provided an overview of key legislation –what we have is specific bits for specialised areas – we still need an overview

Legislation relating to water resources

A recent amendment to the Constitution of Uruguay states that Uruguayan water resources belong to the State, may not be privatized, and may only be managed by legal state entities. In addition to the provisions set forth in the Constitution, the Code of Waters (Law Nº 14.859) determines the norms that regulate the waters in Uruguay. Responsibility falls mainly to two entities:

- National Direction of Hydrography (DNH), part of the Department of Transport and Public Works (MTOP): DNH plans use and sustainable development of water resources, ports and coastal works and enforces regulations.

- Obras Sanitarias del Estado (OSE - the State Water Works) is responsible for the supply of drinking water. It services 2,996,750 inhabitants in 756,282 connections, covering 98 per cent of people living in settlements.

These two bodies control rights over drilling wells and using water. Pumping and use of underground water usually require authorization from the MTOP-DNH, according to Article 3, Law Nº 17142, 14 July 1999. However, there are exceptions in the case of water for human drinking and hygiene needs, and drinking water for cattle. Applicants can request the right to use water and thus be registered in the Public Register of Waters (Article 8 onwards of the Code of Waters), generating rights before the Administration and third parties. Registering drilling with the DNH offers legal protection from future users who may want to make wells nearby or in the same fractures and area of influence, when these could interfere or deplete the flow due to overexploitation of the aquifer. This legal protection can even prevent drilling future wells in areas near registered wells

NEED LEGAL DETAILS ON AQUIFERS – SUPPOSED TO BE IN AN APPENDIX

Legislation relating to natural forests

Riverside forests (as well as other types of native forests) are protected by Law 15.939 and may not be exploited for commercial purposes, except in the ways provided for in Decrees 22/993 and 330/993 – relating to a special request filed previously before the General Directions of the Renewable Resources and Forestry of the Ministry of Cattle Production, Agriculture and Fishing, together with a management plan. The General Forestry Direction assesses the acceptability of the activity and the management plan, and may approve or not the request. Fire prevention rules also establish a protection strip of 20 metres between forestry plantations and the border of the native forest.

Legislation relating to mammals and birds

From the origins of the República Oriental del Uruguay, governments have drawn up a sucession of laws and degrees relating to animals, starting from the Rural Code (Código Rural) of 1875 that elaborates norms about hunting. In additrion, Uruguay is signatory to a variety of international treaties and codes that relate to the protection of animals. A list of the key pieces of legislation, many of which relate to either control of hunting or captive breeding, are outlined in Table ##.

Table ##: Key articles of legislation relating to protection of wild animals

| Law | Details | Date | Details |
| --- | --- | --- | --- |
| Administration and Regulation about Native Fauna. | Law No. 9481 | 4 July 1935 | Law relating to the exploitation of native species (included birds). People interested exploitation of a particular species have to present a proposal to the Commission of Fauna Protection. |
| Convention for the Protection of Flora, Fauna and Natural Scenic Beauties | Law 13776 | 17 Oct 1969 | Convention for the protection of flora, fauna and natural scenic beauties of American countries. In the different Appendixes the rights and duties of the signatories are described |
| Convention relating to Commerce of Threatened Species (CITES) | Law 14205 | 4 June 1974 | Convention regarding commerce of wild fauna and flora threatened species. Species included in the Convention and Appendixes are listed |
| Greater rhea (Rhea Americana) | Decree 483/79 | 1979 | Relating to trade in feathers |
| Amendment to the CITES Convention | Law 15.626 | 11 Sep 1984 | Amendment to the CITES convention of 1973 |
| Greater Rhea – Feathers or feather duster – tenancy | Art. 10 Decree 254/85 | 1985 | Owners of rhea breeding stations have to have proper identification |
| Wild fauna | Decree 693/87 | 9 Dec 1987 | Creation of the Advisory Commission about native fauna and flora |
| Convention – Conservation of Wild Animal Migratory Species | Law 16062 | 6 Oct 1989 | Species including in the convention are listed |
| Fierce or wild animals | Law 16088 | 11 Oct 1989 | Regulation relating to captive wild animals, prohibiting keeping wild animals outside wild parks or zoos |
| Native fauna | Decree 655/91 | 1991 | The decree 693/987 is modified, in reference to an honorary advisory commission on native flora and fauna |
| Chestnut-capped blackbird | Decree 475/91 | 1991 | The Chestnut-capped Blackbird (Agelaius ruficapillus) is declared national pest |
| Hunting | Res. 393/96 | 8 May 1996 | Relating to permission for hunting |
| Hunting | Decree 164/96 | 21 May 1996 | Also about hunting permits, describes the species and times of free hunting |
| Hunting | Decree 165/96 | 21 May 1996 | Sport hunting, permitted species and hunting seasons |
| Hunting | Decree 352/96 |  | Regulates sport hunting derivates |
| Hunting | Decree 119/98 | 8 May 1998 | Regulates hunting seasons and species |
| Hunting | Decree 126/99 | 1999 | About hunting permits and transport of hunted animals including how many animals each hunter can transport |
| Red winged Tinamou (Rhynchotus rufescens) | Decree 416/01 | 24 Oct 2001 | Protection of breeding places for Red-winged tinamou |
| Wild fauna | Decree 514/00 | 7 Jan 2002 | Official list of wild fauna of Uruguay |
| Breeding native fauna | Decree 186/02 | 23 May 2002 | Regulations controlling breeding of native fauna |
| Monk parakeet: control of species | Decree 343/02 | 30 Aug 2002 | Regulation on the campaign to control the Monk Parakeet |
| Greater Rhea – Breeding industry | Decree 186/02 | 11 Feb 2004 | Registration of establishments for the Greater Rhea breeding industry |
| Hunting | Res. 386/04 | 16 Dec 2004 | Relating to sports hunting permit for non-resident people |
| Convention relating to the conservation of wetlands important for migratory birds (Ramsar) | Law 15337 |  | Relating to protection of internationally important wetlands and associated species |

Legislation relating to insects

Uruguay does not have legislation indicating conservation measures for species of insects. The "Red Lists" from Rio Grande do Sul in Brazil (Fontana et al., 2003) may be used as a regional document. The following protected species are listed in the orders Coleoptera and Lepidoptera.

Table ##: Red List insect species in Brazil

| Species | Status in Brazil Red List |
| --- | --- |
| Coleoptera: Cerambycidae |  |
| Plaumanniella novateutoniae Fisher, 1938 | Threatened in the region and vulnerable in all Brazil |
| Quatiara luctuosa | Threatened, vulnerable in Rio Gde do Sul |
| Coleoptera: Chrysomelidae |  |
| Anisobrotica donckieri | Threatened, vulnerable in Rio Gde do Sul |
| Doryphora reticulata | Threatened, vulnerable |
| Ensiforma caerulea | Threatened, in danger |
| Monocesta rubiginosa | Threatened, vulnerable |
| Schematiza aneurica | Threatened, vulnerable |
| Monocesta androgyna | There is not enough information and the records are very old. They may be considered in danger |
| Caraguata bella |  |
| Caraguata tarsalis |  |
| Chlorolochmaea paralella |  |
| Lepidoptera |  |
| Thysania agrippina | Threatened, in danger |
| Pterourus hellanichus |  |

Legislation relating to archaeology

Legislation related to the protection of national historical heritage is undergoing a process of change. Currently, a new law is being studied that would transform heritage management, in compliance with recommendations established by UNESCO over the last 20 years. It would be modeled on current legislation in Spain and Argentina. For archeology, these modifications would raise levels of protection and caution. Therefore, it is recommended that Stora Enso focus on the proposed laws, in concordance with recommendations from UNESCO, ICOMOS and other bodies, even though they currently are not contemplated under national legislation.

The main current legislation includes law 14,040 (with corresponding decree 536/1972) and law 15,819 (decrees 618/86, 138/81, 419/91.)  Departmental legislation also includes protection for prehistoric and historic heritage. The Department of Tacuarembó has led the way in developing protection resolutions for historic stone corrals, fences and narrowing chutes as well as stone pilings and Indian mounds. Detailed legal documents related to heritage protection are available in the working paper on archaeology. These were taken from the Heritage Commission web page (http://www.patrimoniouruguay.net) and the Bureaus of Culture of Tacuarembó and Durazno.

Section 3

Issues relating to sites and landscape

Methods used in the study

Planned activities

Significant environmental, social and economic considerations

Landscape context

Mitigation management

Rights-based development

Monitoring plan

Methods used in the study

Planned activities

The original terms of reference were to provide a detailed analysis of planned activities, but in the event this has proved difficult to achieve because so much of the future developments remains undecided. We are slightly concerned that so little has actually been written down concerning the aims and management of the project, both at the individual level in Uruguay and more generally about the aims of the whole operation in Latin America. Partly this is because it has made our job more difficult but it also suggests there is as yet insufficient clarity in thinking about the final aims of the project, which increases the risks that it will be unsuccessful and/or have detrimental or unforeseen impacts. Lack of documentation also made it more difficult for us to encourage some stakeholders, particularly larger NGOs, to engage in the project.

This is clearly not adequate as it is…

Table ##: Current state of development in project planning

| # Element of an management | # Status |
| --- | --- |
| # Planning |  |
| Feasibility studies | No full feasibility study – but a costing exercise |
| Management plans | No management plan available |
| Work pans | No work plans available |
| Guidance | Draft guidance for contractors covering legal, social, environmental and contractual issues |
| Environmental management systems | Not yet – plans to develop by mid 2007 |
| # Social values |  |
| Stakeholder impacts | As part of the current study |
| Jobs likely to be provided | Not known yet |
| Percentage of jobs going to local people | Favour local people but no guarantee of jobs; a gradual shift to local people over time |
| Percentage of jobs going to local people | Usually senior staff / consultants; strict policies to check if staff are allowed to work in Uruguay |
| Wage levels | Average or slightly above average |
| Trades unions | Trades union organisations allowed |
| Out-grower schemes | Currently about 20 per cent, may be increased |
| Local community liaison officer | Social expert plus important role for regional managers and local staff (e.g. housekeepers) |
| Consulting with local communities | As part of the current study |
| Policy on roads | Most roads on private land and not available to public, SE helps to upgrade some public roads |
| Biological values |  |
| Studies of biological impacts | Continuing studies on some sites also landscape characterisation studies |
| Policy towards: |  |
| - Natural forest fragments | Protected by law |
| - Riparian woodland | Protected by law |
| - Wetland areas | Protected by law |
| - Large individual trees | Sometimes protected (e.g. palms) |
| - Valuable grassland | No policies at present |
| - Corridors, nest sites, lairs | Policies to be developed (as part of the current project) |
| - Rare or threatened species | Protected by law if on the IUCN Red List |
| Cultural values |  |
| Staff responsibility for protection of cultural and aesthetic values | Day to day responsibility by surveyors. Cultural values included on checklist for land purchase |
| Policy towards the following: |  |
| - Archaeological sites | Legal protection in place |
| - Historical sites | Policy yet to be set |
| - Burial sites | Legal protection in place |
| - Areas of local values | No policy |
| - Old management systems | No policy |
| # Environmental values and management |  |
| # Tree species |  |
| # Species and proportions of trees planted | # Current proposals are for 80:20 ration of Eucalyptus to pine, mainly E. duneyi |
| # Expected planting rate | # Planned for 13,000 ha/year |
| # Planting density |  |
| # Chemical use |  |
| Expected use of fertilizers | 610kg/ha – no written policies, needs review |
| Expected use of herbicides | Roundup applied to planting rows before or just after planting and at least once more. Policy unset: may need general treatment |
| Expected use of pesticides | Against ants and possibly other pests |
| Staff member responsible for environmental management | In place |
| Particular issues relating to fertilizers: |  |
| - Matching application to sites | One standard for all operations |
| - Applying during heavy rain | No written policies, use in heavy rain avoided |
| - Applying near water or aquifers | No written policies |
| - Steps to monitor impacts | Investigated in the current study |
| Particular issues relating to biocides: |  |
| - Application methods | No aerial spraying used |
| - Alternatives to herbicides | No policy |
| - Safety training | No training, requirements for contractors |
| - Avoiding drift or pollution | Follow Forestry Law and Water Protection Law |
| - Storage and disposal | No written policy |
| # Invasive species |  |
| Policy towards invasive species | No policy |
| Staff training on invasive species? | No training at present |
| # Site preparation |  |
| Site preparation, including machinery and procedures and covering in addition: |  |
| Policies to prevent soil erosion and soil compaction, including: |  |
| - Ploughing on steep slopes | No steep slopes on the land |
| - Heavy machinery in wet soils | No planting in wet soils |
| - Soil compaction | Need to agree policies |
| - Cut-off drains on slopes | No steep slopes |
| - Construction of settling pools | Not developed yet |
| - Seepage buffer zones | Not developed yet |
| # Planting |  |
| Planting policies | Generally hand planting |
| Maximum size of planted blocs | No size set. Usually < 60% of a site planted |
| Buffer zones, corridors etc | Encouraged, need further guidance from ESIA |
| # Harvesting and extraction |  |
| Details of harvesting procedures: |  |
| - Timing to minimize impacts | Harvesting methods not agreed yet |
| - Planning of coupes |  |
| - Planning extraction routes |  |
| - Safety equipment |  |
| # Road and bridge building and use |  |
| Details of plans for road building, kilometres expected, procedures used and maintenance, with consideration in addition of: |  |
| - Avoiding high erosion risk | Policies still to be set |
| - Bridges, ditches and culverts, drains, silt traps and pools | Policies still to be set |
| - Speed limits | No speed limits |
| - Size and weight of vehicles | Policies still to be set |
| - Contact for local communities | Social expert, local site manager |
| # Fire control and management |  |
| Responsible staff | One team members is appointed fire officer |
| Fire management plan | To be developed |
| Fire breaks | Legal requirement. Fire breaks planned for and maintained – possibly through cattle grazing. |
| Policies towards fire towers | Talking with companies about fire towers |
| Training and equipment for staff | Training planned but not undertaken yet |
| # Staff training |  |
| Details of training courses for permanent and temporary staff | No courses yet, some courses run on site by NGOs |
| Details of training courses for contractors | Instructions available for contractors. |
| Details of training courses on social and environmental issues | None available yet |
| # Processing |  |
| Details of end-products of the process, milling etc | Decision still not taken |

Development of the Environmental Management System will therefore be a major step for the company. We cannot comment on this in the ESIA but we do (in Appendix ##) make some detailed proposals for what might be included, building on the existing voluntary Forestry Code in Uruguay. These proposals also draw from an analysis of over fifty different forest codes that already exist and are being implemented in different parts of the world.

Significant environmental, social and economic considerations

Notes for what we should say…

Environment: key issues

 Biodiversity: recognition of the key importance of grasslands and inclusion of conservation measures for grassland within management strategies including:

- Avoiding planting on the highest value sites

- Taking care of residual areas (grassland and other habitats)

- Adding to the national protected area network through set-aside areas

- Avoiding contamination through in particular fertilizer use

- Landscape approaches to planning creating a viable mosaic of linked natural and semi-natural habitats

 Water balance: care to avoid localised impacts on neighbours through:

- Avoiding planting in aquifer recharge areas

- Avoiding planting close to standing or running water

- Planning at a landscape scale both regarding individual impacts and not over-planting within a single watershed

 Environmental management system: focusing on key areas often neglected in management including:

- Planning transport policy

- Minimum tillage systems to avoid erosion

- Care of agrochemical use

Society: key issues

 Transparency: full information to local communities (currently not being done) through:

- Regular meetings

- Specific iaison officers

- Leaflets, article in local newspapers etc

 Contractors: ensuring that contractors maintain the high standards, full EMS etc through

- Clear guidelines and stipulations in contracts

- Training courses

- Selection of workers that includes local communities

 Local benefits: ensure that a reasonable proportion of the benefits reach local communities including by

- Increasing outgrower schemes

- Supporting additional training opportunities for local workers

Economic benefits: key issues

We need further information here – my instinct is that the company is being overly optimistic about what can be planted and probably also about growth levels

Landscape context

Here we need 1-2 pages on key landscape aspects including total area planted in landscape

Mitigation management

Rights based development

To come from Jessica Campese

Monitoring plan

Section 4

Conclusions

Section 5

Tools and appendices

Tools for planning and managing plantations: use of indicators

Although the term biodiversity refers to ecosystems, species and genetic variation, the number of species is the measurement most frequently used as it represents one of the easiest elements to measure at different geographical scales. It is well known that that the spatial structure at landscape level has effects upon the species richness, abundance, and interactions among species in ecosystems, and therefore number of species is a common measure used to identify conservation priority areas.

With respect to biodiversity, indicators are used in two main ways. The first is their use in measuring environmental change or more specifically anthropogenic influence on natural communities and their second is the use of specific indicator groups to estimate the biological richness of a specific area. In the latter, the term bioindicator is applied in the context of the emerging discipline of "biodiversity indicators". Particular indicator species or groups are selected for measurement because of their ability to provide information on the total diversity of the species, and therefore, on the total diversity of the place of interest. It is assumed that the patterns of diversity, rareness, etc., of these groups are indicators of similar patterns in the rest of the organisms in the area. The main difficulty on using indicator taxons to estimate the biodiversity of a place resides in the selection of the right group. Choosing an indicator that both provides a rich enough source of information and is also possible to use without incurring excessive cost is tricky. In an ideal world, indicators would have as many as possible of the following attributes (based on Pearson 1994, Halffter wt al 2001, Dudley and Jeanrenaud 1997 and discussions during the indicator workshop):

- Well-known and stable taxonomy: species groups that are well understood and, for instance, where taxonomists are not still making fairly frequent additions or changes

- Well known natural history: so that there are not liable to be surprises such as the species suddenly appearing in completely unexpected locations or conditions

- Response patterns reflected in other species: so that for instance a decline in the indicator species is likely to indicate a more general decline in the health of the ecosystem

- Easy to observe and manipulate

- Easy to identify: so that ideally indicators can be used by people with only moderate levels of training rather than relying entirely on specialists

- Habitat specific

- Good knowledge of distribution: ideally indicators will be well studied and there should already be a database of information about where they are found

- Available most or all of the year: allowing surveys to take place at any time

- Provide added value: i.e. tell something that cannot be detected in any other way (some indicators are driven by the interests of specialists rather than necessarily being the easiest way of collecting data about a habitat)

- Make sense to people: particularly where indicators are being used in a transparent reporting framework, as will be the case here, they will be much easier to gain acceptance for if they are “sympathetic” to most people – i.e. this tends to act against obscure or hard-to-measure species that will only be known to specialists

Finding something that fulfils all of these needs is probably impossible! But a good indicator should meet many of the above.

Developing a toolkit for site selection and planning

With respect to site selection and planning, Stora Enso needs several pieces of information:

- Information about social conditions to ensure that the plantations play a positive rather than a negative role in development.

- Details of areas where conditions make it more or less suitable for planting

- Information about the natural and cultural values of proposed areas to avoid causing damage to biodiversity, environment and historical legacy

A series of detailed stakeholder interviews have helped to identify hopes and fears about a new plantations and socio-economic studies have also identified the most suitable areas for development in terms of job availability and workforce.

With regard to location, issues of soil quality are important: both to locate the most suitable soils but also to avoid those that may not be best for tree growth – for instance areas with high calcium content. The consultants cautioned against buying areas with valuable mineral deposits because the government has the power to purchase those sites to foster exploitation. And hydrology is also very important, particularly in terms of providing information about areas where plantations could upset water balance in surface or ground water.

If Stora-Enso wants to take conservation seriously in Uruguay, then it needs to look at grasslands. Natural woodlands including riparian woodlands, wetlands and native palm communities are all protected by law – not perfectly but at least well enough that an international company will be expected to protect any remnants in its lands. Stora Enso has already stated informally that it will protect any inland sand-dunes on its property (and these are not particularly suitable for tree planting in any case). The grassland or campos is by far the most common habitat in Uruguay, also to some extent the most unique and certainly the least protected. While we have not come across any grassland that has not been affected by management, it is clear that its value to conservation varies dramatically with geography, soil type and most important of all with previous management history. Permanent pasture that has not been fertilised and never been ploughed for crops is by far the most valuable, but there are many different degrees of value below this. The challenge for Stora Enso is that the differences in value are not always immediately obvious to a non-expert, and it is clearly impractical to send an expert botanist on every site inspection. For this reason we wanted to develop a simple toolkit to help trained non-specialists – in this case probably the forest engineers who make the first inspection of a site – to make an instant judgement about the value of any grassland from a conservation perspective. A second aim is for the toolkit to be used more systematically within a plantation site during the planning process to help identify the most valuable areas that might be set aside, or used as corridors or buffer zones.

An initial attempt at mapping Valuable Grassland Areas in the campos and pampas regions of South America has already been undertaken by Fundacion Vida Silvestre in Buenos Aires, Argentina. However the scale of mapping was quite coarse and the areas were identified by expert opinion (including by one of the consultants in the current project). Whilst very useful in general terms, the mapping is not detailed enough to help with individual site selection and in any case the area under consideration, being less well surveyed than many others, is likely to have been under-reported.

It was therefore decided to identify a series of indicators – species or habitats – that could help provide a tool for site selection and planning. It was hoped that at least some of these indicators would also be useful as a means of monitoring progress with the project and its long term impacts, positive and negative, on environment and society.

A series of specialists in the fields of geology, hydrology, soils, botany (trees, wetlands and grasses), mammals, birds, reptiles and amphibians and insects, archaeology, social conditions and anthropology were all commissioned to provide background information about the region but also to help develop indicators. The same experts were used to help identify indicators. A four stage process was used:

Initial workshop to introduce the ideas and agree the format

↓

Research by individual consultants or small groups to develop landscape characterisation papers on the key topics including proposals for possible indicators from the various disciplines

↓

Second workshop to present results and work collaboratively to develop indicators:

For site selection and planning

For monitoring

↓

Consolidation phase to agree a final set of indicators and produce visual tools

The first workshop took place in January 2007 in Durazno, followed by a two month period in which the landscape characterisation papers were produced and then a second workshop in early April in Montevideo. At the second workshop experts worked in specialist groups (zoology, botany and geography, social and historical) then all together in plenary to brainstorm indicators and agree collectively what would be most suitable. This was a collaborative, cross-disciplinary effort. At the meeting it was also agreed that a “toolkit” for site selection could usefully include the following:

- A suitability map summarising information on soil, geology and hydrology

- An illustrated key to help distinguish

  - Different quality of grassland

  - Habitats rich in animal life

  - Likely archaeological sites and artefacts

- A digital camera so that surveyors can record key information for experts

- A standardised report sheet for recording field data and photographs

Taken together, the tools provide a simple kit for site selection that should both help save time for the company and avoid causing unnecessary damage to the cultural or biological heritage of Uruguay.

The planning tools therefore draw on both existing information and information to be collected by field staff, summarised in Tables ## and ## below:

Table ##: Indicators for site selection drawing on existing information

| Thematic area | Indicator | Source | Recommendation type |
| --- | --- | --- | --- |
| Geology | Aquifers recharge areas | Hydrogeology map (SIG) | Planning |
| Soils | Soil map | Map CONEAT | Site selection |
| Vegetation | Site agricultural history | Aerial photography | Site selection |
| Archaeology | Archaeological sites | Archaeological map (SIG) | Planning |

The four layers can be integrated into a suitability map to help choose the best general areas to focus on for site selection. Information on aquifer recharge areas is not precise. It will indicate those areas that are relatively trouble free and those where recharge may be an issue – in the latter case further research will be needed.

Table ##: Indicators for site selection drawing on field information

| Thematic area | Indicator | Source | Recommendation type |
| --- | --- | --- | --- |
| Geology | Limiting features: hard rocks, basalts, calcium carbonate | Field observation (photo) | Site selection |
| Soils | Soil texture and depth | Field simple | Site selection |
| Vegetation | Identification of Virgin Campos and Palms | Field observation (photo) | Planning |
| Fauna | Campos birds | Field observation (counting) | Site selection |
| Archaeology | Archaeological sites | Field information (question photo) | Planning |

The field visit should be supported by a field sheet with photos, identification notes and a standardised recording sheet for record information. It is suggested that this information is added to the reporting sheet already used by Stora Enso, which is currently being revised.

The list above is a minimum data set. Stora Enso staff indicated that they wanted to include several other items of information, such as presence of woodland and wetland, because the proportions of these would also help to determine whether or not the site is suitable for purchase and would also need to be included in all planning. Table ## below gives a slightly expanded list of indicators, with more details of what each would entail.

Table ##: Details of proposed indicators for site selection and planning

| Indicator | Methodology used | Notes |
| --- | --- | --- |
| Soils, geology and hydrology |  |  |
| Information presented in the form of a suitability map to help identify areas worth further investigation and areas to avoid |  |  |
| Soils |  |  |
| Suitability for trees | CONEAT national map of soils | Publicly available information |
| Texture | Standard typology including e.g. clay content |  |
| Depth (for roots) | Depth of bedrock |  |
| Geology |  |  |
| Surface rock | Aerial photographs (?) | Not suitable for planting |
| Basalt | Existing information | High calcium sub-optimal for trees |
| Potential mining areas | Existing information | Risk of compulsory purchase |
| Hydrology |  |  |
| Aquifers |  |  |
| - Free | Aerial photographs |  |
| - Confined and semi-confined | Need field studies |  |
| - Recharge areas | Data already available | Need to set a baseline |
| Biodiversity |  |  |
| Vegetation |  |  |
| Agricultural history | Aerial photography |  |
| Information below presented as illustrated keys |  |  |
| Quality of grassland |  |  |
| - Natural grassland | Presence of indicator species | No crops for at least 3 years |
| - Virgin grassland | Presence of indicator species | Never crops |
| - Former cropland | Presence of indicator species |  |
| Palms | Presence of either palm species |  |
| Woodland | Presence of native woodland |  |
| - River woodland | Presence of river woodland | Restoration could be useful |
| Wetlands | Presence of wetlands and flooded areas |  |
| Mobile dunes | Presence of dunes |  |
| Rocky outcrops | Presence of rock outcrops and cacti | Are rocky outcrops ever planted? |
| Fauna |  |  |
| Main identification as an illustrated key of birds with extra information on rare and some seasonal species |  |  |
| Birds |  |  |
| - Abundance | Qualitative information about numbers |  |
| - Habitat quality | Presence of indicator species |  |
| Rare species | Presence recorded |  |
| Seasonal species | Information on good seasonal indicators |  |
| Archaeology |  |  |
| Known sites | Using existing GIS data |  |
| Information below presented as an illustrated key |  |  |
| Key relics | Important relic species that may be found |  |
| Key sites | Rock outcrops etc |  |

The workshop also identified a series of potential indicators for planning. These were less thoroughly developed, because the process of planning and implementing an assessment methodology is planned as a later stage in the project, but the initial list will be useful both in planning a participatory assessment but also in identifying indicators for the project. These are presented below, first in summary conceptual form and then in more detail, in Tables ## and ##.

Table ##: Indicators for monitoring

| Thematic area | Indicator | Method | Intensity |
| --- | --- | --- | --- |
| Geology | Potentiometric levels | Observation wheels | Annual |
|  | Chemical parameters | Observation wheels | Annual |
| Soils | Physical parameters | Field sample | Adaptative |
|  | Chemical parameters | Field sample | Adaptative |
|  | Biological parameters | Field sample | Adaptative |
| Vegetation | Invasive species | Mapping distribution | All land holdings |
|  | Native habitats | Conservation status | High conservation value areas |
| Fauna | Ecosystem integrity (benthonic, coleopteran, amphibian, reptile, birds, mammals) | Conservation status | High conservation value areas |
| Anthropology | Human assets (health, education) | Landscape Outcome Assessment Methodology (LOAM) | Sample of representative villages |
|  | Social assets (services, jobs) |  |  |
|  | Economic (economy status) |  |  |
|  | Political (local policies, stakeholders) |  |  |
|  | Environmental assets (environmental problems) |  |  |

Table ##: Detailed proposals for monitoring and evaluation indicators

| Indicator | Method used | Frequency of collection | Notes |
| --- | --- | --- | --- |
| Soils |  |  |  |
| Visual monitoring as part of standard field surveys |  |  |  |
| Physical | Recording signs of a hard layer of topsoil developing | Annually in erosion-prone areas, less frequently elsewhere |  |
| Chemical | Recording loss of colour |  | Indicates loss of organic matter |
| Biological | Recording signs of soil life |  |  |
| Sampling and analysis (usually only needed if visual survey suggests that there are problems) |  |  |  |
| Physical | Structural analysis | Annually in erosion-prone areas, less frequently elsewhere |  |
| Chemical | Nutrient availability and cation exchange capacity |  | For example borium |
| Biological | Analysis of soil organisms in sample |  |  |
| Hydrology |  |  |  |
| Rate of aquifer recharge | Comparing (for both open and fractured aquifers) - Rate of recharge over time - Recharge in planted and unplanted areas | Annually |  |
| Water quality | Use of surveillance wells to monitor changes in water quality |  |  |
| River quality | Nothing suggested |  |  |
| Lake / reservoir quality | Nothing suggested |  |  |
| Vegetation |  |  |  |
| Invasive species | Monitoring of all sites for the presence of invasive species |  | Need an identification key for survey teams (?) and to establish a baseline |
| High conservation value (HCV) areas | Regular monitoring of changes to flora and fauna in representative sites |  | Need to agree number of HCV sites and details of monitorin |
| Fauna |  |  |  |
| Changes in HCV areas |  | Annual monitoring plan |  |
| - Aquatic communities | Monitoring of benthnic communities and amphibians |  |  |
| - Terrestrial communities | Monitoring of beetles, reptiles, birds, mammals | Detailed proposals exist |  |
| Archaeology |  |  |  |
| Dry-stone structures etc | Monitoring of condition of important cultural sites |  | Most covered by law |
| Social conditions |  |  |  |
| Human | Health, education, training, internet access, migration |  |  |
| Social | Cultural dynamics, stakeholder opinions |  | Cultural indicators to be identified |
| Financial | Employment (existing and forestry-related) |  |  |
| Physical | Housing quality, shops, etc (existing and new) |  |  |
| Natural | Impacts on water, fire, erosion |  | Stakeholder opinions and data |

Forest suitability map

Recommendations for site and landscape planning

Landscape outcome assessment

It is proposed that Stora Enso use the Landscape Outcome Assessment Methodology (LOAM) to track in particular stakeholder opnions regarding the project. The following section explains brefly what this might entail.

There is an increasing trend of focusing efforts on developing programmes at a multifunctional landscape level or scale. In developing initiatives that take into account the landscape scale context, one of the biggest challenges is measuring and monitoring the outcomes in terms of key values or functions of the landscape as a whole. Conventional methods of assessing and monitoring project or programme impacts and results do not generally translate well at a broader landscape scale.

In response to the challenges identified above, the Forests for Life Programme of WWF has been piloting and testing an approach to identifying and putting in place a process to track a set of key landscape level outcomes. Currently it is known as the LOAM – Landscape Outcome Assessment Methodology.

Put simply the approach aims (through a participatory, representative stakeholder process) at the identification and application of a small representative set of locally appropriate indicators grouped under a framework of common key landscape values or assets. From this basis a scoring system is then developed which can be used to measure, monitor and communicate to the range of stakeholders, the nature and extent to which the landscape is changing over time with respect to a small number of commonly identified and agreed conservation and livelihood outcomes.

How and where should the LOAM be applied?

Experience to date is showing that the perceptions and/or expectations and needs of what the LOAM can deliver are very different depending on the situation in each landscape. It is important that these are clarified at the outset in order that the LOAM is applied in the way it is intended, rather than as a proxy for something else. This technique – LOAM – is about understanding landscape change and negotiating with other stakeholders futures for the landscape that will be good for everyone. It is NOT about monitoring and evaluation of projects and tracking the direct impacts of project interventions.

LOAM is about understanding how projects or programmes can have an impact within the context of larger landscapes. This means that we have to understand what is important for the people in the landscape and we have to know how the landscape is changing and why. That way we can adapt project activities so that, along with all the other things that influence the future of that landscape, they tend to cause it to develop in a positive way.

Figure 1 shows an interpretation of where the LOAM approach fits in a spectrum of scale from a site, through landscape up to the ecoregion or regional level. It also illustrates its recommended role as between project level monitoring at the smaller end of the spectrum and full-blown inter-sectoral scenario building at the region or ecoregional level, although the boundaries are not absolutely distinct and the approaches may overlap slightly with each other in some cases. The diagram then goes on to show some of the other key features of the LOAM, with respect to the other features.

Figure ##: Reltive positioning of LOAM with respect to scale and some key features

How to implement the LOAM process

A suggested series of key steps is outlined below, although this should not be seen as definitive, and can be adapted to suit circumstances in a particular landscape.

Define the landscape(s)

Identify a multi-stakeholder group covering all parties with interests in the landscape

Explore scenarios for the landscape

Facilitate a discussion of indicators of landscape-level outcomes with the stakeholders

Facilitate regular assessment and debate

Adaptation as a result of lessons learned

Compile a first baseline using the initial set of agreed indicators

Convene a meeting/workshop of this group

Note that there are no arrows between the boxes: the order given is one possibility but in practice many stages may take place simultaneously, or at different times in different landscapes

Each of these stages is described in greater detal below.

Define the landscape: the landscape can be defined in terms of a geographic area or something like the “area of distribution of certain habitats like Stony Campos”. Normally it will be geographic.

### Identify a multi-stakeholder group covering all parties with interests in the landscape: this group should include conservation partners, social development NGOs, Government representatives from key sectors, private sector, local community groups and individualities

Convene a meeting or workshop of the multi-stakeholder group: Try to gather as many representatives from the group above as possible to an initial workshop. Experience has shown that it is rarely possible to get all interests equally representatives, but what is important is to initiate a process, and more people may join later

Explore scenarios in the landscape: This should be a facilitated multi-stakeholder process. A good way to start is to ask participants for them the possible future of the landscape. Then discuss the “drivers of change” or external factors that will influence the future of the landscape. For example, what will tend to take it to the case scenario

### Facilitate a discussion of indicators of landscape-level outcomes with the stakeholders: Encourage a discussion of what would be indicators of “improvements” in the landscape. Ask stakeholders to write down their own list of indicators of progress in the landscape. This can lead to a useful discussion of “what constitutes success”. Once there is general agreement on ideas for indicators proceed to a more formal discussion guided by the points below:

1. List the indicators:  Group the indicators under five categories based upon the Capital Assets, or Sustainable Rural Livelihoods (DfID) Framework. This was adapted to give 5 categories of indicators that seem to apply in most of the landscapes:

- Human assets

- Social assets

- Physical assets

- Natural assets

- Environmental assets

The reasons for using the assets framework are that it encompasses all of the features of a landscape that are likely to be of concern of local stakeholders.

2. Indicators are then defined in a score out of 5 – the so-called Likert scale - which moves across a scale from a value of 1 at the lowest end through increasing levels of “performance” to 5 at the top end: e.g. if the indicator is “Frequency of forest fires”, examples of scoring using the Likert scale might be:

- Biannual forest fire;

- One forest fire per annum;

- One fire per annum but stops at forest edge;

- <1 fire per annum, not penetrating forest;

- No fire

- The scores can be combined in several visual ways and presented graphically.

### Compile a first baseline using the initial set of agreed indicators: Whilst an exceptionally well facilitated and productive workshop may achieve good consensus on a first indicator set and Likert scoring ranges, it is more likely that the framework produced will only be partially complete. In addition the latest data for a specific indicator may not be immediately available, or in the most extreme case need to be collected. Therefore it is best to plan for the time of a technically skilled person, or ideally a small team, for post-workshop follow-up to complete the indicator set and Likert scoring scales, gather and/or collect the required data and compile the first baseline assessment. This process in itself will provide a first feasibility test of the proposed indicator set.

### Facilitate regular assessment and debate: After a suitable interval – most of the cases yearly – reconvene the group and see if the scores have changed: Two things can happen when you reconvene. First people will challenge the indicators, they will argue that other indicators would have been better or they can challenge the scoring matrix. Debate this – this debate is part of the process. Second you may find that the group has difficulty on agreeing on changed scores. This does not matter; again it is part of the sharing of understanding. What matters finally is that there is a structured debate about whether progress has been made or not. At these periodic meetings people will argue about whether the project is really helping to improve the landscape. This is the most valuable part of the entire process. It is the key to adaptive management. A major outcome of these periodic meetings should be a revisiting of the work plan – are we getting maximum effectiveness in terms of improving the landscape? If not, what should we be doing differently?

## Commonly used indicators

Similar indicators tend to emerge from these processes wherever LOAM have been applied. However it is strongly advised against going into a multi-stakeholder meeting with a pre-cooked list of indicators. The process of building shared visions requires that stakeholders can all contribute and feel ownership of the process.

However it does help if the facilitator has some experience of the sort of indicators that work and those that are difficult to reach agreement on or are difficult to measure objectively. Some examples of indicators that often come up are:

Natural assets

- Forest extent – or rate of change of forest

- Numbers of a key species

- Integrity of critical habitats – e.g. wetlands, grasslands, forests

Environmental assets

- Frequency of forest fires

- Quality of water

- Erosion

Human assets

- Availability of health care

- Availability of education

- Skills and education levels, and opportunities

Social capital

- Women social integration

- Employment

- Services

Physical assets

- Road access

- Quality of housing

- Local industries

The number of potential indicators is endless. The value of this is to determine what is really important to the local stakeholders and what they would like to see improve in the short term.

## Results Presentation

The results of these indicator measures can be presented in many ways depending upon the objectives and the audience. One way that we have found useful and easy to understand is in the form of a Radar diagram showing the scores of each of the asset categories separately.

High conservation value concept

High conservation value area concept

Luis’ initial draft to be expanded

The concept of High Conservation Values Forest (HCVF) was defined by the Forest Stewardship Council (FSC) for use in forest certification. Now it is increasingly being used in other fields for mapping, nature resource conservation, purchasing policies and policies of government agencies. WWF is also increasingly usng the HCVF concept in its conservation programmes, and there is a trend to use the HCV concept both wthin and separate from certified forests

WWF uses the terminology of High Conservation Value Areas for representing the concept of High Conservation Value Forests from FSC, in order to better include non forest Habitats. Uruguay is a country where non forest habitats are dominant.

An HCVA is an area containing significant concentrations of biodiversity values of viable ecosystems or provide basic services or needs.

These tools provde a practical method to apply the HCVA concept wthin a Forest Management Unit, using a three stage approach:

- Charactersation of the conservation values present

- Effective conservaton of these values

- Monitoring the conservaton status of these values

The HCVA methodology requires users to identify HCVs that cover a wide range of ecological, environmental, and social issues and requires an understanding of regional biodiversity issues, animal and plant ranges and behaviour, water and soil resources, ecosystem health, anthropology, and local economy.  It is very likely that users will not have all of the expertise necessary to evaluate these issues alone.  The process is highly reliant upon the input of local and national expert organizations and individuals. External experts and organizations will also be able to provide input on the status of forest types and of rare, threatened and endangered species, and help design management strategies to ensure the maintenance or enhancement of high conservation values

High conservation value assessment

Characterisaton

HCVA is an area containing significant concentrations of biodiversity values, which encompasses species, ecosystems or basic services or needs.

Consultation

Indicator: National expert organizations and individuals provided input on the list of potential values and help design strategies to ensure the maintenance or enhancement of high conservation values

Verifier: List of consulted organisations and individuals

Identificaton

Indicator: Verify if the potential Conservation Values for the Forest Management Unit are identified

Verifier: List of Potential Values

Description

Indicator: Verify if the potential Conservation Values for the Forest Management Unit are described

Verifier: This description must identify for each Conservation Value what are the attributes that characterise the Value and that aloud his effective identification

Mapping

Indicator: Verify if the Conservation Values in presence are identified in the field and mapped through GIS, with the attributes of the Value available in the GIS data base

Verifier: percentage ofHCV in the management area

Conservation status

Indicator: Verify the Conservation Status classification of the Values in presence

Verifier: % Degraded (neither natural species nor ecological processes in place) (D); % Evolving (natural species present but ecological processes being restored) (E), and % Favourable (natural species and habitat present in a mature phase (F)

Conservation status of a natural habitat means the result of influences acting on a natural habitat and its typical species that may affect its long-term natural distribution, structure and functions as well as the long-term survival of its typical species within the territory.

Conservation models

Indicator: Verify if there are Models defined for the different Conservation Status (CS) of the Values in presence

Verifier:

- Conservation status degraded: conservation model – restoration, removal of forest residuals and invasive species. Artificial regeneration

- Conservation status evolving: conservation model – management, removal of forest residuals and invasive species. Facilitation of natural regeneration

- Conservation status favourable: conservation model – protection, prevention against invasive species, pests, diseases and fire

Field management

Indicator: Verify in the field if the Conservation Models are being implemented

Verifier: percentage of HCV being effectively conserved

Monitoring

Monitoring the conservation status

Indicator– Verify if the conservation status of the HCVA is being monitored

Verifier: Variation in the conservation status  of the area (% Degraded; % Evolutive; % Favourable)

Applying the HCVA concept to Stora Enso plantations in Uruguay

Characterisation: first the HCVA needs to be characterised – involving consultation, identification and description. The first step requires contact with relevant experts and stakeholders and/or specific research and consultation. Stora Enso has conducted an expert’s consultation process in terms of biodiversity and cultural values for the landscape where their operations will be running. This has provided the first list and characterisation of conservation values that are potentially present in Stora Enso management units.

Cartography: The next step in the characterisation process is a simple methodology to see if HCVs are likely to occur or not. This has already been addressed through development of site selection indicators (see page ##).  This acts as a coarse filter, rapidly excluding all sites that definitely do not contain HCVs, and identifying sites that potentially contain specific HCVs. The preliminary assessment should be straightforward enough to be undertaken by people without special knowledge of biological or social sciences (e.g. forest managers). The preliminary assessment is usually in the form of a “yes or no” question and asks about the presence of certain values.  For example, a preliminary assessment may ask users to identify if Virgen Campos and respective birds communities appears in the plantation site. If the answer to the preliminary question is yes, then a more complete or full assessment of the value will be needed, starting by mapping in a digital format the value distribution. Assessments often utilize maps and other information that can be easily accessed by forest managers

Advice on Environmental Management Systems

One of the challenges in completing the impact analysis was that many of the decisions about management have yet to be taken and

Existing best management practice

Concerns about the potential social and environmental impacts of plantations have led to the development of a series of guidelines and codes of practice relating to location, choice of species, management and harvesting of plantations. Some of these have been developed for use throughout the world or in major forest biomes, while others have been developed expressly for local needs at national or provincial level, such as those developed in South Africa. Some forest certification systems also have specific requirements for plantations. In addition, there are a far wider range of guidelines relating to general forest management and most of these contain advice that can be applied to aspects of plantation development; for example virtually every state in the USA and Australia has its own agreed forestry guidelines.

Guidelines are produced by industry groups, NGOs, international organisations, research bodies and non-governmental organisations, sometimes in partnership. Most are in the form of best management practices while others are codes of practice or indicators for certification. With the exception of voluntary certification schemes, where many controls will be mandatory for a certified operation, most “best management practices” are a mixture of legal requirements under national or regional laws and suggestions, which are voluntary. It is sometimes difficult to distinguish between the two. A wide range of existing guidelines have been examined and are compared and assessed in the following paper. Standards are variable and many of the guides that exist are of limited usefulness, in that it is hard to see how working foresters or forest managers could use them to make substantial changes or improvements to practice. For example, many have the following restrictions:

- Vague general demands: phrases such as “do not damage wildlife” confront managers with a potentially huge management challenge without provide useful information about how to achieve this or advice about prioritisation: general, idealised demands are likely to be ignored or misunderstood.

- Undefined exemptions: many codes include a clause such as “apart from in exceptional circumstances…” or “if reasonably possible…” without giving any advice about what might constitute an exceptional circumstance. Whilst there will often be circumstances in which a particular best management practice is not really necessary, by allowing exceptions without giving any guidance, the whole code is weakened.

- Incomplete coverage: guidelines tend to be very variable in detail, for example typically giving a lot of quite specific and detailed information about buffer zones around streams, bridge-building and road design and construction, but little information about chemical use, wildlife management or social impacts. Most guidelines had limited information about biodiversity and none at all about potential social impacts.

- Poor layout: guidelines are often extremely long, poorly laid out, with no index and with no summary of key points, making it difficult to find specific information and off-putting for busy managers.

International guidelines

A number of attempts have been made to outline best practice for plantations at an international scale. Such approaches have clear limitations, because of the widely varying social and environmental conditions that these must encompass, and also the different aims of plantations, and most have limited themselves to establishing some general principles; the majority have also focused on the wet tropics.

The Food and Agriculture Organization of the United Nations provided a series of guides on key aspects of plantation management starting as early as the 1970s, many of which are still useful, covering such topics as: harvesting (FAO, 1976); road building (FAO 1977) and maintenance (Sedlak 1988); and a more general guide to forestry practice that includes many issues relevant to plantations (Dykstra and Heinrich 1995). At a more general level an early review by IUCN outlined environmental concerns and included some preliminary suggestions for best practice (Sawyer 1994). Following that, at an institutional level there have been four general guidelines:

- Shell and WWF

- The International Tropical Timber Organisation (ITTO)

- The Center for International Forestry Research (CIFOR)

- Food and Agricultural Organisation (FAO) (draft) planted forest code

In the early 1990s, Shell and WWF collaborated on a detailed plantation review, which resulted in a 12 volume set of papers including one the outlined 20 principles and some accompanying guidelines for good management (Poore 1993). Both principles and guidelines are general and do not provide many concrete best management practices that plantation managers can follow. The guidelines were clearly aimed at the tropics; they focused more on general principles for choosing sites than on details of management. The publication contains a useful contents list for a feasibility study and tables of positive and negative impacts of plantations with ways of mitigation

One of the most comprehensive global attempts to define principles for plantations is still that of the International Tropical Timber Organisation (ITTO 1992). The majority of its 66 principles and associated recommendations are applicable in any conditions. However, they were drawn up for a far less intensive approach than the “fast wood” plantations; for example they have a bias towards replanting with native species and with species mixes. They were developed primarily with a view to protecting existing tropical rainforest and ensuring that local communities did not suffer adversely from plantation development through direct expulsion from traditional lands, both not issues in Uruguay. Notwithstanding, the recommended actions do contain clear proposals that can usefully be incorporated into contemporary best management practices.

The Center for International Forestry Research has produced criteria and indicators for sustainable development of industrial tree plantations in the tropics (Poulsen and Applegate 2001) and a linked code of practice (Applegate and Raymond 2001), which is currently the most detailed and specific available on a global scale. Table ## outlines the major areas covered by the code, some contain detailed tables and instructions, others more general advice.

Table ##: CIFOR Code of Practice for plantations in the tropics

| Area | Key issues addressed |
| --- | --- |
| Planning | Outline of planning levels |
|  | Summary of main elements that will be identified in land use plans |
|  | Requirements for effective planning (biological, economic, social) |
|  | Information required by planners at stand and site level |
| Landscape-level planning | Biophysical data needs |
|  | Cadastral data |
|  | Socio-economic data (tenure, stakeholder involvement, economics etc) |
|  | Environment-biodiversity data |
|  | Exclusion zones – 7 identified and stream typing and buffers explained |
| Stand-level planning | Buffers, roads, watercourses, contours, firebreaks, landings, skid trails |
| Roading and drainage | Road classification |
|  | Road design including location and drainage, grade, width and corners |
|  | Road and track construction – timing, survey, drainage, revegetation |
| Road watercourse crossings | Types and uses |
|  | Construction of bridges, culverts and pipes, fords and silt traps |
| Quarries | Sources of quarry material |
|  | Quarry management and rehabilitation |
| Plantation development at a stand level | Planning and pre-harvest activity |
|  | Log landings (location, construction, operations and rehabilitation) |
|  | Planning skid trails including construction and stream gulleys |
|  | Harvesting operations including tree felling |
|  | Log cross-cutting and de-branching plus details of skidding, storage |
|  | Weather limitations on harvesting |
|  | Monitoring and evaluation of development activity including checklist |
| Log pond and wharf | Location and timing of construction |
|  | Design of log pond, wharf and loading areas |
|  | Maintenance and decommissioning |
| Plantation establishment | Site preparation including detailed table of options with assessment |
|  | Site preparation on peat soils |
|  | Planting |
|  | Herbicide and pesticide application |
|  | Fertilizer application |
|  | Alternatives to agrochemicals |
| Employee conditions | Working conditions |
|  | Personnel safety requirements including table of protective clothing |
|  | Equipment safety requirements |
| Fire management | Education and preparation including checklist for high risk periods |
|  | Fire suppression |
| Harvesting equipment maintenance | Moving equipment |
|  | Workshop facilities |
|  | Toxic wastes and refuse disposal |
|  | Main fuel and oil storage |
|  | Field servicing and maintenance |
| Camp hygiene | Design, water supply, waste disposal and drainage |
| Training systems | Outline of minimum competency standards |

Some of the detailed guidelines have been conflated to save space and all have been summarised

Still in draft form, the Food and Agricultural Organisation code is currently the most up to date, consisting of 12 principles and five cross-cutting themes. However it should be noted that the document is currently incomplete, lacking references and also being a code rather than guidance it tends to be conceptual: Table ## below outlines the principles.

Table ##: FAO Draft Code of Practice for Planted Forests

| Number | Principle |
| --- | --- |
| 1. Good governance – ensuring stable conditions to encourage long term investments, sustainable land-use practices and socio-economic stability | Transparency |
|  | Precautionary approach |
|  | Good policies and legal framework |
|  | Monitoring of compliance to reduce illegal use |
|  | Recognition of legal tenure |
|  | Equitable distribution of benefits |
|  | Workers rights, including decent wages and rights to organise |
| 2. Integrated decision-making and a multi-stakeholder approach | Integrated approaches within the landscape |
|  | Encouragement of participation in free, prior-informed consent |
|  | Conflict resolution mechanisms to resolve disagreements |
| 3. Enhanced organisational capacity | Decentralisation |
|  | Investment and long-term financing |
|  | Needs and aspiration of stakeholders |
|  | Motivated, well-trained workforce |
|  | Strengthening of research capacity and knowledge-sharing |
|  | Use of science, traditional knowledge and experience |
|  | Support for both large- and small-scale investors |
| 4. Recognition of value of goods and services | Balance economic returns with needs of sustainable development |
|  | Employ full valuation techniques in assessment |
|  | Use full valuation for justification with investors and governments |
| 5. Promotion of investment – creating enabling conditions | Create a stable and clear investment policy – revised as needed |
|  | Encourage direct and indirect incentives |
|  | Avoid perverse incentives and economic distortions |
|  | Promote equity between competing land uses through policies |
| 6. Recognition of the role of the market | Transparent access to market information |
|  | Market intelligence information on current markets and trends |
|  | Policies, regulations and guidelines for stable investment |
|  | Recognise carbon markets for productive or protective plantations |
|  | Recognise that the market may not reflect all a forest’s values |
| 7. Recognition of social and cultural values | Recognition of rights, values and tenure of indigenous peoples |
|  | Strengthening capacity of indigenous / local communities to benefit |
|  | Recognition of the diverse contribution of smallholder investors |
|  | Provision of employment, training and health and safety equipment |
|  | Collaboration in strengthening education, health and social services |
| 8. Maintenance of social and cultural services | Establish socio-economic M&E systems and baselines |
|  | Establish clear conflict resolution mechanisms |
|  | Provide safe and healthy working environment and conditions |
|  | Protect sites of archaeological, cultural and spiritual significance |
|  | Recognise community ancestral rights wherever possible |
|  | Discourage community displacement without prior informed consent |
| 9. Maintenance and conservation of environmental services | Policy and legal frameworks to preserve / restore env’tal services |
|  | Maintain and conserve integrated ecosystem functions |
|  | Adopt integrated watershed and soil management approaches |
|  | Prepare environmental impact assessments to establish baselines |
|  | Protect forest crops from wind or other adverse weather conditions |
|  | Include carbon sequestration and carbon sinks in planning |
|  | Minimise environmental impacts through operation management |
|  | Recognise the positive impact plantations can have in restoration |
| 10. Conservation of biological diversity | Adapt management to maintain or conserve biodiversity |
|  | Protect habitat through stand and landscape-level measures |
|  | Prepare environmental impact assessments |
|  | Select appropriate silvicultural systems to protect biodiversity |
|  | Avoid illegal hunting, trapping, foraging and harvesting of plants |
|  | Select native species if they are equal to or better than exotics |
| 11. Maintain forest health and productivity | Promote reforestation, soil conservation etc after harvest |
|  | Reduce risk of invasive species through biosecurity measures |
|  | Adopt integrated pest management and biological when possible |
|  | Make efforts to reduce use of herbicides, pesticides, fungicides |
|  | Dispose of chemical materials, containers and waste carefully |
|  | Adopt sound policies, practices and monitoring in the use of GMOs |
|  | Select species, provenance etc considering objectives, risks and site |
|  | Reduce fires by prediction, prevention, monitoring & rapid response |
|  | Use prescribed fire to minimize risks but not to convert natural forest |
|  | Use fertilizers based specific nutrient requirements |
|  | Support education and training of staff, scientific research etc |
|  | Balance plantation success with reduction of environmental risks |
| 12. A landscape approach | Retain riparian reserves on permanent water courses |
|  | Reduce negative visual impacts of harvesting and forest operations |
|  | Designate reserves within which plantations will be restricted |
|  | Designate buffer zones adjoining communities |
|  | Designate road, stream crossings etc to fit the landscape |
|  | Monitor upstream and downstream water quality and quantity |
|  | Educate local communities and public by outreach programmes |
|  | Design plantations to provide corridors between natural forest areas |
|  | Recognise the continuum of protected areas to plantations |

The cross cutting themes provide discussion about some key issues, including:

- Institutional roles

- Strategic and economic planning

- Stakeholder relations

- Learning research

- Operational planning and maintenance

National or regional guidelines

A far larger body of literature can be found summarising best practice in a country or a state – our survey below is wide-ranging but by no means comprehensive. The large majority of these guidelines do not distinguish plantations from natural forests, although some separate out planted forests in general. There is inevitably a considerable amount of repetition and, as mentioned above, a large variation in quality. We reviewed over 50 separate guidelines and have extracted what we believe to be the best of these that might help Stora Enso to draw up its EMS and complement the existing forestry code in Uruguay.

Appendix ##: consultants

The majority of the consultants were from Uruguay and were emploed on a fairly hsort term basis to collect information and take part in the workshops identifying indicators. A small team of global consultants were also used to collect information on specific topics or because they had particular expertise. The following summarises information on all those involved in the assessment [still incomplete]

Physical geography and geology

Jorge Montaño Xavier (geology)

Mauricio Montaño (geology)

Sergio Gagliardi (geology)

Ximena Lacués (geology)

Fauna

Susana González (mammals)

Mario Clara (birds)

Enrique Morelli (insects)

Patricia González (insects)

Gabriela Bentancur (insects)

Flora

Pablo Boggiano (grasslands)

Carlos Brussa (wetlands)

Iván Grela (woodland)

Historical heritage

Arturo Toscano

Andrés Florines

Prof. Jorge Femeninas

Social issues

Leticia Cannella (anthropologist)

Enrique Gallicchio (socologist)

Viviana Martínez (socologist)

Forest management

Luis Neves Silva (forester, based in Portugal)

Stephanie Mansourian-Stephenson (based in Switzerland)

Jeffrey Sayer (monitoring and evaluation)

Coordinator

Nigel Dudley

Support in Stora Enso

Kaisa Tanra-Maini

Horacio Giordano

Andrea Storace

Appendix ##: Birds of the plantation landscape

The main resident and migratory species are outlined in Tables ## and ## below.

Table ##: Main resident species in the plantation landscape

| Species | Common name | Status |
| --- | --- | --- |
| Rhea Americana | Greater Rhea | Co |
| Rynchotus rufescens | Red-winged tinamou | Co |
| Nothura maculosa | Spotted tinamou | Co |
| Podylimbus podiceps | Pied-billed Grebe | Co |
| Rollandia Rolland | White-tufted Grebe | Co |
| Podiceps major | Great Grebe | Co |
| Phalacrocorax brasilianus | Neotropic Cormorant | Co |
| Anhinga anhinga | Anhinga | Pc |
| Botaurus pinnatus | Pinnated Bittern | Pc |
| Ixobrychus involucris | Stripe-backed Bittern | Pc |
| Tigrisoma lineatum | Rufescent Tiger-heron | Pc |
| Nycticorax nycticorax | Black-crowed Night-heron | Co |
| Ardeola ibis | Cattle Egret | Co |
| Syrigma sibilatrix | Whistling Heron | Co |
| Butorides striatus | Striated Heron | Co |
| Egretta thula | Snowy Egret | Co |
| Casmerodius albus | Great Egret | Co |
| Ardea cocoi | White-necked Heron | Co |
| Ciconia maguari | Maguari Stork | Co |
| Harpiprion caerulescens | Plumbeous Ibis | Co |
| Theresticus caudatus | Buff-necked Ibis | Pc |
| Phimosus infuscatus | Bare-faced Ibis | Co |
| Plegadis chihi | White-faced Ibis | Co |
| Ajaia ajaja | Roseate Spoonbill | Co |
| Chauna torquata | Southern Screamer | Co |
| Dendrocygna bicolour | Fulvous Tree-duck | Co |
| Dendrocygna viduata | White-faced Tree-duck | Co |
| Cygnus melancoryphus | Black-necked Swan | Co |
| Coscoroba coscoroba | Coscoroba Swan | Pc |
| Cairina moschata | Muscovy Duck | Ra |
| Amazonetta brasiliensis | Brazilian Duck | Co |
| Anas flavirostris | Speckled Teal | Co |
| Anas georgica | Brown Pintail | Co |
| Anas sibilatrix | Southern Wigeon | Pc |
| Anas versicolor | Silver Teal | Co |
| Cathartes aura | Turkey vulture | Co |
| Cathartes burrovianus | Lesser Yellow-headed Vulture | Pc |
| Coragyps atratus | Black Vulture | Pc |
| Elanus leucurus | White-tailed Kite | Co |
| Geranospiza caerulescens | Crane Hawk | Ra |
| Circus buffoni | Long-winged Harriere | Co |
| Circus cinereus | Cinereous Harrier | Ra |
| Accipiter bicolour | Bicoloured Hawk | Ra |
| Accipiter striatus | Sharp-shinned Hawk | Pc |
| Buteogallus urubitinga | Great Black Hawk | Ra |
| Heterospizas meridionalis | Savannah Hawk | Pc |
| Geranoaetus melanoleucus | Black-chested Buzzard-eagle | Pc |
| Parabuteo unicinctus | Bay-winged Hawk | Pc |
| Buteo magnirostris | Roadside Hawk | Co |
| Polyborus plancus | Crested Caracara | Co |
| Milvago chimachima | Yellow-headed Caracara | Pc |
| Milvago chimango | Chimango Caracara | Co |
| Falco femoralis | Aplomado Falcon | Pc |
| Falco sparverius | American Kestrel | Co |
| Penelope obscura | Dusky-legged Guan | Co |
| Aramus guaruana | Limpkin | Co |
| Ortygonax sanguinolentus | Plumbeous Rail | Co |
| Pardirallus maculates | Spotted Rail | Ra |
| Aramides cajanea | Grey-necked Wood-Rail | Co |
| Aramides ypecaha | Giant wood-Rail | Co |
| Laterallus leucopyrrhus | Red-and-White Crake | Pc |
| Laterallus melanophaius | Rufous-sided Crake | Co |
| Coturnicops notata | Speckled Crake | Ra |
| Porphyriops melanops | Spot-flanked Gallinule | Co |
| Gallinula chloropus | Common Gallinule | Co |
| Fulica armillata | Red-gartered Coot | Co |
| Fulica rufifrons | Red-fronted Coot | Co |
| Cariama cristata | Red-legged Seriema | Pc |
| Jacana jacana | Wattled Jacana | Co |
| Nycticryphes semicollaris | South American Painted-Snipe | Pc |
| Himantopus mexicanus | South American Stilt | Co |
| Vanellus chilensis | Southern Lapwing | Co |
| Charadrius collaris | Collared Plover | Co |
| Gallinago paraguaiae | Common Snipe | Co |
| Larus cirrocephalus | Grey-hooded Gull | Pc |
| Larus dominicanus | Kelp Gull | Co |
| Larus maculipennis | Brown-hooded Gull | Co |
| Phaetusa simplex | Large-billed Tern | Pc |
| Gelochlidon nilotica | Gull-billed Tern | Co |
| Sterna superciliaris | Yellow-billed Tern | Co |
| Columba maculosa | Spot-winged Pigeon | Co |
| Columba picazuro | Picazuro Pigeon | Co |
| Zenaida auriculata | Eared Dove | Co |
| Columbina picui | Picui Ground-Dove | Co |
| Leptotilia rufaxilla | Gray-fronted Dove | Pc |
| Leptotilia verreauxi | White-tipped Dove | Co |
| Aratinga acuticaudata | Blue-crowned Parakeet | Ra |
| Aratinga leucophthalmus | White-eyed Parakeet | Pc |
| Myiopsitta monachus | Monk Parakeet | Co |
| Piaya cayana | Squirrel Cuckoo | Pc |
| Crotophaga ani | Smooth-billed Ani | Pc |
| Guira guira | Guira Cuckoo | Co |
| Tyto alba | Barn Owl | Pc |
| Otus choliba | Tropical Screech-Owl | Co |
| Bubo virginianus | Great Horned Owl | Pc |
| Glaucidium brasilianum | Ferruginous Pygmy-Owl | Ra |
| Speotyto cumicularia | Burrowing Owl | Co |
| Pseudoscops calmator | Striped Owl | Pc |
| Asio flammeus | Short-eared Owl | Pc |
| Leucochloris albicollis | White-throated Hummingbird | Co |
| Ceryle torquata | Ringed Kingfisher | Co |
| Chloroceryle amazona | Amazon Kingfisher | Co |
| Chloroceryle Americana | Green Kingfisher | Co |
| Picumnus nebulosus | Mottled Piculet | Pc |
| Melanerpes candidus | White-fronted Woodpecker | Co |
| Picoides mixtus | Checkered Woodpecker | Pc |
| Veniliornis spilogaster | White-spotted Woodpecker | Co |
| Colaptes campestris | Field Flicker | Co |
| Colaptes melanochloros | Green-barred Woodpecker | Co |
| Drymornis bridgesii | Scimitar-billed Woodcreeper | Co |
| Lepidocolaptes angustirostris | Narrow-billed Woodcreeper | Co |
| Geositta cunicularia | Common Miner | Pc |
| Furnarius rufus | Rufous Hornero | Co |
| Limnornis curvirostris | Curve-billed Reedhaunter | Pc |
| Phleocryptes melanops | Wren-like Rushbird | Co |
| Lepatsthenura platensis | Tufted Tit-Spinetail | Pc |
| Schoeniophylax phryganophila | Chotoy Spinetail | Pc |
| Synallaxis frontalis | Sooty-fronted Spinetail | Co |
| Synallaxis spixi | Chicli Spinetail | Co |
| Certhyaxis cinnamomea | Yellow-throated Spinetail | Ra |
| Cranioleuca pyrrhophia | Stripe.crowned Spinetail | Co |
| Cranioleuca sulphurifera | Sulphur-beared Spinetail | Pc |
| Asthenes baeri | Short-billed Canastero | Pc |
| Asthenes hudsoni | Hudson's Canastero | Pc |
| Phacellodomus striaticollis | Freckle-breasted thornbird | Co |
| Anumbius annumbi | Firewood-gathered | Co |
| Pseudoseisura lophotes | Brown Cachalote | Pc |
| Syndactyla rufosuperciliata | Buff-browed Foliage-gleaner | Co |
| Lochmnias nematura | Sharp-tailed Streamcreeper | Pc |
| Thamnophilus caerulescens | Variable Antshrike | Co |
| Thamnophilus ruficapillus | Rufous-capped Antshrike | Co |
| Sublegatus modestus | Scrub Flycatcher | Pc |
| Suiriri suiriri | Siuriri flycatcher | Pc |
| Serpophaga nigricans | Sooty Tyrannulet | Co |
| Serpophaga subcristata | White-crested Tyrannulet | Co |
| Pseudocolopteryx flaviventris | Warbling Doradito | Pc |
| Phylloscartes ventralis | Mottled-cheeked Tyrannulet | Co |
| Xolmis cinerea | Gray Monjita | Co |
| Xolmis irupero | White Monjita | Co |
| Heteroxolmis dominicana | Black-and-white Monjita | Pc |
| Knipolegus cyanirostris | Blue-billed Balck Tyrant | Co |
| Knipolegus lophotes | Crested-black Tyrant | Pc |
| Hymenops perspicillatus | Spectacled Tyrant | Co |
| Satrapa icterophrys | Yellow-browed Tyrant | Co |
| Machetornis rixosus | Cattle Tyrant | Co |
| Pitangus sulphuratus | Great Kiskadee | Co |
| Anthus correndera | Correndera Pipit | Pc |
| Anthus furcatus | Short-billed Pipit | Co |
| Anthus hellmayri | Hellmayr's Pipit | Pc |
| Anthus lutescens | Yellowish Pipit | Pc |
| Troglodytes aedon | House Wren | Co |
| Mimus saturninus | Chalk-browed Mockingbird | Co |
| Turdus albicollis | White-necked Thrush | Pc |
| Turdus amaurochalinus | Creamy-bellied Thrush | Co |
| Turdus rufiventris | Rufous-bellied Thrush | Co |
| Polioptila dumicola | Masked Gnatcatcher | Co |
| Zonotrichia capensis | Rufous-collared Sparrow | Co |
| Ammodramus humeralis | Grassland Sparrow | Co |
| Donacospiza albifrons | Long-tailed Reed-finch | Co |
| Poospiza lateralis | Red-rumped Warbling-Finch | Co |
| Poospiza nigrorufa | Black-and-rufous Warbling-finch | Co |
| Sicalis flaveola | Saffron Finch | Co |
| Sicalis luteola | Grassland Yellow-Finch | Co |
| Embernagra platensis | Great Pampa-Finch | Co |
| Emberizoides ypirangus | Lesser Grass-Finch | ? |
| Gubernatrix cristata | Yellow Cardinal | Ra |
| Coryphospingus cucullatus | Red-crested Finch | Ra |
| Paroaria coronata | Red-crested Cardinal | Co |
| Volatinia jacarina | Blue-black Grassquit | Co |
| Saltator aurantiirostris | Golden-billed Saltator | Co |
| Saltator similes | Green-winged Saltator | Pc |
| Piranga flava | Hepatic Tanager | Co |
| Thraupis bonariensis | Ble-and-yellow Tanager | Co |
| Thraupis sayaca | Syaca Tanager | Co |
| Stephanonphorus diadematus | Diademed Tanager | Co |
| Pipraeidea melanota | Fawn-breasted Tanager | Pc |
| Tangara preciosa | Chestnut-backed Tanager | Co |
| Parula pitiayumi | Tropical Parula | Co |
| Basileuterus culicivorus | Golden-crowned Warbler | Co |
| Basileuterus leucoblepharus | White-browed Warbler | Co |
| Cyclarhis gujanensis | Rufous-browed Peppershrike | Co |
| Icterus cayanensis | Epaulet Oriole | Co |
| Cacius chrysopterus | Golden-winged Cacique | Co |
| Cacius solitarius | Solitary Black Cacique | Co |
| Agelaius ruficapillus | Chestnut-capped Blackbird | Co |
| Sturnella defilippii | Lesser Red-breasted Meadowlark | Ra |
| Sturnella superciliaris | White-browed Blackbird | Co |
| Pseudoleistes guirahuro | Yellow-rumped Marshbird | Pc |
| Pseudoleistes virescens | Brown-and-yellow Marshbird | Co |
| Amblyramphus holosericeus | Scarlet-headed Blackbird | Pc |
| Gnorimopsar chopi | Chopi Blackbird | Pc |
| Molothrus badius | Bay-winged Cowbird | Co |
| Molothrus bonariensis | Shiny Cowbird | Co |
| Molothrus rufoaxillaris | Screaming Cowbird | Co |
| Carduelis magellanica | Hooded Siskin | Co |
| Cyanocorax chrysops | Purplish Jay | Co |

Table ##: Main migrant species in the plantation landscape

| Species | Common name | Summer | Winter | Status |
| --- | --- | --- | --- | --- |
| Mycteria Americana | Wood-Stork |  |  | Co |
| Netta peposaca | Rosy-billed Pochard |  |  | Co |
| Rosthramus sociabilis | Everglade Kite |  |  | Co |
| Buteo albicaudatus | White-tailed Hawk |  |  | Pc |
| Buteo polysoma | Red-backed Hawk |  |  | Ra |
| Buteo swainsoni | Swainson's Hawk |  |  | Pc |
| Falco peregrinus | Peregrine Falcon |  |  | Pc |
| Falco peregrinus | Peregrine Falcon |  |  | Pc |
| Porzana flaviventer | Yellow-breasted Crake |  |  | Ra |
| Porphyrula mattinica | Purple Gallinule |  |  | Pc |
| Pluvialis dominica | Golden Plover |  |  | Co |
| Charadrius falklandicus | Two-banded Plover |  |  | Ra |
| Eudromias ruficollis | Tawny-throated Dotterel |  |  | Pc |
| Bartramia longicauda | Upland Sandpiper |  |  | Pc |
| Tringa flavipes | Lesser Yellowlegs |  |  | Co |
| Tringa melanoleuca | Greater Yellowlegs |  |  | Pc |
| Tringa solitaria | Solitary Sandpiper |  |  | C |
| Calidris fuscicollis | White-rumped Sandpiper |  |  | Co |
| Calidris melanotos | Pectoral Sandpiper |  |  | Co |
| Tryngites subruficollis | Buff-breasted Sandpiper |  |  | Pc |
| Rynchops nigra | Black Skimmer |  |  | Co |
| Coccyzus americanus | Yellow-billed Cuckoo |  |  |  |
| Coccyzus cinereus | Ash-coloured Cuckoo |  |  | Ra |
| Coccyzus melacoryphus | Dark-billed Cuckoo |  |  | Pc |
| Tapera naevia | Striped Cuckoo |  |  | Co |
| Chordeiles minor | Common Nighthawk |  |  | Ra |
| Podager nacunda | Nacund Nighthawk |  |  | Co |
| Caprimulgus parvulus | Little Nighthawk |  |  | Pc |
| Hydropsalis brasiliana | Scissor-tailed Nightjar |  |  | Co |
| Chlorostilbon aureoventris | Festive Coquette |  |  | Co |
| Hylocharis chrysura | Gilded Sapphire |  |  | Co |
| Cinclodes fuscus | Bar-winged Cinclodes |  |  | Co |
| Camptostoma obsoletum | Southern Tyrannulet |  |  | Co |
| Elaenia parvirostris | Small-billed Elaenia |  |  | C |
| Polystictus pectoralis | Bearded Tachuri |  |  | Ra |
| Myiophobus fasciatus | Bran-coloured flycatcher |  |  | Co |
| Empidonax euleri | Euler's Flycatcher |  |  | Ra |
| Pyrocephalus rubinus | Vermilion flycatcher |  |  | C |
| Neoxolmis rufiventris | Chocolate-vented Tyrant |  |  | Ra |
| Muscisaxicola macloviana | Dark-faced Ground-Tyrant |  |  | Ra |
| Lessonia rufa | Rufous-backed Negrito |  |  | Co |
| Hirundinea ferruginea | Cliff Flycatcher |  |  | Pc |
| Myarchus swainsoni | Swainson's Flycatcher |  |  | Co |
| Myiodynastes maculates | Streaked Flycatcher |  |  | Co |
| Empidonomus auratioatrocristatus | Crowned Slaty-Flycatcher |  |  | Pc |
| Empidonomus varius | Variegated Flycatcher |  |  | C |
| Tyrannus melancholicus | Tropical Kingbird |  |  | C |
| Tyrannus savana | Fork-tailed Flycatcher |  |  | C |
| Pachyramphus polycopterus | White-winged Becard |  |  | Co |
| Tachycineta leucorhoa | White-rumped Swallow |  |  | C |
| Progne chalybea | Gray-breasted Martin |  |  | C |
| Progne modesta | Southern Martin |  |  | Ra |
| Progne tapera | Brown-chested Martin |  |  | C |
| Notiochelidon cyanoleuca cyanoleuca | Blue-and-white Swallow |  |  | C |
| Notiochelidon cyanoleuca patagonica | Blue-and-white Swallow |  |  | C |
| Alopochelidon fucata | Tawny-headed Swallow |  |  | Co |
| Stelgidopteryx ruficollis | Rough-winged Swallow |  |  | Co |
| Hirundo rustica | Barn Swallow |  |  | Pc |
| Petrochelidon pyrrhonota | Cliff Swallow |  |  | Pc |
| Sporophila caerulescens | Double-collared Seedeater |  |  | Pc |
| Sporophila cinnamomea | Chestnut Seedeater |  |  | Pc |
| Cyanoloxia glaucocaerulea | Indigo Grossbeak |  |  | Pc |
| Geothlypis aequinoctialis | Masked Yellowthroat |  |  | Co |
| Vireo olivaceus | Red-eyed Vireo |  |  | Co |

Appendix ##: Amphibians and reptiles of the plantation landscape

Table ##: Amphibians in the plantation landscape

| Taxon | Common name | Record in Collection | RECA | IUCN Global | IUCN Regional | Expected Presence | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chthonerpeton indistinctum | Cecilia |  | SA | LC | DD | X | Specimen not from Uruguay |
| Hypsiboas pulchellus |  | X | LC | LC | LC |  |  |
| Dendropsophus minutus | Lesser Tree-frog |  | A | LC | LC | X | Inferred |
| D. sanborni |  | X | LC | LC | LC |  |  |
| Pseudis minuta |  | X | LC | LC | LC |  |  |
| Scinax aromothyella |  |  | NE | NE | NE | X | Inferred |
| S. berthae | Dwarf Snout Tree-frog |  | T | LC | LC | X | Inferred |
| S. fuscovarius |  |  | T | LC | LC | X | Inferred |
| S. granulatus |  | X | LC | LC | LC |  |  |
| S. uruguayus |  | X | T | LC | LC |  |  |
| S. squalirostris |  | X | LC | LC | LC |  |  |
| Phyllomedusa iheringii |  | X | T | LC | LC |  |  |
| Leptodactylus gracilis | Striped Frog | X | LC | LC | LC |  |  |
| L. latinasus | Oven Frog | X | LC | LC | LC |  |  |
| L. mystacinus | Moustached Frog | X | LC | LC | LC |  |  |
| L. ocellatus | Criolla Frog | X | LC | LC | LC |  |  |
| Physalaemus biligonigerus | Weeping Frog | X | LC | LC | LC |  |  |
| P. gracilis |  | X | LC | LC | LC |  |  |
| P. henselii | Hensel’s Dwarf Frog | X | LC | LC | LC |  |  |
| P. riograndensis |  | X | LC | LC | LC |  |  |
| Pleurodema bibroni | Four-eyed Frog |  | T | NT | NT | X | Inferred |
| Pseudopaludicola falcipes | Dwarf Frog | X | LC | LC | LC |  |  |
| Odontophrynus americanus |  | X | LC | LC | LC |  |  |
| Limnomedusa macroglossa |  | X | LC | LC | LC |  |  |
| Chaunus achavali |  |  | T | NE |  | X | Inferred |
| C. arenarum |  |  | LC | LC | LC | X | Inferred |
| C. dorbignyi |  | X | ** | LC | ** |  |  |
| C. fernandezae |  | X | ** | LC | ** |  |  |
| Melanophryniscus atroluteus | Redbelly Toad | X | LC | LC | LC |  |  |
| M. devincenzii |  |  | T | EN | EN | X | Inferred |
| M. sanmartini |  |  | SP | NT | NT | X | Inferred |
| M. pachyrhynus |  |  | NE | DD | NE | X | Inferred |
| Elachistocleis bicolor |  | X | LC | LC | LC |  |  |
| AMPHIBIANS –T: threatened, SA: special attention; CR: critical risk; DD: data deficient; EN: endangered; LC: least concern; NT: near threatened; VU: vulnerable, NE: not evaluated. Taxa with * should be considered as data deficient. ** indicates that the species were studied as a group in the genre (Chaunus aff. granulosus) |  |  |  |  |  |  |  |

Table ##: Reptiles in the plantation landscape

| Taxon | Common name | Record in Collection | RECA | IUCN Global | IUCN Regional | Expected Presence | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Trachemys dorbignyi | Brazilian Slider | X |  |  |  |  |  |
| Acanthochelys spixii | Spiny Neck Turtle | X |  | NT |  |  |  |
| Hydromedusa tectifera | Snake-necked Turtle | X |  |  |  |  |  |
| Phrynops hilarii | Side-necked Turtle | X |  |  |  |  |  |
| Phrynops williamsi | Williams' Side-necked Turtle |  |  |  |  | X | Inferred |
| Caiman latirostris | Broad-snout Caiman |  |  | LC |  | X | Confirmed observation |
| Anisolepis undulatus | Wiegmann's Tree Lizard |  | VU | VU |  | X | Inferred |
| Liolaemus wiegmanii | Sand Lizard | X | LC |  |  |  |  |
| Stenocercus azureus |  | X | VU |  |  |  |  |
| Homonota uruguayensis |  | X | LC |  |  |  |  |
| Amphisbaena darwinii | Darwin's Worm Lizard | X | LC |  |  |  |  |
| A. munoai | Chica | X | LC |  |  |  |  |
| Anops kingii | Keel-headed Worm Lizard | X | LC |  |  |  |  |
| Cnemidophorus lacertoides | S. American Teiid Lizard | X | LC |  |  |  |  |
| Teius oculatus |  | X | LC |  |  |  |  |
| Tupinambis merianae | Tegu Lizard | X | LC |  |  |  |  |
| Cercosaura schreibersii |  | X | LC |  |  |  |  |
| Mabuya dorsivittata |  | X | LC |  |  |  |  |
| Ophiodes aff. Striatus |  | X | LC |  |  |  |  |
| O. vertebralis | Jointed Worm Lizard | X | LC |  |  |  |  |
| Leptotyphlops munoai |  | X | PC |  |  |  |  |
| Bothrops alternatus |  | X | LC |  |  |  | Dangerous |
| B. pubescens |  | X | LC |  |  |  | Dangerous |
| Chironius bicarinatus |  |  | MC |  |  | X | Inferred |
| Tantilla melanocephala | Black-head Snake |  | CP? |  |  | X | Inferred presence |
| Calamodontophis paucidens |  |  |  | VU |  | X | Inferred presence |
| Taeniophallus occipitalis | Jan's Forest Snake |  | CP |  |  | X | Inferred presence |
| T. poecilopogon | Cope's Forest S. |  | MC |  |  | X | Inferred |
| T. hypoconia |  | X | LC |  |  |  |  |
| T. strigatus |  |  | CP |  |  | X | Inferred |
| Tomodon ocellatus |  | X | LC |  |  |  |  |
| Boiruna maculata |  | X | CP |  |  |  |  |
| Clelia rustica | Brown Mussurana |  | CP |  |  | X | Inferred |
| Helicops infrataeniatus |  |  | CP |  |  | X | Inferred |
| Liophis almadensis |  | X | CP |  |  |  |  |
| Liophis anomalus |  | X | LC |  |  |  |  |
| Liophis flavifrenatus |  | X | CP |  |  |  |  |
| Liophis jaegeri |  | X | LC |  |  |  |  |
| Liophis semiaureus |  | X | LC |  |  |  |  |
| Liophis poecilogyrus sublineatus |  | X | LC |  |  |  |  |
| Lystrophis dorbignyi |  | X | LC |  |  |  |  |
| Lystrophis histricus |  |  | MC |  |  | X | Inferred |
| Oxyrhopus rhombifer rhombifer |  | X | LC |  |  |  |  |
| Phalotris lemniscatus |  | X | CP |  |  |  |  |
| Philodryas aestiva |  | X | LC |  |  |  |  |
| P. olfersii olfersii |  |  | CP |  |  | X | Inferred |
| P. patagoniensis |  | X | LC |  |  |  |  |
| Pseudablabes agassizii |  | X | MC |  |  |  |  |
| Psomophis obtusus |  | X | LC |  |  |  |  |
| Micrurus altirostris | Southern Coral Snake | X | CP |  |  |  | Dangerous |
| REPTILES – NT: near threatened; LC: Least Concern; VU: Vulnerable; PC: conservation priority, MC: maximum conservation priority |  |  |  |  |  |  |  |

Appendix ##: People interviewed in the local stakeholder analysis

| La Paloma | Clara continued |
| --- | --- |
| Pablo Langone: secretary of Local Mayor | Daniela Cardozo: merchant |
| Julián Lamadrid: retired ex secretary of mayor | Elida Cuadraro: merchant |
| Héctor María Hernández: mechanic | Antonio Almeida: rural worker |
| Carlos Alvariza: Magistrate | Zamora |
| Carmelita Selhay: Dir. of Secondary School | Juan Méndez truck driver |
| Shirley del Pino: temporary forestry worker | Solano Colinas: employee of rural business, buys cattle |
| Dogomar Rivas: contractor, shearing and truck driving | Roberto Caetano: rural business employee |
| Teresa Hernández: teacher | Julio Viera Rodríguez: medium-sixed farm |
| Blanquillo | Mariana ¿?: employee |
| Hugo Núñez: secretary of local Mayor | Daniel Sampayo: secretary of local Mayor |
| María Glora Albes: representative of the local crafts cooperative | María Isabel Barcia: tourist |
| Wilson Tabaré Santana: bar keeper | Hugo Fierro: retired air controller |
| Eber Umpierrez: merchant | Charles Cuadrado fisherman (Secretary Copehum) |
| Carina Soca de Umpierrez: housekeeper | José Mendez Ortiz fisherman (President of Copehum Cooperative) |
| Liliana Sosa: teacher | Jorge Fitzgerald Cuadrado: fisherman |
| Anibal.Miñaur: cattle dealer | Juan Carballo: raftman MTOP. |
| Villa Carmen | Ramoncita Rienzo: nurse |
| Petrona Iris Remedios: teacher and social worker | Juan Domingo Silva Teacher politician & intellectual |
| Luis Iturria: secretary of local Mayor | Juan Silva: manager of Radio Ibirapitá |
| Silvia D´Avis: Public employee, sweeper | Dra. Anayde Lena: Director of Auxiliary health centre |
| Valeria Santos: Public employee, sweeper | Segundo Ademar Muga Prieto: oldest Merchant of San Gregorio |
| Sergio González: temporary forestry worker. | Federico Godoy: waiter, forestry worker. |
| Daniel Milán: temporary forestry and construction worker. | José Antonio Pereira: Journalist Public employee. |
| Padre Gregorio Rodríguez: priest | Ricardo Lopez Laxalde: cattle dealer |
| . Julio César Etcheverry: bee keeper | Carlos Larregui: artist. |
| Amado Dante López: public employee works with Rural studios | Bernardo Ruete: vet, small farmer and bee keeper |
| Pedro Soust: Ing. Agr Owner of a Nursery, local politivian | Adrián Fernández temporary fores worker |
| Pedro González: Grape Producer Director of Vitis Center, Vinos del Carmen | Grecco |
| Gladis Santiago de Gonzales: Integrate social, promotion and devt. group work | Carolina Elhordoy: secretary of local Mayor. |
| Paso de los Toros | Marta Etchebarne: school director. |
| Dr. Otormin: secretary of local Mayor | María Inés Angelo: merchant |
| Carmen Gambeta: social worker | Seferino Miño: young man |
| Wilson Malseñido: environment director of the Local Mayor. | Edgardo Vasella employee of local mayor. |
| Jorge Rodríguez Labruna: ex politician of Partido Nacional | Alvaro Vidal: employee of local mayor |
| Alvaro Silveira: cattle dealer, treasurer of the Rural Association of Paso de los Toros | Richard Vega: repairs |
| Washington Aizpún: cattle dealer. amateur archaeologist. | Guichón |
| Cristina Coture: Association of Isabelinas | Angela Núñez president of local Mayor |
| Teresa Varela de Association of Isabelinas | Danny Silveira Fiure secretary of local Mayor |
| Irineo Pérez: Pte of Rotary Club. Business man with transport company. | Pablo Valdez: member of the commotion of the cemetery of Guichón. |
| Administrator of Charrúa Neighborhood: Olga Antúnez, Teresa Spanga, Mirian Pérez, Jorge Flores, Ester Cáseres, Beatriz Viera | Dr Eduardo Urruti Vet: President of association for work of Guichón |
| Pueblo Centenario | José Texeira: forestry employee. |
| Sr. Denis: Secretary of loval mayor | Niní Gonzales: merchant employee. |
| Hugo Revelo: rural producer | Luis María Frugoni: public employee. |
| Clara | José Vigo: foreman of the Corralón Municipal. |
| Manuel Cortés: businessman and merchant | Víctor Puentes: gas station employee |
|  | Emilia Novo: employee |

Appendix ##: People interviewed in the national stakeholder analysis

| Institution | Contact | Chosen | Interview |
| --- | --- | --- | --- |
| National authorities |  |  |  |
| Sub-secretary MGAP | Ing. Agr. Ernesto Agazzi | YES | Did not accept |
| General Bureau of Forestry. MGAP | Director Ing. Agr. Andrés Berterreche | YES | YES |
| OPYPA – MGAP | Director Ing. Agr. Martín Buxedas | YES | YES |
| National Bureau of Employment. MTSS | Director Sara Paysee. | YES | YES |
| Labor and Social Security General Inspection. MTSS | Director María Narducci. | YES | YES |
| National Bureau of Environment. MVOTMA | Ing. Agr. Alicia Torres. (with Counselor Luis Sayagues) | YES | YES |
| National Bureau of Industry. MIEM | Ing. Agr. Carlos Blasi (Representing MIEM) | YES | YES |
| DINACYT/PDT – MEC | Dr. Amílcar Davyt | NO | NO |
| Producers’ associations |  |  |  |
| Association of Forestry Producers. | Gte. Ing. Agr. Edgardo Cardozo – [Manager] and Gte. Andrea Regusci [Technical Manager] | YES | YES |
| Uruguayan Association of Agronomists. | Pte. Ing. Agr. Enrique Estol [President, Agronomist] | YES | YES |
| Association of Forestry Contractor Companies in Uruguay | Ex. Pte. José Lestido [Former President] | YES | YES |
| Association of wood and derivatives workers in Uruguay | Pte. Delfino Álvarez [President] | NO | NO |
| Rural Association of Uruguay | Pte. Ing. Agr. Guzmán Tellechea Otero [President, Agronomist] | NO | NO |
| Syndicate of Forestry Workers |  | NO | NO |
| Academics and field researchers |  |  |  |
| National Programme Forestry Production Research. INIA | Director Ing. Agr.(Ph.D) Zohra Bennadji [Agronomist] | YES | YES |
| Forestry Unit. School of Agronomy. University of the Republic | Director Ing. Agr. Luis Soria [Agronomist] | YES | YES |
| School of Social Sciences. | Dean Diego Piñeyro | YES | Did not accept |
| School of Economics-Institute of Economics. | Agricultural and Industrial Area - Juncture | NO | NO |
| School of Science. | Dean Julio Fernández | YES | Did not confirm |
| CIEDUR | Ing. Agr. Pérez Arrarte [Agronomist] | YES | YES |
| Public-private |  |  |  |
| National Institute of Technical Regulations [UNIT] |  | NO | NO |
| LATU – [Uruguayan Technological Laboratory] | Ing. Quím. Raúl de Castro [Chemist Engineer] | NO | NO |
| Institute for Promoting Investment and Exports |  | NO | NO |
| Provincial stakeholders |  |  |  |
| Commercial Unit of Durazno |  | YES | Not fixed |
| Durazno Borough Hall | Chief. Carmelo Vidalín | YES | YES |
| Tacuarembó Borough Hall | Sec. Gral. José Menéndez | YES | YES |
| Development Agency of Tacuarembó] | Ing. Agr. Gustavo Ferreira [Agronomist] | YES | YES |
| CLAEH Tacuarembó | Ing. Agr. Daniel Cal [Agronomist] | YES | YES |

Appendix ##: Interview guidelines for national stakeholder analysis

Interviews were based around the following issues:

General aspects of national forestry policy:

- Do you know about the national forest development strategy? If so, what are its main components?

- Do you believe there is an alternative forestry model to the one that is being currently implemented? What features would it have?

- Do you know about any concrete actions that are being or should be carried out to achieve a national forestry scenario integrated with the economy?

Specific aspects of national forestry policy

- What is your opinion about the repeal of Decree 330/90 by the government?  Do you think it promotes a new national forestry policy for the country?

- What do you think about the integration of forestry workers to the negotiations of the Board of Salaries and the meetings of the three parties [Government, Businessmen and Workers]? What concrete progress do you believe it achieves in the sector?

- What impact do you think the incentives to forestry have on the collection of the Borough Halls in rural areas?

- Do you believe that the state is able to assess and follow the environmental impact of forestry?, what does it count on and what does it need?

Vision of the forestry sector in relation to other activity sectors

- What is your vision of forestry in relation to agricultural and livestock sectors?

- What is the role of big companies in the development of the forestry sector? What opportunities do the national (and local) companies have?

- Do you think they are integrated with respect to other national stakeholders? What do you think about it? What conditions are needed to move towards complementarity among the different productive sectors?

- What is your vision of the labour market in the forestry sector, the relationship among the primary stages, industry and services (squads, subcontracts, sawmills)? From your point of view, how would the labour requirements evolve? Which skills are necessary for this sector?

Assessment and future vision

- What expectations does the population have regarding forestry development?

- What is your opinion about the process of forestry in Uruguay? (Mention at least 3 positive and 3 negative aspects of the evolution of this sector):

- In the primary stage, what are the economic stakeholders involved? Which are the main benefits and costs of the sector?

- What are the main impacts on the environment in the primary stage?

- Regarding the social impact, could you mention at least 3 positive and 3 negative aspects of the plantations?

- At present, what is the main technological demand in the sector?

- What type of research do academics believe the sector has to carry out?

- What is your vision for development of the sector in the future? (Year 2020)

Specific questions relating to the Stora Enso operation

- What risks do big companies run when setting up in a region or locality? What things need to be noted and what action is required?

- What aspects does the company need to strengthen to exert a positive impact on the region as well as on the country?

References

Abellán, P., D. Sánchez-Fernández, J. Velasco and  A. Millán. 2004. Selección de áreas prioritarias de conservación en la provincia de Albacete utilizando los coleópteros acuáticos. Instituto de estudios albacetenses “Don Juan Manuel”. Serie I-Estudios- Nº 151: 157pp

Achaval, F. 2007. Listas de especies de vertebrados de Uruguay. http://zvert.fcien.edu.uy. Accessed 8th February 2007

Achaval, F., M. Clara and A. Olmos. 2004. Mamíferos de la República Oriental del Uruguay: Una guía fotográfica. 174 pp

Altesor, Alice, Martín Oesterheld, Elsa Leoni, Felipe Lezama and Claudia Rodríguez. 2005. Effect of grazing on community structure and productivity of a Uruguayan grassland. Plant Ecology 179 (1): 83-91

Altesor, A., G. Piñeiro, F. Lezama, R. B. Jackson, M. Sarasola and J. M. Paruelo. 2005. Ecosystem changes associated with grazing in subhumid South American grasslands, Journal of Vegetation Science 17: 323-332

Andreassian, V., 2004. Water and forests: from historical controversy to scientific debate. Journal of Hydrology 291: 1-27

Anon. 1999. International Experts Meeting on the Role of Planted Forests in Sustainable Forest Management, Santiago Chile 6-10 April 1999, Ministry of Agriculture, Santiago Chile

Anon. Undated (a). Mission, Vision and Values. Sora Enso

Anon. Undated (a). Stora Enso's principles for the development of forest certification. Stora Enso

Araya, E.M. 2000. Colonización de sustratos artificiales por macroinvertebrados bentónicos en un ecosistema fluvial de baja intervención antrópica, Estero Nonguén (VIII Región, Chile). Tesis de Magíster en Zoología. Facultad de Ciencias Naturales y Oceanográficas. Universidad de Concepción. 128 pp

Astrid, L., R. Riveros, F. Gast and P. von Hilderbrand. 2003. Escarabajos coprófagos (Coleoptera: Scarabaeidae: Scarabaeinae) del Parque Nacional Natural “Serranía de Chiribiquete”, Caqueta, Colombia, (Parte I). In Onore, G.; P. Reyes and M. Zunino (eds.) Escarabeidos de Latinoamérica: Estado del Conocimiento (3): 51-58

Baguette, M., B. Deceuninck, and Y. Muller. 1994. Effects of spruce afforestation on bird community dynamics in a native, broad-leaved forest area. Acta Oecologica 15: 275-288

Bazett, M. 2000. Public Incentives for Industrial Tree Plantations. WWF and IUCN, Gland, Switzerland

Behling, Hermann, Valerio DePatta Pillar and Soraia Girardi Bauermann. 2005. Late Quaternary grassland (Campos), gallery forest, fire and climate dynamics, studied by pollen, charcoal and multivariate analysis of the São Francisco de Assis core in western Rio Grande do Sul (southern Brazil). Review of Palaeobotany and Palynology 133: 235-248

Berazategui, M., A. Camargo and R. Maneyro. In press. Environmental and seasonal variation in the diet of Elachistocleis bicolor (Guérin-Méneville 1838) (Anura: Microhylidae) from northern Uruguay. Zoological Science

Bernhard-Reversat, F (ed). 2001. Effect of Exotic Tree Plantations on Plant Diversity and Biological Soil Fertility in the Congo Savanna: With Special Reference to Eucalypts. CIFOR, Bogor, Indonesia.

Bildstein, K. and K. Meyer. 2000. Sharp-shinned hawk (Accipiter striatus). In: A. Poole, F. Gill, (eds.) The Birds of North America, No. 482. Philadelphia, PA: The Birds of North America, Inc

Biodiversity Planning Support Programme. 2001. The Integration of Biodiversity into National Environmental Assessment Procedures: National Case Studies: Uruguay. UNDP / UNEP / GEF

Boothroyd, I.K.G., J.M. Quinn, E.R. Langer, K.J. Costley, and G. Steward. 2004. Riparian buffers mitigate effects of pine plantation logging on New Zealand streams - 1. Riparian vegetation structure, stream geomorphology and periphyton. Forest Ecology and Management 194 (1-3): 199-213

Bosch, J.M. and J.D. Hewlett. 1982. A review of catchment experiments to determine the effect of vegetation changes on water yield and evapotranspiration. Journal of Hydrology 55: 3-23

Bournard, M., P.Richoux and P. Usseglio-Polaterra. 1992. An approach to the synthesis of qualitative ecological information from aquatic coleoptera communities. Regulated Rivers: Research and Management 7:165-180

Brown, A.E., L. Zhang, T. A. McMahon, A. W. Western, R. A. Vertessy. 2005. A review of paired catchment studies for determining changes in water yield resulting from alterations in vegetation. Journal of Hydrology 310: 28-61

Brown, S., E. Palola, and M. Lorenzo. 2006. The Possibility of Plantations: Integrating Ecological Forestry into Plantation Systems. National Wildlife Federation, VA, USA

Calder, I. R. 1986. A stochastic model of rainfall interception. Journal of Hydrology 89: 65–71

Calder, I. R. 2000. Forests and hydrological services: reconciling public and science perceptions, Land Use and Water Resources Research 2: 2.1-2.12

Calder, I. R. 2002.

Cannell, M. G. R. 1999. Environmental impacts of forest monoculture: water use, acidification, wildlife conservation and carbon storage. New Forests 17 (1-3): 239-262

Carnus J-M, J. Parrotta, E. G. Brockerhoff, M. Arbez, H. Jactel, A. Kremer, D. Lamb, K. O’Hara K and B. Walters. 2003. “Planted forests and biodiversity”. In: Buck A., J. Parrotta and G. Eolfrum (eds.).  Science and Technology-Building the future of the world’s forests.  Planted forests and biodiversity.  IUFRO Occasional Paper No. 15. IUFRO, Vienna, Austria, pp. 33-49

Carreira, S. and I. Lombardo. In press. Tomodon dorsatus (NCN). Distribution. Herpetological Review

Carreira, S., M. Meneghel and F. Achaval. 2005. Reptiles de Uruguay. DI.R.A.C. Facultad de Ciencias, Universidad de la República, Montevideo, 639 pp

Carrere, Ricardo. 1999. Plantations Campaign: Ten replies to ten lies. World Rainforest Movement, Montevideo

Carrere, Ricardo. 2001. Monte Indígena. Ediciones de Brecha and Nordan Communidad, Montevideo

Carrere, Ricardo. 2006. Greenwash: Critical Analysis of FSC Certification in Industrial Tree Monocultures in Uruguay. World Rainforest Movement, Montevideo

Carrere, Ricardo and Larry Lohmann. 1996. Pulping the South: Industrial tree plantations and the world paper economy. Zed Books and The World Rainforest Movement, London and Montevideo

CBI. 2005. Uruguay Pulp Mills Stakeholder Assessment Findings. The Consensus Building Institute Cambridge MA and Washington DC, USA

Chapman P. J., A. C. Edwards, B. Reynolds, C. Neal and L. Heathwaite. 1999. The nitrogen composition of streams draining grassland and forested catchments : influence of afforestation on the nitrogen cycle in upland ecosystems , IAHS Press, Wallingford, UK

Charnley, S. 2006. Industrial Plantation Forestry: Do Local Communities Benefit? Journal of Sustainable Forestry 21 (4)

Chomitz, K, M, and K, Kumari, 1996, The domestic benefits of tropical forests: A critical review emphasizing hydrologic functions, World Bank Policy Research Working Paper, Number 1601, Washington DC

Ciesla, W. M. 1991. Cypress aphid: a new threat to Africa's forests; Unasylva 167:42

CIFOR and IUFRO. 1999. Biodiversity Conservation in Production Forests. Center for International Forestry Research and International Union of Forest Research Organisation, Bogor Indonesia and Vienna

Clout, M. N. 1984. Improving exotic forests for native birds. New Zealand Journal of Forestry 24:2

Commonwealth of Australia. 1999. A Study on the Global Outlook for Plantations: Executive Summary. A technical paper for the Intergovernmental Panel on Forests. Commonwealth of Australia, Canberra

Cossalter, Christian and Charlie Pye-Smith. 2003. Fast-Wood Forestry: Myths and realities. Forest Perspectives number 1. Center for International Forestry Research, Bogor Indonesia

Counsell, Simon and Kim Terje Loraas. 2002. Trading in Credibility: The myth and reality of the Forest Stewardship Council. The Rainforest Foundation, London and Oslo

Cregg, B., C. Rios, J. Hart, and D. Briggs. 2004.  Fate of Nitrates in Field Nursery Production Systems, USDA Forest Service Proceedings RMRS-P-33

Croke, J, P. Wallbrink, P. Fogarty, P. Hairsine, S. Mockler, B. McCormack and J. Brophy. 1999. Managing Sediment Sources And Movement In Forests: The Forest Industry and Water Quality, INDUSTRY REPORT, 99/11, Australia

De’Nadai, A, Overbeek, W and L A Soares (2005) ;  Promises Of Jobs And Destruction Of Work, The case of Aracruz Celulose in Brazil, World Rainforest Movement, Montevideo

Department of Sustainability and Environment. 2006. Code of Practice for Timber Production – Draft for Public Comment. Victoria Government, Melbourne, Australia

Down to Earth. 1991. Pulping the Rainforest: The rise of Indonesia’s paper and pulp industry. Down to Earth International Campaign for Ecological Justice in Indonesia, Asia Pacific People’s Environmental Network, Penang, Malaysia

Dudley, N. 1985. Nitrates in Food and Water. London Food Commission, London

Dudley, Nigel. 1992. Forests in Trouble: A review of the status of temperate forests worldwide. WWF International, Gland, Switzerland

Dudley, N. and J-P. Jeanrenaud. 1997. Needs and prospects for international cooperation in assessing forest biodiversity: an overview from WWF, in Assessment of Biodiversity for Improved Forest Planning: Proceedings of the Conference on Assessment of Biodiversity for Improved Forest Planning, 7-11 October 1996 in Monte Verità, Switzerland, European Forest Institute, Proceedings number 18, Kluwer Academic Publishers

Dudley, Nigel, Sue Stolton and Jean-Paul Jeanrenaud. 1995. Pulp Fact: The environmental and social impacts of the pulp and paper industry. WWF International, Gland, Switzerland

Duarte, J.M.B. 2001. O Cervo do Pantanal (Blastoceros dichotomus) de Porto Primavera. -Resultado de dois anos de Pesquisa. C.D. Rom - Jaboticabal FUNEP-Brasil

Dykstra, Dennis P and Rudolf Heinrich (1995); FAO Model Guide of Forestry Practice, Food and Agricultural Organisation of the United Nations, Rome

Eckholm, Eric. 1975. The Other Energy Crisis: Firewood. Worldwatch Paper number 1. Worldwatch Institute, Washington DC

Eckholm, Eric, Gerald Foley, Geoffrey Barnard and Lloyd Timberlake. 1984. Fuelwood: The energy crisis that won’t go away. Earthscan, London

El-Swaify S. A. 2002. Impacts of Land Use Change on Soil Erosion and Water Quality—A Case Study from Hawaii. 12th ISCO conference, Beijing

Elliott, C. 2003. WWF Vision for Planted Forests, Keynote paper for the UNFF Intersessional meeting on Planted Forests, New Zealand, March 24-30, 2003

Elliott H. J. and B.S. Hodgson. 2004. Water sampling by Forestry Tasmania to determine presence of pesticides and fertiliser nutrients, 1993–2003. Tasforests 15, June 2004

Escobar, F., 2000. Diversidad de coleópteros coprófagos (Scarabaeidae: Scarabaeinae) en un mosaico de hábitats en la reserva Natural Nukak, Guaviare, Colombia. Acta Zoológica Mexicana 79: 103-121

Evans, Julian. 1999. Sustainability of Forest Plantations: The evidence. Department for International Development, London

Evans, J. and J. W. Turnbull. 2005. (3rd edition). Plantation Forestry in the Tropics

The role, silviculture and use of planted forests for industrial, social, environmental and agroforestry purposes. Oxford University Press, Oxford. UK

Fahey B. and R. Jackson. 1997. Hydrological impacts of converting native forests and grasslands to pine plantations, South Island, New Zealand. Agricultural and Forest Meteorology 84: 69-82

Faithful, J., J. Brodie, C. Armstrong, P. Frayne and K. Bubb.2005.; Water Quality Of Runoff Draining From Pine Plantation, Native Forest And Agricultural Crops In The Whitfield Creek Catchment, Located In The Wet Tropics Of North Queensland, Australia In The 2003/04 Wet Season. ACTFR Report No. 05/02, Australia

FAO. 1977. Planning Forest Roads and Harvesting Systems. Forestry Department, FAO, Rome

FAO. 1987. Appropriate Wood Harvesting in Plantations. FAO Forestry Paper 78. FAO, Rome

FAO. 1998. Forest Resource Assessment 2000 Terms and Definitions. Forest Resource Assessment Programme Working Paper number 1. Food and Agriculture Organisation of the United Nations, Rome

FAO. 2000. Global Forest Resources Assessment 2000: Main Report. FAO Forestry Paper 140. Food and Agricultural Organization of the United Nations, Rome

FAO. 2001. Global Forest Resources Assessment 2000. FAO Forestry Paper number 140. Food and Agriculture Organisation of the United Nations, Rome

FAO. 2001. Biological Sustainability of Productivity in Successive Rotations, (Based on the work of Julian Evans, Consultant, edited by D. J. Mead). FAO, Rome

FAO. 2005. Global Forest Resources Assessment 2005: Progress towards sustainable forest management. FAO Forestry Paper number 147. Food and Agriculture Organisation of the United Nations, Rome

FAO. 2006. Draft Planted Forest Code, 2nd draft 14 March 2006, FAO, Rome

Favila, M. and G. Halffter. 1997. The use of indicator groups for measuring biodiversity as related to community structure and function. Acta Zool. Mex. (n.s.):72:1-25

Figuero, R., C. Valdovinos, E. Araya and O. Parra. 2003. Macroinvertebrados bentónicos como indicadores de calidad de agua de ríos del sur de Chile. Revista Chilena de Historia Natural 76 (2): 275-285

Fontana, C., A. G. Bencke, E. R. Reis. 2003. Livro vermelho do fauna ameacada de extincao no Rio Grande do Sul. Porto Alegre. Brasil. 632 p.

Forest Owners Conference (2003); Plantation Design Guidelines, 12 organisations based in Australia

Forest Safety Advisory Group. 2002. Forestry Safety Code (Tasmania). Hobart

Forest Service. 2000. Code of Best Forest Practice – Ireland. Dublin

Friedman, S. T. 2006. Environmental aspects of the intensive plantation/reserve debate. Journal of Sustainable Forestry 14 (21)

Garforth, M. and J. Mayers. 2005. Plantations, privatisation, poverty and power. International Institute for Environment and Development, London

Garson, J.; A. Aggarwal and S. Sarkar. 2002. Birds as Surrogates for Biodiversity: An Analysis of a Data Set from Southern Québec. Journal of BioSciences 27 (Nº 4, Suppl.2) 347-360

Geary, T. F. 2001. Afforestation in Uruguay: a changing landscape. Journal of Forestry 99: 35-39

Girvetz, E.H. and F.M. Shilling. 2003. Decision support for road system analysis and modification on the Tahoe National Forest. Environmental Management 32: 218-233

Goncalves, J.L. and J.L. Stape (eds.). 2002. Conservacao e cultivo de solos para plantacoes florestais. Instituto de Pesquisas e Estudos Florestais, Piracicaba, Brazil

Gonzalez, S. 2004. Biología y conservación de Cérvidos Neotropicales del Uruguay. Informe Final de Proyecto CSIC-UdelaR,  57pp

González, S., R. Álvarez –Alvarez and J. E. Maldonado. 2004. Técnica molecular para la determinación taxonómica y análisis poblacional en cérvidos neotropicales VI International Conference on Wildlife Management in Amazonia and Latin America: 5- 11 September, Iquitos-Perú

González, S., J. E. Maldonado, J. E. Garcia andJ. M. B. Duarte. 2001. Ecologia e Distribuição de Mazama bororo (MAMMALIA: CERVIDAE) Diseño de primers especie-específicos”. Informe de proyecto Financiado por Programa de Biodiversidade do  Ministerio do Meio Ambiente- Brasil

Green, T. (ed.) 2001. Ecological and Socio-Economic Impacts of Close-to-Nature Forestry and Plantation Forestry: A Comparative Analysis. EFI Proceedings number 37. European Forest Institute. Joensuu, Finland

Gregersen, H. M., K. N. Brooks, J. A. Dixon and L. S. Hamilton. 1987. Guidelines for economic appraisal of watershed management projects, FAO Conservation Guide No. 16, FAO, Rome

Grieg-Gran, Maryanne. 1996. Towards a Sustainable Paper Cycle. International Institute for Environment and Development, London

Grönroos, J. 2007. Stora Enso and Forest Certification: Sustainability Fact Sheets, Stora Enso, London

Halffter, G. 1998. Una estrategia para medir la biodiversidad a nivel de paisaje . In G. Halffter (ed.). La Diversidad Biológica de iberoamérica. Vol.II. Acta Zool Mex. (n.s.). Número especial :3-18

Halffter, G. and L. Arellano. 2002. Response of dung beetle diversity to human-induced changes in a tropical landscape. Biotropica 34(1):144-154

Halffter, G. and M. Favila. 1993. The Scarabaeinae (Insecta: Coleoptera) an animal group for analizing, inventorying and monitoring biodiversity in tropical rainforest and modified landscapes. Biology International 27:15-21

Halffter, G, C. Moreno and E. Pineda. 2001. Manual para evaluación de la biodiversidad en Reservas de la Biosfera. M&T- Manuales y Tesis SEA, vol.2. Zaragoza

Hall, R. 2003. Interception loss as a function of rainfall and forest types: stochastic modelling for tropical canopies revisited. Journal of Hydrology 280: 1-12

Hammond, P. 1995. Practical approaches to the estimation of the extent of biodiversity in species groups. In D. Hawksworth (ed.). Biodiversity: Measurement and Estimation: 119-136. Chapman and Hall, London

Härmälä, J. 2006. Stora Enso Code of Ethics States, 5 December 2006, Stora Enso

Harper, R.J., G. Mauger, N. Robinson, J.F. McGrath, K.R.J. Smettem, J.R. Bartle and R.J. George. 2000. Manipulating Catchment Water Balance using Plantation and Farm Forestry: Case studies from South-Western AustraliaIn: S. Nambiar, and A.G. Brown (e4ds.). Plantations, Farm Forestry and Water. RIRDC Publication No. 01/20: 9-19

Hartley, M.J. 2002. Rationale and methods for conserving biodiversity in plantation forests. Forest Ecology and Management 155: 81-95

Hartshorn, G. S. 1983. Ecological implications of tropical plantation forestry, in R. Sedjo (ed.) The Comparative Economics of Plantation Forestry: A Global Assessment, Resources for the Future, Washington DC

Helle P. and M. Mönnönnen. 1990. Forest succession and birds communities: theoretical aspects and practical considerations. In Keast A. (ed.) Biogeography and ecology of forest bird communities. S. P. B. Academic Publishing. The Hague, Netherlands

Helliwell, R. C., R. C. Ferrier, L. Johnston, J. Goodwin and R. Doughty. 2001.   Land use influences on acidification and recovery of freshwaters in Galloway, south-west Scotland. Hydrology and Earth System Sciences 5(3): 451–458

Helsingen Sanomat. 2007. “Demonstration at Stora Enso forest plantation in Brazil”. 7th March 2007, Helsinki

Hisham, Nohamad Ahmed, Jan Sharma, Anthony Ngaiza and Nicholas Atampugre. 1991. Whose Trees? A People’s View of Forestry Aid. Panos Institute, London

Houston, C. S. and D. E. Bowen. 2001. Upland Sandpiper (Bartramia longicauda). In: A. Poole and F. Gill (eds.). The Birds of North America. Nr. 580. Philadelphia: Academy of Natural Sciences, Washington; American Ornithologists' Union

Hull, H., S. Freitag, S. Chown and C. Bellamy. 1988. Identification and evaluation of priority conservation areas for Buprestidae (Coleoptera) in South Africa, Lesotho, Swaziland and Namibia. African Entomology 6: 265-274

Humphrey, J. W., R. Ferris and C. P. Quine. 2003. Biodiversity in Britain’s Planted Forests. Forestry Commission, Edinburgh

Hunter, M. L. 1990. Wildlife, forests and forestry: principles of managing forests for biological diversity. Prentice and Hall, Englewood Cliffs NJ

IUCN 2006. 2006 IUCN Red List of Threatened Species. <www.iucnredlist.org>. Downloaded on 16 March 2007

ITTO. 2002. ITTO Guidelines for the Restoration, Management and Rehabilitation of Degraded and Secondary Tropical Forests, ITTO Policy Series No 13, International Tropical Timber Organisation, Yokohama, Japan

Jackson, R.B.; Jobbagy, E.G.; Avissar, R., Roy, S.B.; Barret, D.J.; Cook, C.W.; Farley, K.A.; Maitre, D.C.; McCarl, B.A.; Murray, B.C., 2005. Trading water for carbon with biological carbon sequestration. Science 310: 1944-1947

James, R. and A. del Lungo. 2005. The potential for fast-growing commercial forest plantations to supply high-value roundwood. Planted Forests and Trees Working Paper FP 33. FAO, Rome

Jiayu, B. and G. Siming. 1996. Eucalyptus Plantations in China, In: FAO, Reports Submitted to the Regional Expert Consultation on Eucalyptus - Volume II, FAO, Rome, Italy

Kanowski, Peter. 2005. Intensive planted forests. The Forest Dialogue, Yale, New Haven

Keenan, R. J., M. Parsons, A. Gerrand, E. ’Loughlin, S. Beavis, S., D. Gunawardana, M. Gavran. and A. Bugg. 2004. Plantations and water use: a review prepared for the Forest and Wood Products Research and Development Corporation. Bureau of Rural Sciences, Canberra

Kelly, G. and K. Lymon. 2003. To Trees or Not to Trees? An Assessment of the Social Impact of the Plantation Industry on the Shire of Plantagenet. Forests and Wood Products, Research and Development Corporation, Australian Government, Australia

Klein, B. 1989. Effects of forest fragmentation on dung and carrion beetle communities in central Amazonia. Ecology 70(6): 1715-1725

Knutson, M. G., J. R. Sauer, D. A. Olsen, M J. Mossman, L. M. Hemesath and M. J. Lannoo. 1999. Effects of landscape composition and wetland fragmentation on frog and toad abundance and species richness in Iowa and Wisconsin, USA. Conservation Biology 13(6): 1437 – 1446

Knutson, M. G., J. R. Sauer, D. A. Olsen, M J. Mossman, L. M. Hemesath and M. J. Lannoo. 2000. Landscape associations of frog and toad species in Iowa and Wiconsin, USA. Journal of Iowa Academy of Sciences 107(3): 134 – 145

Kroesa, Renate. 1990. The Greenpeace Guide to Paper. Greenpeace International, Amsterdam

Kuczera, G. 1987. Prediction of water yield reductions following a bushfire in ash-mixed species eucalypt forest. Journal of Hydrology 94: 215-236

Leach, Gerald and Robin Mearns. 1988. Beyond the Fuelwood Crisis: People, Land and Trees in Africa. Earthscan Publications, London

Lemckert, F., T. Brassil and A. Towerton. 2005. Native vegetation corridors in exotic pine plantations provideo long-term habitat for frogs. Ecological Management and Restoration, 6(2): 132-134

Lemenih, M. 2006. Expediting ecological restoration with the help of foster tree plantations in Ethiopia. Journal of the Drylands 1 (1): 72-84

Leonel. M. H. 2005. Forest plantations in Brazil. Address to the FAO Advisory Committee on Paper and Wood Products. Vacouver, May 31 2005

Locklin, Claudia. 2001. Uruguayan savannah ecoregion, summarised in the website http://www.worldwildlife.org/wildworld/profiles/terrestrial/nt/nt0710_full.html

Majer, J. and G. Beeston. 1996. The Biodiversity Integrity Index: An illustration using ants in Western Australia. Conservation Biology 10(1): 65-73

Maneyro, R. and M. Beheregaray. In press. First record of Physalaemus cuvieri Fitzinger, 1826 (Anura, Leptodactylidae) in Uruguay, with comments on the anuran fauna along the borderline Uruguay-Brazil. Boletín de la Sociedad Zoológica del Uruguay

Maneyro, R. and S. Carreira. 2006. La herpetofauna de la costa uruguaya. In: Menafra et al (eds.): Bases para la conservación y el manejo de la costa uruguaya. Editorial Graphis. Montevideo: 233 – 246

Maneyro, R., F. Forni and M. Santos. 1995. Los anfibios del departamento de Rocha. Serie Divulgación Técnica. PROBIDES I: 1-24

Mayers J. and S. Vermeulen. 2002. Company-community forestry partnerships: From raw deals to mutual gains? An international review with proposals for improving forests, enterprise and livelihoods. International Institute for Environment and Development, London, UK

Millán, A., J. Moreno and J. Velasco. 2001a. Estudio faunístico y ecológico de los coleóptros y heterópteros acuáticos de las lagunas de Alb acete (Alboraj, Los Patos, Ojos de Villaverde, Ontafia y Pétrola). Sabuco 1:43-94

Millán, A., J. Moreno and J. Velasco. 2001b. Estudio faunístico y ecológico de los coleópteros y heterópteros acuáticos de las lagunas y humedales de Albacete . Sabuco 2:167-214

Miller, Alan S., Irving M. Mintzer and Sara H. Hoagland. 1986. Growing Power: Bioenergy for development and industry. World Resources Institute, Washington DC

Mones, A., J. González, R. Praderi and M. Clara 2003. Diversidad de la Biota Uruguaya. Mammalia. Anales del Museo de Historia Natural de Montevideo 2ª seria 10 (4): 1-28

Montagnini, F. 2005. “Attempting to Restore Biodiversity in Even-Aged Plantations”. In: S. Mansourian, D. Vallauri, and N. Dudley (eds.) Forest Restoration in Landscapes: Beyond Planting Trees. Springer, New York, USA

Morales Fagundes, S. and S. Carreira. 2001. Calificación del estado de conservación de la fauna de ofidios (Reptilia, Squamata, Serpentes) de Uruguay. FACENA 16: 45-51

Morelli, E.; P. Gonzalez-Vainer and A. Baz. 2002. Coprophagous beetles (Coleoptera: Scarabaeoidea) in Uruguayan prairies: Abundance, diversity and seasonal occurrence. Studies on Neotropical Fauna and Environment 37(1): 53-57

Myers, N. 1979. The Sinking Ark: A new look at the problem of disappearing species. Pergamon Press, Oxford

Myers, N. R. A. Mittermeier, C. G. Mittermeier, G. A. B. da Fonseca and J. Kent. 2000. Biodiversity hotspots for conservation priorities. Nature 403: 853-858

Neal, C., B. Reynolds, M. Neal and B. Williams. 2004. The hydrochemistry of plantation spruce forest catchments with brown earth soils, Vyrnwy in mid-Wales. Hydrology and Earth System Sciences 8(3), 460-484

Nores, Manuel, Maria M. Cerana and Diego A. Serra. 2005. Dispersal of forest birds and trees along the Uruguay River in southern South America. Diversity and Distributions 11(3): 205-217

Nuñez, D., R. Maneyro, J. A. Langone and R. O. de Sá. 2004. Distribución geográfica de la fauna de anfibios del Uruguay. Smithsonian Herpetological Information Service, (134): 1-34

Pallarés, Olegario Royo, Elbio J. Berretta and Gerzy E. Maraschin. 2005. The South American Campos Ecosystem. In: Grassland of the World. [Edited] J.M. Suttie, S.G. Reynolds and C. Batello. Food and Agricultural Organization of the United Nations. Rome

Palmer, M., 1999. The application of biogeographical zonation and biodiversity assessment to  the conservation of freshwater habitats in Great Britain. Aquatic Conservation-Marine and Freshwater Ecosystems 9: 179-208

Panario, Daniel, and Mario Bidegain. 1997. "Climate change effects on grasslands in Uruguay," Climate Research 9 (1-2): 37-40

Parris, K. M. and D. B. Lindenmayer. 2004. Evidence that creation of a Pinus radiata plantation in south-eastern Australia has reduced habitat for frogs. Acta Oecologica 25: 93 – 101

Pearson, D., 1994. Selecting indicator taxa for the quantitative assessment of biodiversity. Philosophical Transactions of the Royal Society of London, Series B 345:75-79

Pearson, D. and F. Cassola 1992. World-wide Species Richness Patterns of Tiger Beetles (Coleoptera:Cicindelidae): Indicator Taxon for Biodiversity and Conservation Studies. Conservation Biology 6 (3): 376-390

Pennington, P. I. and M. Laffan. 2004.  Evaluation of the use of pre- and post-harvest bulk density measurements in wet Eucalyptus obliqua forest in Southern Tasmania. Ecological Indicators 4: 39-54

Perlin, John. 1989. A Forest Journey: The role of wood in the development of civilisation. Harvard University Press, Cambridge Massachusetts, p 67

Pharo, E., A. Beattie and D. Binna. 1999. Vascular plant diversity as a surrogate for bryophyte and lichen diversity. Conservation Biology 13: 282-292

Poore, M. E. D. and C. Fries. 1985. The Ecological Effects of Eucalyptus. FAO Forestry Paper 59. Food and Agricultural Organization of the United Nations, Rome

Poore, M. E. D. 1993. Guidelines: Shell/WWF Tree Plantation Review. Shell International Petroleum Company Ltd and WWF UK

Pretty, J. and G. Conway. 1988. The Blue Baby Syndrome and Nitrogen Fertilisers: A High Risk in the Tropics? Gatekeeper Series number 5. International Institute for Environment and Development, London

Red Uruguay de ONGs Ambientalistas de Uruguay. 2006. Comentarios De La Red Uruguaya De Ongs Ambientalistas De Uruguay (Red) Al Estudio De Impactos Acumulativos De La Instalación De Plantas De Celulosa En Fray Bentos, Realizado Por La Corporación Financiera Internacional (Bm), Red Uruguay de ONGs Ambientalistas de Uruguay, Uruguay

Relyea, R. A. 2005a. The impact of insecticides and herbicides on the biodiversity and productivity of aquatic communities. Ecological Applications 15(2): 618-627

Relyea, R. A. 2005b. The lethal impact of Roundup on aquatic and terrestrial amphibians. Ecological Applications 15(4): 1118-1124

Renderos Durán, R. V., J.-M.  Harmand,  F. Jiménez, and D. Kass. 2002. Contaminación del agua con nitratos en microcuencas con sistemas agroforestales de Coffea arabica con Eucalyptus deglupta en la Zona Sur de Costa Rica. Agroforestería en las Américas 9: 35-36)

Republica Oriental del Uruguay, OEA, and BID. 1992. Estudio Ambiental Nacional: Plan de Accion Ambiental. Organizacion de los Estados Americanos, Washington, DC.

Ribera, I., J. E. Hogan and A. P. Vogler. 2002. Phylogeny of Hydradephagan water beetles inferred from 185 rRNA sequences. Molecular Phylogenetics and Evolution 23(1) 43-62

Richard, B. N. 1993. Guidelines for Site Classification and Determination of Fertilizer Requirements of Industrial Forest Plantations. Asian Development Bank

Richoux, P. 1994. Theoretical habitat templates, species traits, and species richness: aquatic Coleoptera In the Upper Rhône River and its floodplain. Freshwater Biology 31: 377-395

Rickett S, T., G. Daily and P. Ehrlich. 2002. Does butterfly diversity predict moth diversity? Testing a popular indicator taxon at local scales. Biological Conservation 103: 361-370

Roche, M. M. and R. B. Le Heron. 1993. New Zealand: Afforestation policy in eras of state regulation and de-regulation. In A. Mather (ed.) Afforestation: Policies, planning and progress. Belhaven Press, London

Rodríguez, Claudia, Elsa Leoni, Felipe Lezama and Alice Altesor. 2003. Temporal trends in species composition and plant traits in natural grasslands of Uruguay. Journal of Vegetation Science 14: 433-440

Rosoman, G., 1994. The Plantation Effect. Greenpeace New Zealand. 48p.

Sahin V. and M. J. Hall. 1996. The effects of afforestation and deforestation on water yields. Journal of Hydrology 178(1–4): 293–309

Sawyer, J. 1994. Plantations in the Tropics: Environmental Concerns. IUCN Forest Conservation Programme, IUCN, Gland

Scott, D.F., 2005. On the hydrology of industrial timber plantations. Hydrological Processes 19: 4203-4206.

Scott, D.F. and W. Lesch. 1997. Streamflow responses to afforestation with Eucalyptus grandis and Pinus patula and to felling in the Mokobulaan experimental catchments. South African Journal of Hydrology 199: 360–377

Sedlak, O. 1988. Principles of Forest Road Nets. Food and Agriculture Organisation of the United Nations, Rome

Sedlak, O. 1988. Maintenance of Forest Roads. Food and Agriculture Organisation of the United Nations, Rome

Sedlak, O. 1994. Forest harvesting and environment in Austria, paper presented at the Meeting of Experts on Forest Practices. FAO, Rome

Shiva, V. and J. Bandyopadhyay. 1985. Ecological Audit of Eucalyptus Cultivation. The English Book Depot, Dehradun, India

Shure, D. and D. Phillips. 1991. Patch size of forest openings and arthropod populations. Oecologia 86: 325-334

Sluys, R. 1999. Global diversity of land planarians (Platyhelminthes, Tricladidda, Terricola): a new indicator-taxon in biodiversity and conservation studies. Biodiversity and Conservation 8: 1663-1681

Södergren, Anders [editor]. 1988. Biological Effects of Bleached Pulp Mill Effluents. National Swedish Environmental Protection Board Report 3558. NSEPB, Solna, Sweden.

Stattersfield, A.J., M. J. Crosby, A. J. Long, and D. C. Wege, editors. 1998. Endemic bird areas of the World: Priorities for biodiversity conservation. BirdLife International, Cambridge, UK.

Strauss, S. H., P. Coventry, M. Campbell, S. M. Pryor and J. Burley. 2001. Certification of genetically modified forest plantations. International Forestry Review 3 (2): 85-102

Stroud, David A., T. M. Reed, M. W. Pienkowski and R. A. Lindsay. Undated. Nirds, Bogs and Forestry: The Peatlands of Caithness and Sutherland. Nature Conservancy Council, Peterborough, UK

Tompkins, S. C. 1986. The Theft of the Hills: Afforestation in Scotland. The Ramblers Association and the World Wildlife Fund, London

UNESCO. 1999. Practices on the application of the Convention for the Protection of World Heritage.  UNESCO, Paris: Articles 37, 38

Vaz-Ferreira, R., L. Covelo de Zolessi and F. Achaval. 1970. Oviposición y desarrollo de ofidios y lacertilios en hormigueros de Acromyrmex. Physis 29 (79): 431-459

Vaz-Ferreira, R., L. Covelo de Zolessi and F. Achaval. 1973 Oviposición y desarrollo de ofidios y lacertilios en hormigueros de Acromyrmex II. Trab. V Congr. Latinoam. Zool. 1: 232-244

Vertessy, R.A., 2000. Impacts of plantation forestry on catchment runoff. In: S. Nambiar, and A.G. Brown (e4ds.). Plantations, Farm Forestry and Water. RIRDC Publication No. 01/20: 9-19

Vertessy, R.A.; F.G.R. Watson and S.K. O´Sullivan. 2001. Factors determining relations between stand age and catchment water balance in mountain ash forests. Forest Ecology and Management 143: 13-26

Watkins, Charles [editor]. 1993. Ecological Effects of Afforestation. CAB International, Oxford

Wheeler, G. L., K. F Steele and E. R. Lawson. 2000. Water and nutrient movement in small, forested watersheds in the Boston mountains of Arkansas. Forest Science 46: 335-343

White, A. and A Martin. 2002. Who Owns the World’s Forests? Forest Trends, Washington D.C.

White, D.A., C.L. Beadle, M. Battaglia, R.G. Benyon,  F.X. Dunin and J.L. Medhurst. 2000. A Physiological Basis for Management of Water Use by Tree Crops. In S. Nambiar, and A.G. Brown, (Eds.). Plantations, Farm Forestry and Water. RIRDC Publication No. 01/20: 9-19

Zhang, L.; Dawes, W.R.; Walker, G.R., 2001. Response of mean annual evapotranspiration to vegetation changes at catchment scale. Water Resources Research 37: 701-708

Additional references from Walter

.

Dye, P. & Versfeld, D., 2005. Managing the hydrological impacts of South African plantation forests: an overview. XII IUFRO World Congress. Brisbane, Australia, August 2005.

Lima, W.P., 2006. Efeitos hidrológicos do manejo de florestas plantadas. In: Lima, W.P. & Zakia, M.J.B. (Eds.). As Florestas Plantadas e a Água: implementando o conceito da microbacia hidrográfica como unidade de planejamento. Rima Editora, São Carlos, SP: 9-28

Lima, W.P. & Zakia, M.J.B., 1998. Integrated monitoring of hydrological indicators of sustainable management of forest plantations in Brazil. In: International Conference on Indicators for Sustainable Forest Management. Natural Resources and Environment, Australia: 119-121

Mackensen J. J. Bauhus and E. Webber. 2003. Decomposition rates of coarse woody debris: A review with particular emphasis on Australian tree species. Australian Journal of Botany 51: 27-37

Ryan, M.G.; Binkley, D.; Fownes, J.H.; Giardina, C.P.; Senock, R.S., 2004. An experimental test of the causes of forest growth decline with stand age. Ecological Monographs 74: 393-414.

Sharda, V. Nn, P.Samraj, J. S. Samara and V. Lakshmanan. 1998. Hydrological behavior of first generation coppiced bluegum plantations in the Nilgiri sub-watersheds. Journal of Hydrology 211: 50-60

Versfeld, D. B. 1996. Forestry and water resources: Policy development for Equitable Solutions, South African Forestry Journal 176: 55-60

The grassland is often known as pampas, particularly outside Uruguay. Strictly speaking, campos is sub-tropical while pampas is temperate grassland, generally identified as starting further south in Argentina. There is considerable overlap of species.